3GPP TSG RAN WG1 #108-e	                                	   R1-2201820

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.2

Source: 	ASUSTeK

Title:  	Remaining issues on V2X mode 2 enhancements

Document for:	Discussion and Decision

- Introduction

RP-212880 [1] lists remaining issues on inter-UE coordination for mode 2 enhancements, as below:

- Physical layer aspects on solution(s) on enhancement(s) in mode 2 for enhanced reliability and reduced latency including

  - Scheme 1

    - Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

    - Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

    - Finalization of when UE-B generates and/or transmits an explicit request

    - Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of prioritization of inter-UE coordination information and explicit request

    - Combination of preferred/non-preferred resources with explicit request/condition triggers

  - Scheme 2

    - Finalization of determination of PSFCH resource/index for conflict indication

    - Finalization of behaviour of UE-B receiving a conflict indication from UE-A

    - Finalization of prioritization of conflict indication

    - Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

In this contribution, we discuss some remaining issues about scheme 1 for V2X mode 2 enhancements.

- Discussion

- Contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request

Based on RAN1 #107-e agreement [2], at least MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B. For indication of resource set, N combinations of TRIV, FRIV, and/or resource reservation period are utilized based on Rel-16 TS 38.214 Section 8.1.5 with some modifications. For instance, one modification is that the first resource location of each TRIV is separately indicated by the inter-UE coordination information.

Moreover, there is one additional difference that TRIV in SCI is used to indicate non-overlapped resources in time domain, and thus slot offsets t1 and t2 are restricted as 1≤t1 and t1< t2. However, for indicating the resource set, it is quite possible that more than one candidate resources are in the same slot. To indicate these candidate resources in the same slot, TRIV for indication of resource set will need to indicate slot offsets t’1 and t’2, which values are possible to be zero, as example in Fig. 2.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st |  | 2nd |  | 3rd |  |  |
|  |  | 1st |  |  |  | 4th |  |  |
|  |  |  |  | 2nd |  |  |  | 4th |
|  |  |  |  | 3rd |  |  |  |  |

Reference slot

Slot

Sub-channels

Fig. 2 Modification of TRIV interpretation

To avoid complex formula design, a simple modification can be set as t’1 = t1 -1 and t’2 = t2 -2. The TRIV for indication of resource set can be determined as follows:

---------------------------------------------------------------------------------------

if the one combination indicates one resource

TRIV for indication of resource set =t'1 + 1

else

if (t’2 - t’1) ≤15

TRIV for indication of resource set = 30 (t’2 - t’1) + t’1 +32

else

TRIV for indication of resource set = 30 (30 - t’2 + t’1) - t’1 +61

end if

end if

---------------------------------------------------------------------------------------

Proposal 1:  Comparing to slot offsets t1 and t2 indicated by TRIV in SCI, the TRIV for indicating resource set indicates slot offset t’1 and t’2, wherein t’1 = t1 -1 and t’2 = t2 -2.

- Which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

It is agreed that there are two types of “set of resources” in scheme 1.

    - the set of resources preferred for UE-B’s transmission (White resource set)

In this type, UE-A excludes resource(s) overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than RSRP threshold (Condition 1-A-1). Based UE-A’s sensing result, UE-A can identify candidate resources not reserved by other UEs or with lower interference, and then include the identified sidelink resources in the set of resource. Accordingly, if UE-B does not perform sensing, UE-B may select sidelink resources directly from the set of resources (Option B). If UE-B performs sensing, UE-B can select sidelink resources based on both the set of resources and UE-B’s sensing result (Option A).

When UE-A performs sensing and resource exclusion, UE-A needs to compare measured RSRP and RSRP threshold based on a specific TX priority. UE-A can deliver such information with the set of resource to UE-B. Then, UE-B knows how to use the set of resources based on the information.

    - the set of resources non-preferred for UE-B’s transmission (Black resource set)

In this type, UE-A identifies sidelink resources reserved by other UEs or with higher interference, and then include the identified sidelink resources in the set of resource (Condition 1-B-1). Accordingly, UE-B can exclude the set of resource from its resource selection. Besides, since UE-A performs sensing and identifies the sidelink resources via comparing measured RSRP and RSRP threshold based on a specific TX priority, UE-A can deliver such information with the set of resource to UE-B. Then, UE-B knows how to use the set of resources based on the information. For instance, if UE-B has available sidelink data with higher priority than the specific TX priority, some sidelink resources in the set may be still available for resource selection.

Moreover, UE-A may include some sidelink resources, where UE-A does not expect to perform SL reception, in the set of resource (Condition 1-B-2). It is because UE-A cannot perform transmission and reception simultaneously. For these kind of sidelink resources, it is not feasible for UE-B to perform sidelink transmission to UE-A. One possible way is that UE-A indicates higher priority to these kind of sidelink resources.

Based on above, we propose to provide additional information for assisting following UE-B’s behavior in its resource (re-)selection.

Proposal 2:  For inter-UE coordination information triggering based on condition(s), UE-A provides additional information with the set of resource, e.g. TX priority, or RSRP used in sensing procedure.

Moreover, as described in WID Justification, SL DRX can be supported for power saving, such that UE-A may receive PSCCH/PSSCH during SL DRX active time and not receive PSCCH/PSSCH during SL non-active time. From our aspect, SL DRX can be one consideration for deriving the resource set, and impact of SL DRX can be considered in Condition 1-A-2 and Condition 1-B-2.

Proposal 3:  For inter-UE coordination information in scheme 1, UE-A determines the set of resource with consideration of its SL DRX active time or non-active time.

In RAN1 #107bis-e meeting [3], it is agreed that unicast is used for UE-A to transmit inter-UE coordination information transmission triggered by UE-B’s explicit request. The UE-B may transmit the explicit request for its data packets, and then selects resources based on the inter-UE coordination information. Considering UE-B’s data packet may be higher priority or HARQ feedback enabled, reliability and latency of the inter-UE coordination information transmission should be guaranteed to avoid impact on UE-B data delivery. Thus, it should support to enable sidelink HARQ feedback for inter-UE coordination information transmission, at least for explicit request case.

Proposal 4:  Sidelink HARQ feedback enabled is supported for inter-UE coordination information transmission at least for explicit request case.

When UE-B receives inter-UE coordination information from UE-A, following UE-B’s behaviour in its resource (re-)selection is discussed with possible options. For the case of preferred resource set, UE-B will select sidelink resources for its transmissions based on the received preferred resource set. In order to acquire benefit of inter-UE coordination, following UE-A behaviour shall be handled after UE-A sending inter-UE coordination information to UE-B. Especially if UE-A is intended destination UE of a TB transmitted by UE-B, UE-A shall attempt to perform sidelink reception at least in slots of the preferred resource set. For instance, after UE-A sending inter-UE coordination information, UE-A does not select candidate resources in slots of the preferred resource set in its resource selection later. Alternative way is that if UE-A selects sidelink resource in slots of the preferred resource set in its resource selection later, UE-A can perform prioritization between its data priority and specific TX priority used for inter-UE coordination information. Based on the prioritization result, UE-A performs corresponding sidelink transmission or reception.

Proposal 5:  In scheme 1, after UE-A sends UE-B preferred resource set for transmission, 
UE-A can prevent from sidelink transmission in slots of the preferred resource set.

- Conclusion

In this contribution, we have following proposals:

Proposal 1:  Comparing to slot offsets t1 and t2 indicated by TRIV in SCI, the TRIV for indicating resource set indicates slot offset t’1 and t’2, wherein t’1 = t1 -1 and t’2 = t2 -2.

Proposal 2:  For inter-UE coordination information triggering based on condition(s), UE-A provides additional information with the set of resource, e.g. TX priority, or RSRP used in sensing procedure.

Proposal 3:  For inter-UE coordination information in scheme 1, UE-A determines the set of resource with consideration of its SL DRX active time or non-active time.

Proposal 4:  Sidelink HARQ feedback enabled is supported for inter-UE coordination information transmission at least for explicit request case.

Proposal 5:  In scheme 1, after UE-A sends UE-B preferred resource set for transmission, 
UE-A can prevent from sidelink transmission in slots of the preferred resource set.

- Reference

- RP-212880

- Report of 3GPP TSG RAN1#107-e meeting

- Report of 3GPP TSG RAN1#107bis-e meeting