3GPP TSG RAN WG1 #108-e			                                                                 R1-2201843

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	5

Source: 	CMCC

Title: 	Discussion on RAN2 LS on Stage 2 description for Coverage Enhancements

Document for:	Discussion and Decision

Introduction

In LS [1] from RAN2, it was requested for RAN1 reviewing the stage 2 running CR for NR coverage enhancement. In this contribution, we provide our views on the text of the running CR.

Discussion

The stage 2 running CR for NR coverage is copied below.

- 18 Support for NR coverage enhancements To improve NR uplink coverage for both FR1 and FR2 as well as TDD and FDD, the following enhancements on PUSCH, PUCCH and MSG3 PUSCH are supported: - For PUSCH repetition Type A, the maximum number of repetitions is increased up to 32, applicable to both PUSCH transmission with and without dynamic grant. In addition, counting based on available slots is supported, the increased maximum number of repetitions for counting based on available slots and counting based on physical slots are both 32. - TB processing over multi-slot is supported for PUSCH transmission with and without dynamic grant. For a single transmission of TB processing over multi-slot PUSCH, the TB size is based on all the allocated REs across the multiple slots, and the number of slots is counted based on the available slots for UL transmission. In addition, repetition of TB processing over multi-slot PUSCH is also supported. - DMRS bundling is supported for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multi-slot PUSCH and for PUCCH repetitions of PUCCH format 1, 3, 4. - Dynamic PUCCH repetition factor indication configured per PUCCH resource is introduced, applicable to all PUCCH formats. - PUSCH repetition Type A for MSG3 transmission is supported on both NUL and SUL, applicable to 4-step CBRA. If configured, the UE requests MSG3 repetition via separate PRACH resource when the RSRP of DL path-loss reference is lower than a configured threshold. Editor’s Note: The support for repetition of CFRA PUSCH is FFS, depending on whether the work assumption made in RAN1#107-e meeting that support repetition for CFRA PUSCH is confirmed in RAN1 or not.

The texts in the running CR seem clear and matching with the conclusions form the meeting. But for the last bullet of PUSCH repetition Type A for Msg3, the “configured” in the 2nd sentence may bring some confusions. Since the PUSCH repetition Type A for Msg 3 would be used during the UE initial access procedure. Before the initial access procedure, there won’t be any “RRC configuration” procedure from gNB to UEs. Considering that the subject of this sentence is UE, “if supported” is more proper to cover the case that if the UE has the capability of supporting the Msg 3 repetition, it can request Msg 3 repetition during the initial access procedure. While during the RRC connected mode, there would also be RRC configurations for Msg 3 repetition. Since “if supported” is more general and could cover both procedure of initial access and RRC connected, we suggest to replace the “configured” with “supported”.

Proposal 1:

Considering “if supported” is more general and could cover both procedure of initial access and RRC connected, we suggest to replace the “configured” with “supported” for the last bullet. The updates are as below.

- PUSCH repetition Type A for MSG3 transmission is supported on both NUL and SUL, applicable to 4-step CBRA. If configured supported, the UE requests MSG3 repetition via separate PRACH resource when the RSRP of DL path-loss reference is lower than a configured threshold.

Conclusions

In this contribution, we provide our views on the text of the running CR. The proposal is as below.

Proposal 1:

Considering “if supported” is more general and could cover both procedure of initial access and RRC connected, we suggest to replace the “configured” with “supported” for the last bullet. The updates are as below.

- PUSCH repetition Type A for MSG3 transmission is supported on both NUL and SUL, applicable to 4-step CBRA. If configured supported, the UE requests MSG3 repetition via separate PRACH resource when the RSRP of DL path-loss reference is lower than a configured threshold.

References

- [1] R1-2200879, LS on Stage 2 description for Coverage Enhancements, RAN2, 3GPP TSG RAN WG1 #108-e, e-Meeting, February 21st – March 3rd, 2022.