3GPP TSG RAN WG1 #108-e			R1-2201858

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.5.3

Source: 	CMCC

Title: 	Remaining issues on DL-AoD enhancements

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, there were still some remaining issue on the definition of path DL PRS RSRP and the Rx beam index indication of which the DL PRS RSRP or DL PRS RSRPP measurements are performed. Companies’ views on these issues should be further aligned. We provide our views in this contribution.

Remaining issues on DL-AoD enhancements

## Normalization of RSRPP measurement

In RAN1#106b-e meeting, the definition of path DL PRS RSRP for ith path delay was agreed with two FFS bullets. The 2nd FFS bullet was closed with the following conclusion made in RAN1#107-e meeting. There was still a remaining issue on whether the path RSRP measurement is normalized with PRS RSRP.

Agreement (RAN1#106b-e): The measured path DL PRS RSRP for ith path delay is defined as the power of the received DL PRS signal configured for the measurement at the ith path delay of the channel response, and - path DL PRS RSRP for 1st path delay is the power corresponding to the first detected path - FFS: Whether the path RSRP measurement is normalized with PRS RSRP. - FFS: Whether the definition of the ith path delay (other than i=1) is required. - Note: UE may choose to use a time window to compute path DL PRS RSRP by UE implementation (there is no impact to specifications managed by RAN1 for this) - Note: This does not imply that the path delay has to be reported in DL-AoD positioning - Send LS to RAN4 to check the details of the definition and feedback if they identify any update is necessary Conclusion (RAN1#107-e) The current definition of measured path DL PRS RSRP for ith path delay is sufficient and will not be discussed further in AI 8.5.3 - Note: discussion in other agenda items, if necessary, is not precluded.

During the discussion, except that some companies preferred to let RAN4 handle this issue, most companies were supportive of reporting the normalized DL PRS-RSRPP in the measurement reporting. The latest proposal at the end of the RAN1#107-e meeting was recapped:

Proposal 1.2c: for the reporting of DL-PRS RSRPP, the relative power of DL PRS RSRPP to a DL PRS RSRP is reported. • FFS: whether the PRS RSRP used for is for the same PRS resource or can be from a different PRS resource.

The main divergence was what is the reference of the DL PRS RSRPP normalization. Apparently, reporting the relative power of DL PRS RSRPP to the DL PRS RSRP for the same resource is beneficial for the LMF to identify the DL AoD from LOS path. From the accuracy improvement point of view, seems no additional benefit can be obtained by using the DL PRS RSRP from a different PRS resource as a normalization reference. Some companies argued that using PRS RSRP from a different PRS may have benefit in reporting overhead. We believe that this lies in the scope of RAN2 on how to define the measurement reporting content.

Proposal 1: From RAN1’s perspective, the relative power of DL PRS RSRPP to the DL PRS RSRP from the same DL PRS resource is reported.

- Send LS to RAN2 and RAN4 to check in case they have concerns.

## Extension of Rx beam index indication

During the discussion in the last RAN1 meeting, an intermediate proposal regarding the Rx beam index indication was proposed as follows [1]:

Proposal 2.2: - The UE may indicate which DL PRS RSRP or DL PRS RSRPP measurement from PRS resources in the same PFL associated with the same rx beam index if there are at least 2 DL PRS RSRP or 2 DL PRS RSRPP associated with this Rx beam in the PFL. - The LMF may request the UE to perform multiple RSRP or RSRPP measurements with the same rx beam

In Rel-16, the DL PRS-RSRP measurement reporting was specified as:

The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex  [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set.

It is noted that the DL PRS-RSRP measurements associated with the same Rx beam index is within one DL PRS resource set. During the discussion, some companies argued that the Rel-16 restriction within one resource set should be updated to DL PRS-RSRP or DL PRS RSRPP measurements within the same PFL, so that the LMF does not get confused when the DL PRS-RSRP or DL PRS-RSRPP measurements associated with the same Rx beam index are measured from DL PRS resources from different resource sets.

In our views, however, we don’t see the necessity to support the above proposal. In Rel-17, the intention to extend the number of DL PRS-RSRP or DL PRS-RSRPP measurements to be up to 24, is to allow the TRP to use finer beams toward the UE, or to allow the LMF obtain a better understanding of Tx-Rx beam alignment if the measurements are corresponding to multiple Rx beams, so that the DL-AoD positioning accuracy can be further improved. In such cases, if a UE reports DL PRS-RPRP or DL PRS-RSRPP measurements that are taken from two different DL PRS resource sets, one with wide beams and the other with narrow beams, the LMF can use the results from the resource set with finer beams. In addition, even if a UE reports, e.g., 16 DL PRS-RSRP or DL PRS-RSRPP measurements, 8 from resource set 1 and 8 from resource set 2, which have the same beam width, we don’t see the ambiguity of the LMF to identify the DL-AoD with Rel-16 mechanism. Therefore, we propose that:

Conclusion: No need to extend the Rx beam index indication of DL PRS-RSRP measurement reporting defined in Rel-16.

Conclusions

In this contribution, we provide our views on the remaining issues on DL-AoD enhancements, and the following proposals are made:

Proposal 1: From RAN1’s perspective, the relative power of DL PRS RSRPP to the DL PRS RSRP from the same DL PRS resource is reported.

- Send LS to RAN2 and RAN4 to check in case they have concerns.

Conclusion: No need to extend the Rx beam index indication of DL PRS-RSRP measurement reporting defined in Rel-16.

References

- Draft report of 3GPP RAN1#107-e meeting v0.1.0