3GPP TSG RAN WG1 #108-e											R1-2201882

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.1

Title:	Discussion on FeMIMO UE features

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In the previous several meetings, a list of preliminary UE features for NR FeMIMO has been achieved for further discussion. In this contribution, we present our views on UE features for NR FeMIMO and proposals for moving forward.

- Discussion on features for Unified TCI

In RAN1#107bis-e meeting [1], the following 14 components for UE feature of Unified TCI have been achieved as a starting point for further discussion.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI [with joint DL/UL TCI update] for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) FFS: whether to include the following components 2-14 into this FG or one or more separate FGs FFS: Whether basic FGs are defined, and if so, which components are basic FGs, i.e., a UE that supports FG 23-1-1 must also support said basic FGs FFS: basic FGs for UEs supporting CA FFS: separate FGs for inter/intra/joint/separate - Common multi-CC TCI update and activation - For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - The maximum number of MAC-CE activated joint TCI states across all CCs [in a band] [in a band combination] a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC Note: agree component, final wording may change (e.g., when this is merged with other components/FGs) - Maximum number of CCs configured with BFR FFS whether this is a component or just a note in the FG to reuse R16 signaling - Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, and SRS reusing the Rel-15/16 signaling/configuration design(s) Note: This has no impact on detail signaling design for SRS TCI indication - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination] FFS: Whether to make component 9 a prerequisite or merge with 9 - [Alt. 1: [The maximum number of PDSCH-Configs containing TCI states that can referred to from a PDSCH-Config without TCI states Alt. 2: Support PDSCH-Config which contains a reference to another CC/BWP, in which the PDSCH-Config contains the TCI state list] [14. The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For component 6 and 8, we suggest to remove the bracket.

For component 10, we suggest to remove it. Reuse R16 signaling is sufficient.

We think component 2, 9 ,12 ,13 are related with common TCI state update for multi-CC, and these components can be a separate FG. For the TCI state pool for CA, two options have been agreed. Option 1 is to share a single TCI state pool for the set of configured CCs, and option 2 is to configure RRC TCI state pool per individual CC. We propose to do not merge component 2 and component 9, and do not make component 9 as a prerequisite for component 12. Regarding component 13, we suggest to adopt Alt2. The description of Alt1 is overlapped with component 12.

Besides, we think if a UE support unified TCI, it should support both joint TCI and separate TCI. The FG of Unified TCI with separate DL/UL TCI update should be discussed.

Proposal 1: For FG 23-1-1, suggest to adopt the following changes.

- For component 6 and 8, remove the bracket

- Remove component 10

- Put component 2, 9 ,12 ,13 for a separate FG for common TCI state update

- For component 13, remove Alt1

- Discussion on features for Multi-TRP PDCCH repetition

In RAN1#107bis-e meeting [1], the following FGs for Multi-TRP PDCCH repetition have been achieved as a starting point for further discussion.

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates FFS: 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  | Component 2 candidate values: details 2 or 3 Component 4 candidate values: [{0,1,2,3}] [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] [Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps is counted as one.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] | [Note: [If 2 is reported in component 2 of FG 23-2-1,] the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] | 23-2-1 |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  | Optional with capability signalling |

Basically, the structure of these FGs is proper for Multi-TRP PDCCH repetition. However, PDCCH repetition for Type 3 CSS will not require additional complexity comparing with PDCCH repetition for USS. Therefore, there is no need to introduce additional capability for Type 3 CSS PDCCH repetition. The text of “including PDCCH repetition for Type 3 CSS” in the blanket of FG 23-2-1 should be included and the FG 23-2-3 should be deleted.

Proposal 2: Delete FG 23-2-3.

Besides, both FG 23-2-1and FG 23-2-2 are only used for PDCCH repetition, not SFN PDCCH. Therefore, the text of “with non-SFN scheme” in the blanket of FG 23-2-1and FG 23-2-2 should be included.

Proposal 3: Keep the text of “with non-SFN scheme” in FG 23-2-1 and FG 23-2-2.

In addition, for FG 23-2-1a, we think the number of BDs should not be tied with whether the individual PDCCH candidate is monitored or not for the case of overlapping. Because even if 3 BDs is reported in component 2, it cannot be mandatory that the UE can perform two individual decoding and one soft combing. Some UE may still need 3BDs for one individual decoding and one soft combing, so whether the individual candidate can be monitored or not should be reported even if 3BDs is reported as required number of BDs for the two PDCCH candidates. Therefore, the note in FG 23-2-1a should be deleted.

Proposal 4: Delete the note in FG 23-2-1a.

Based on the above analysis, we suggest to adopt the following changes using red colour for the FGs for Multi-TRP PDCCH repetition:

Proposal 5: Adopt the following changes using red colour for the FGs for Multi-TRP PDCCH repetition:

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates FFS: 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  | Component 2 candidate values: details 2 or 3 Component 4 candidate values: [{0,1,2,3}] [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] [Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps is counted as one.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] | [Note: [If 2 is reported in component 2 of FG 23-2-1,] the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] | 23-2-1 |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  | Optional with capability signalling |

- Discussion on features for Multi-TRP PUSCH repetition

In RAN1#107bis-e meeting [1], the FG 23-3-1 and FG 23-3-1-1 have been agreed for M-TRP PUSCH.

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A)[-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) ] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) ] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) [-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Same as the structure of Rel-15 capability (FGs 2-14, 2-15, 2-15a, 2-15b), separate capabilities for codebook-based and non-codebook are required for M-TRP PUSCH transmission. In RAN1#103e meeting, the maximum number of SRS resource sets to two is supported for M-TRP PUSCH transmission. For codebook based PUSCH transmission, the following components should be taken into consideration:

- Supported max number of SRS resource per set (SRS set use is configured as for codebook).

- Supported max number of SRS resource set (SRS set use is configured as for codebook).

Similarly, for non-codebook based PUSCH transmission, the following components should be taken into consideration:

- Supported max number of SRS resource per set (SRS set use is configured as for non-codebook).

- Supported max number of SRS resource set (SRS set use is configured as for non-codebook).

Proposal 6: Separate capabilities for codebook-based and non-codebook based M-TRP PUSCH transmission.

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A)[-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] 4. Supported max number of SRS resource per set (SRS set use is configured as for codebook). 5. Supported max number of SRS resource set (SRS set use is configured as for codebook). |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A)-NCB | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] 4. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook). 5. Supported max number of SRS resource set (SRS set use is configured as for non-codebook). |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) [-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources 2. Supported max number of SRS resource per set (SRS set use is configured as for codebook). 3. Supported max number of SRS resource set (SRS set use is configured as for codebook). |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-3 | Multi-TRP PUSCH repetition (type B) -NCB | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources 2. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook). 3. Supported max number of SRS resource set (SRS set use is configured as for non-codebook). |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

- Discussion on features for Multi-TRP PUCCH repetition

In RAN1#107bis-e meeting [1], the FG 23-3-2, 23-3-x, 23-3-2b, and 23-3-2c have been agreed for M-TRP PUCCH.

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition-inter-slot | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions ] [2. Support of up to two PUCCH spatial relation per PUCCH resource] 32. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 43. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-x | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2b |  | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS |  |  |  |  |  |  |  | FFS: candidate values | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c |  | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource should be basic feature for UE supported M-TRP PUCCH inter or intra-slot repetition. So, component 2 in FG 23-3-2 should be modified to “Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource” and remove the corresponding bracket. This component should be also added in FG 23-3-x as a basic component.

Proposal 7: Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource as basic feature in FG 23-3-2 and FG 23-3-x.

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition-inter-slot | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions ] [2. Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource] 32. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 43. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-x | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

- Discussion on features for Multi-TRP CSI

In RAN1#107bis-e meeting [1], the following FGs have been achieved as a starting point for further discussion on Multi-TRP CSI:

| Features | Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] | FFS | [Per band and per BC] | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both] Component 5 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |  |  | Component 1 candidate value set: { X=1, X=2} |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  | [{4, 8, 12, 16, 24, 32}] |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Per band |  |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 | Per band | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 |

Firstly, there are two ways to handle the component of maximum number of NZP CSI-RS resources in a resource set for Multi-TRP CSI. One way is to separate it as another FG (FG 23-7-3) and determine the candidate value Ks,max in this FG. However, taking this way means that we also need to decide a default value for Ks,max  in case that the optional FG is not reported. The other way is to put this component in as FG 23-7-1 as a basic feature. In our view, it is much simpler and straighter to keep the component of maximum number of NZP CSI-RS resources in a resource set for Multi-TRP CSI as a part of FG 23-7-1.

Proposal 8: Delete FG 23-7-3.

In addition, we think the minimum value of Ks,max should be Ks,max = 4. At least in FR2, if the CMR sharing between Single-TRP and NCJT is not supported, at least 4 resources are needed in resource set. Two NZP CSI-RS resources are used for NCJT measurement hypothesis, while the other two NZP CSI-RS resources are used for two different Single-TRP measurement hypotheses. Therefore, the candidate value set in component 2 of FG 23-7-1 should be {4, 5, 6, 7, 8}.

Proposal 9: The candidate value set in component 2 of FG 23-7-1 should be {4, 5, 6, 7, 8}.

Besides, the total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement hypothesis should be twice as the number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis, since the resources pair associated with one NCJT measurement hypothesis consist of two resources and the two resources have the same Tx ports. It means in component 4 of FG 23-7-1 the number of g) is twice as the number of b) in the combination. Therefore, we suggest to delete the “g) Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement” in component 4 of FG 23-7-1 since the number of g) is useless.

Proposal 10: Delete the “g) Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement” in component 4 of FG 23-7-1.

Based on the above analysis, we suggest to adopt the following changes using red colour for the FGs for Multi-TRP CSI:

Proposal 11: Adopt the following changes using red colour for the FGs for Multi-TRP CSI:

| Features | Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] | FFS | [Per band and per BC] | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both] Component 5 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |  |  | Component 1 candidate value set: { X=1, X=2} |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per resource/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per resource in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  | [{4, 8, 12, 16, 24, 32}] |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Per band |  |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 | Per band | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 |

- Conclusion

Proposal 1: For FG 23-1-1, suggest to adopt the following changes.

- For component 6 and 8, remove the bracket

- Remove component 10

- Put component 2, 9 ,12 ,13 for a separate FG for common TCI state update

- For component 13, remove Alt1

Proposal 2: Delete FG 23-2-3.

Proposal 3: Keep the text of “with non-SFN scheme” in FG 23-2-1 and FG 23-2-2.

Proposal 4: Delete the note in FG 23-2-1a.

Proposal 5: Adopt the following changes using red colour for the FGs for Multi-TRP PDCCH repetition:

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates FFS: 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  | Component 2 candidate values: details 2 or 3 Component 4 candidate values: [{0,1,2,3}] [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] [Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps is counted as one.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] | [Note: [If 2 is reported in component 2 of FG 23-2-1,] the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] | 23-2-1 |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  | Optional with capability signalling |

Proposal 6: Separate capabilities for codebook-based and non-codebook based M-TRP PUSCH transmission.

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A)[-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] 4. Supported max number of SRS resource per set (SRS set use is configured as for codebook). 5. Supported max number of SRS resource set (SRS set use is configured as for codebook). |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A)-NCB | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] 4. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook). 5. Supported max number of SRS resource set (SRS set use is configured as for non-codebook). |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) [-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources 2. Supported max number of SRS resource per set (SRS set use is configured as for codebook). 3. Supported max number of SRS resource set (SRS set use is configured as for codebook). |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-3 | Multi-TRP PUSCH repetition (type B) -NCB | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources 2. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook). 3. Supported max number of SRS resource set (SRS set use is configured as for non-codebook). |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Proposal 7: Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource as basic feature in FG 23-3-2 and FG 23-3-x.

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition-inter-slot | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions ] [2. Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource] 32. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 43. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-x | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of up to two PUCCH spatial relation / up to two sets of power control parameters per PUCCH resource | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

Proposal 8: Delete FG 23-7-3.

Proposal 9: The candidate value set in component 2 of FG 23-7-1 should be {4, 5, 6, 7, 8}.

Proposal 10: Delete the “g) Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement” in component 4 of FG 23-7-1.

Proposal 11: Adopt the following changes using red colour for the FGs for Multi-TRP CSI:

| Features | Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] | FFS | [Per band and per BC] | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both] Component 5 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |  |  | Component 1 candidate value set: { X=1, X=2} |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per resource/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per resource in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  | [{4, 8, 12, 16, 24, 32}] |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Per band |  |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 | Per band | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 |

- References

- 3GPP RAN1#107bis-e RAN1 Chairman’s Notes, Jan. 2022.