3GPP TSG RAN WG1 #108                                              R1- 2201908

e-Meeting, Feb 21st – Mar 3rd, 2022

Source:	NEC

Title:	Remaining Issues on Group Scheduling Mechanisms for RRC_CONNECTED UEs

Agenda Item:	8.12.1

Document for: 	Discussion and Decision

- Introduction

The work items for NR support of Multicast and Broadcast Service were approved in RAN #86. After several RAN1 e-meetings involved the discussion about mechanisms of group scheduling for UEs in RRC_CONNECTED state, the basic functions for it can be achieved, but some remaining issues are left which will be solved in Rel-17 maintenance stage.

According to the agreements and conclusions achieved in RAN1 #107b e-meeting and previous e-meetings, one issue will be discussed in this contribution:

- HARQ process number management across unicast service and multicast service

- HARQ process number management across unicast service and multicast service

In previous RAN1 e-meetings, it was agreed that the maximum number of HARQ process per cell is kept unchanged for UE to support multicast reception, and there is no intention to separate the HARQ process ID for multicast from current unicast service. Recording the HARQ process number, the agreements and conclusion achieved in the previous e-meetings are listed below[1].

Agreement: For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast. Agreement: The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB. Conclusion: The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception. - How to allocate HARQ processes between unicast and multicast is up to gNB.

In RAN1 #107 e-meeting, how to determine LBRM/TBS for PTP retransmission of multicast was discussed, and two options were given[2]. One (Option1) is that the determination of LBRM/TBS is based on the LBRM/TBS determination of the PTM initial transmission using the same HPID and NDI. Another (Option2) is that the determination of LBRM/TBS based on the LBRM/TBS determination of the legacy unicast PDSCH transmission.

Option2 was selected as the final rule since the opponents of Option1 argue that it is impossible for UE to determine whether this is PTP retransmission for multicast or PTP transmission for unicast, especially when the PTM initial transmission is missed by the UE according to the current mechanisms. The issue regarding how to differentiate PTP (re)transmission for unicast and PTP retransmission for multicast was first raised in RAN1 #105 e-meeting. This illustration for it is shown in Figure 1. For the same HARQ process, different UEs in an MBS group have different PTP scheduling, thus the NDI states are different for UE1 and UE2 (t1). When PTM scheme 1 is used for multicast initial transmission, its NDI toggle should be relative to the NDI in a previous DCI with the same G-RNTI (t2). Then PTP is used for the retransmission of PTM1 with the same NDI value as initial transmission. However, for UE2, if the initial PTM1 transmission was miss detected, when UE2 detects a DCI scheduling the retransmission of TB3 with NDI not toggled, it may make incorrect soft-combination between the retransmission TB with previous unicast transmission (t3). This issue was discussed in the last two meetings, but there are no conclusions for it.

Figure 1

Since there is no intention to separate the HARQ process number for multicast service from the current maximum 16 HARQ processes used for unicast service, and the total maximum number of HARQ process is kept unchanged for UE to support the multicast reception, it leads to that a given HPN can be used for unicast now, and also can be used for multicast later even the expected transmission of HARQ-ACK feedback for that HARQ process is not receive. However, for the scenario illustrated in Figure 1, the UE sides, especially for UE2, must know whether the transmission is for unicast service or multicast. The initial transmission may be differentiated by RNTI type, i.e., schedules unicast service scrambled by C-RNTI and schedules multicast service scrambled by G-RNTI, but the retransmission cannot be differentiated by RNTI type since PTP retransmission can be supported for multicast service.

This issue was discussed in the last e-meeting, the updated proposal can be refer to[3]

Updated proposal 4-2: For multicast, UE can be configured to enable receiving PTP retransmission for multicast PTM initial transmission with same HPID by RRC signaling. If UE is not configured to enable receiving PTP retransmission for multicast PTM initial transmission with same HPID, UE assumes only PTM is used for retransmission for multicast. - The configuration is per CFR

Several companies argued this proposal cannot solve the issue essentially. Considering it is difficult to converge in a compact time, the moderator suggested deprioritize the discussion in this e-meeting.

In our opinion, a direct way is to use a certain field in scheduling DCI to indicate the service type, i.e., unicast (re)transmission or multicast retransmission. Since it was agreed in the last e-meeting that the unused DCI field, like ‘Scell dormancy indication’ and ‘BWP indicator’ doesn’t be included in DCI format 4_2, a new DCI field should be included in that DCI. If it is different to converge in this meeting, it should be gNB implemented to avoid the same HARQ process being used for unicast PTP transmission and multicast PTP transmission.

Proposal 1:

- Introduce a new DCI field to differentia PTP (Re)Tx for unicast or PTP ReTx for multicast.

  - If it is different to converge in this meeting, it should be gNB implemented to avoid the same HARQ process being used for unicast PTP (Re)Tx and multicast PTP ReTx.

- Conclusion

In this contribution, we discuss the remaining issues for group scheduling mechanisms for RRC_CONNECTED UEs. Proposal is summarized as follows:

Proposal 1:

- Introduce a new DCI field to differentia PTP (Re)Tx for unicast or PTP ReTx for multicast.

  - If it is different to converge in this meeting, it should be gNB implemented to avoid the same HARQ process being used for unicast PTP (Re)Tx and multicast PTP ReTx.

References

- 3GPP RAN1 #106b e-meeting, chairman’s notes.

- R1-2112542, Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS, Moderator, RAN1 #107.

- R1-2200733, Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS, Moderator, RAN1 #107b.