3GPP TSG RAN WG1 #108-e																			 R1-2201922

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.4.2

Source:		Xiaomi

Title:	Remaining issues on UL time and frequency synchronization for NTN

Document for:	Discussion

Introduction

In RAN1#107e [1], the following were agreed for uplink timing and frequency synchronization for NTN.

Agreement

The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time.

Agreement

A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB.

Working assumption

Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation and [TACommonThirdOrder] are indicated with the following range, granularity and bits allocation:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommon | 0 ...66485757 (i.e: 0… 270.73 ms) | 4.07×10-3 μs | 26 bits |
| TACommonDrift | - 261935… + 261935 (i.e: --53.33 μss… +-53.33 μss) | 0.2×10-3μss | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 μss2) | 0.2×10-4μss2 | 15 bits |
| [TACommonThirdOrder] | -4912…+4912 (-0.015 μss3…+0.015 μss3) | 0.3×10-5μss3 | 14 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Agreement

NTN validity duration is configured per cell and indicated to the UE in X bits with:

- Value range { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, Infinity}

- Unit is second

- FFS (to be resolved in current meeting): Additional values for GEO

Agreement

Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network:

  - Position and velocity state vector ephemeris format is 17 bytes payload.

    - The field size for position (m) is 78 bits

      - Position range is driven by GEO : +/- 42 200 km

      - The quantization step is 1.3m for position

    - The field size for velocity (m/s) is 54 bits

      - Velocity range is driven by LEO@600 km: +/- 8000 m/s

      - The quantization step is 0.06 m/s for Velocity

  - Orbital parameter ephemeris format 18 byte payload

    - Semi-major axis α (m) is 33 bits

      - Range: [6500, 43000]km

    - Eccentricity e is 19 bits

      - Range: ≤ 0.015

    - Argument of periapsis ω (rad) is 24 bits

      - Range: [0, 2π]

    - Longitude of ascending node (Ω rad) is 21 bits

      - Range: [0, 2π]

    - Inclination i (rad) is 20 bits

      - Range: [- π/2 , + π/2]

    - Mean anomaly M (rad) at epoch time to is 24 bits

      - Range: [0, 2π]

Agreement

The reference point of the epoch time for assistance information (i.e. Serving satellite ephemeris and Common TA parameters) should be known by UE.

- FFS: the definition of the reference point

Conclusion

NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay, which is calculated using the UE position and the serving satellite ephemeris.

- How the UE calculates/updates NTA, UE-specific is left to UE implementation.

Agreement

Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common  calculation as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2

- TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e

- Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset.

- NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

Agreement

Confirm the Working assumption on granularity and bits allocation for Common TA parameters: Value range, granularity and bits allocation of Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are as follows:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
|  | 0 ...66485757 (i.e: 0… 270.73 ms) |  | 26 bits |
| TACommonDrift | - 261935… + 261935 (i.e: --53.33 … +-53.33 ) |  | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 ) |  | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Agreement

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

Agreement

The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

Working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

Where, TA is the TAC field in msg2/msgB

Conclusion

DL frequency compensation by gNB for the service link Doppler is not supported in Release 17.

In this contribution, we discuss the remaining issues of UL time and frequency synchronization in NTN.

UL timing and frequency synchronization

Combination of open and closed loop TA control

Open-loop TA was introduced besides close-loop TA in NTN, and the open-loop TA is composed by UE-specific TA and common TA. Wherein, the UE-specific TA is affected by both satellite ephemeris and UE GNSS location, while common TA mainly affected by the satellite ephemeris. The close-loop TA is zero when UE initiate the access procedure. Therefore, RAN4 defines the initial transmit timing error requirement for initial access. Which includes the legacy timing error (Te), GNSS accuracy (Te_GNSS) and the serving satellite position estimation error(Te_SAT), and can be expressed by Te_NTN = Te + Te_GNSS + Te_SAT.Note that it is RAN1’s working assumption that no offset beween the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

In terrestrial network, when the transmission timing error between UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te, in condition that TA value unchanged. While in NTN, any one of the following factor will cause TA value changing.

- UE received new TA command

- UE GNSS location update

- Satellite ephemeris update

- Common TA update with time

Double correction is mainly caused by UE GNSS location update as TA command, satellite ephemeris and common TA are controlled/known to the gNB. However, when would the UE update the GNSS location and/or how much would the location changing affect the UE-specific TA, is unknown to the gNB. And the double correction will cause larger TA error with larger UE GNSS location changing. Therefore, as long as the TA changing caused by GNSS locating changing is reduced, the influence of double-correction on TA can be effectively reduced.

Based on RAN4’s LS [2], if option 1 is adopted, Te_NTN applies to all the uplink channels.  That is, if the difference between UE's uplink timing and reference timing is Δd, Δd should be less than Te_NTN. If it exceeds the Te_NTN, UE needs to adjust TA to meet Te_NTN.  This is an implicit indication that UE needs to update GNSS frequently to make the whole TA error less than Te_NTN.  If following Option 2, similar to gradual timing adjustment requirement, RAN4 can specify the gradual TA value to be adjusted when UE's GNSS changes greatly. Based on the analysis above, the double-correction issue can be solved by RAN4 and which option to be choose should be decided in RAN4.

Proposal 1: The solution to resolve the issue on combination of open and closed loop TA control is up to the UE implementation to meet the RAN4’s requirements.

NTN neighbor cell and satellite information

Based on RAN2’s LS [3], RAN2 has agreed the assumption that feeder link delay is known to and compensated by the network. The network can compensate feeder link delay to configure SMTCs to UEs in the connected mode. In addition, RAN2 has agreed for IDLE mode measurements that UE autonomously adjusts the SMTCs based on location and ephemeris. It is FFS whether network assistance information is provided to UEs.

In our understanding, for RRC connected mode, the network configures the appropriate offset in SMTC for UE measurement. While the offset value affected by the propagation delay difference between different satellites, which includes the feeder link delay and service link delay. In assumption that the feeder link delay difference is known to and compensated by the network. Based on the assistance information from UE e.g., the propagation delay difference between different satellites calculated based on UE location and satellite ephemeris, the gNB can configure appropriate offset values for the service link propagation delay difference in the SMTC. And the UE does not need to know the common TA and Kmac of  neighbor satellite/cell. While if the feeder link delay difference is not compensated by the network, the UE needs the common TA parameters to report the total propagation delay difference and the gNB needs to configure a larger offset for the measurement window in SMTC.

For RRC idle mode, UE needs to move the SMTC window based on the propagation delay difference. Same with RRC connected mode, if the feeder link delay difference is known to and compensated by the network, UE only needs to move the SMTC window based on the propagation delay difference of service link.

Based on the discussion above, we think the common TA parameters  of neighbor cells need to be provided to the UEs if the feeder link delay is not compensated by the network. The common TA parameters  of neighbor cells is not needed to the UEs if the feeder link delay is compensated by the network.

For RAN2’s question that whether the validity timer information for serving and neighbour/target cell needs to be different. We think in case that the serving cell and neighbour cell are in different orbits, different validity timer are needed.

For RAN2’s question that whether there will be separate validity timers for PVT parameters and orbital parameters. The validity time could be different for PVT parameters and orbital parameters in principle. However, considering the timer is valid not only for satellite ephemeris, but also for other parameters such as common TA related parameters, Koffset and so on, we think that a single timer is enough.

Proposal 2: The common TA parameters of neighbour cells need to be provided to the UEs if the feeder link delay is not compensated by the network. The common TA parameters  of neighbour cells is not needed to the UEs if the feeder link delay is compensated by the network.

Proposal 3: The validity timer information for serving and neighbour/target cell can be different.

Proposal 4: A single validity timers for PVT parameters and orbital parameters.

Validity timer for GEO

In last meeting, whether to define an additional parameter for GEO is still FFS. In our view, either defining infinity as a parameter of validity timer or assuming UL sync parameters are always valid when no validity timer configured is okay.

Conclusions

In this contribution, we discuss UL time and frequency synchronization for NTN, the following proposals have been made:

Proposal 1: The solution to resolve the issue on combination of open and closed loop TA control is up to the UE implementation to meet the RAN4’s requirements.

Proposal 2: The common TA parameters of neighbour cells need to be provided to the UEs if the feeder link delay is not compensated by the network. The common TA parameters  of neighbour cells is not needed to the UEs if the feeder link delay is compensated by the network.

Proposal 3: The validity timer information for serving and neighbour/target cell can be different.

Proposal 4: A single validity timers for PVT parameters and orbital parameters.

References

- Chair's Notes RAN1#107-e

- R4-2120417 (R1-2200870), Reply LS on combination of open and closed loop TA control in NTN

- R2-2201884 (R1-2200883), Reply LS on NR NTN Neighbor Cell and Satellite Information