3GPP TSG RAN WG1 #108-e			R1-2201931

e-Meeting, February 21st – March 3rd, 2022

Source: 	     Xiaomi

Title:		Remaining issues on group scheduling for RRC_CONNECTED UE

Agenda item:    8.12.1

Document for:  Discussion

Introduction

In RAN1#107bis e-meeting, the mechanisms to support group scheduling for RRC_CONNECTED UE was extensively discussed. In this contribution, we provide our views on remaining issues of group scheduling for MBS. The achieved agreements in RAN1#107bis e-meeting can be found in the appendix.

Discussion

Remaining issues on DCI format 4_1

In RAN1#107 e-meeting, the following agreement on the content of first DCI format was achieved: [1]

Agreement The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by G-RNTI for multicast: - Frequency domain resource assignment - Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of TS38.214 - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 in TS38.212 - Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of TS38.214 - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 in TS38.212 - HARQ process number – [4 or 5] bits - Downlink assignment index – 2 bits as defined in Clause 9.1.3 of TS 38.213, as counter DAI - PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of TS38.213 - PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of TS38.213 - Reserved bits –3 bits - FFS: Some of the fields may be not useful and can be reserved in some conditions, and FFS the details of the conditions - FFS: other fields, e.g. for HARQ enabling/disabling Note: Whether new fields are defined for multicast DCI format 1_0 can be discussed separately. The reserved bits can be used for new fields if needed.

There are three open issues need to be resolved as below:

- Open issue 1: whether the bit length of HPN is 4 bits or 5 bits. First of all, the motivation of extending HPN to 5 bits is to hard split HARQ processes between unicast and multicast, which is against the previous conclusion that the HARQ process should be shared. [1] Secondly, 4 bits for HPN is already captured in the endorsed CR. [2] Hence we don’t see the necessity of extending HPN to 5 bits.

RAN1#104 bis Conclusion: The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception. - How to allocate HARQ processes between unicast and multicast is up to gNB.

- Open issue 2: Although DCI format 4_1 is very basic, the functionality of HARQ feedback is optional. If the HARQ feedback is disabled dynamically or semi-statically, the bit fields related to HARQ feedback should be reserved or ignored by the UE when interpret the received DCI.

- Open issue 3: Considering dynamic HARQ enabling/disabling can be configured for DCI format 4_2, it is OK to only supporting semi-static configuration of HARQ enabling/disabling for DCI format 4_1. Especially the current CR is workable and is the straightforward direction with least additional standard efforts.

Proposal 1: If HARQ feedback is disabled, UE ignores the following information fields when interprets DCI format 4_1:

- HARQ process number

- Downlink assignment index

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

Remaining issues on DCI processing

Currently, the number of DCI scheduling unicast DL data in a slot or monitoring occasion UE can monitor depends on the UE capability, e.g. FG3-1 and FG 3-5b respectively.  For the broadcast DCI scheduling system information/paging/RAR, there is no restriction for reception. MBS DCI is different from both unicast DCI and broadcast DCI which is used to schedule a group of UE and multiple MBS DCI with different G-RNTI is needed. In RAN1#107bis e-meeting, the following agreement was achieved:

Agreement Regarding the number of DCIs that a UE can process in a slot or span, multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

The basic idea is to treat multicast DCI scheduling MBS PDSCH as unicast DCI scheduling legacy PDSCH. However, a Rel-17 MBS UE supports MBS scheduling and unicast scheduling simultaneously, there would be ambiguity on how to process DCIs when MBS DCI and unicast DCI are transmitted within the same time unit. One example is shown in Figure 1 assuming there are three MBS traffic with different periodicity. In this example, FG3-1 and FDD is assumed. It can be observed that UE has to process four DCIs in a MO for the worst case, i.e. 1 unicast UL DCI, 1 unicast DL DCI, 1 MBS DCI for service#1 and 1 MBS DCI for service#3. It would be ambiguous on how to process these 4 DCIs under the umbrella of FG 3-1.

Figure 1: Illustration on the number of processing DCIs within a time unit

In order to avoid the misunderstanding between network and UE, the following rule can be considered when MBS DCI and unicast DCI collides in time domain and the number of expected DCI is larger than the cap defined by the UE capability:

- UE processes DCI in CSS firstly.

  - The CSS with lower SS ID has higher priority.

- If the number of detected multicast DCI does not exceed UE capability, UE continues PDCCH monitoring in USS

Proposal 2: When UE monitors MBS DCI and unicast DCI in the same slot or same span, and the number of expected DCI is larger than the cap defined by the UE capability:

- UE processes DCI in CSS firstly.

  - The CSS with lower SS ID has higher priority.

- If the number of detected multicast DCI does not exceeds UE capability, UE continues PDCCH monitoring in USS.

Remaining issues on HARQ process management

In RAN1#103 e-meeting, three possible transmission schemes for MBS were identified, i.e. PTP transmission scheme, PTM transmission scheme 1 and PTM transmission scheme 2. There is still no consensus on PTM transmission scheme 2. The basic idea of PTM transmission scheme 2 is to use multiple UE-specific DCIs to schedule the same group common PDSCH. The first motivation of PTM scheme 2 is to use separate DCI for different UEs to schedule the MBS PDSCH so that the PDCCH transmission can carter to the channel condition for each UE. The second motivation of PTM scheme 2 is to use individual DCI to indicate UE-specific resources for HARQ-ACK. However, the PDCCH overhead would be much larger than that of PTM transmission scheme 1 as the DCI is redundantly transmitted. On the other hand, the performance of PDCCH transmission under the umbrella of PTM transmission scheme 1 can also be guaranteed via adopting a conservative strategy to deliver group scheduling DCI. For PUCCH resources, it is agreed that up to gNB implementation to guarantee orthogonal PUCCH resources among UEs in the same group.

Proposal 3:  Do not support PTM transmission scheme 2.

It was agreed that the retransmission of PTM transmission scheme 1 can be PTM transmission scheme 1 or PTP transmission scheme. However, views on whether to support PTM transmission scheme 1 retransmission and PTP transmission scheme simultaneously for different UEs in the same group are still divergent. Generally speaking, PTP retransmission is used if small partition of UEs in the same group feedback NACK for multicast PDSCH. More accurate scheduling can be achieved for MBS PDSCH. In this case, retransmission with PTM transmission scheme 1 is redundant. On the other hand, if network decides to re-transmit the failed MBS transmission with PTM transmission scheme 1, it should guarantee the MBS PDSCH is robust for all the UEs in the group. Again the additional PTP-based retransmission is unnecessary.

Proposal 4:  Do not support PTM scheme 1 based retransmission and PTP scheme based retransmission simultaneously for dynamic MBS transmission in the same MBS group.

In the last meeting, it was agreed to further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast. Actually in RAN1#104 e-meeting, we made the following agreement:

Agreement: The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

For a PTP retransmission, the HARQ process ID should be same as that of PTM scheme 1 initial transmission. As the HARQ process is still on going for the PTM scheme1 transmission, the HARQ process ID should not be used for another HARQ process. In the other words, PTP (re)transmission for unicast should be allocated with different HARQ ID.

Observation 1: There is no issue on differentiating the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Remaining issues on MBS SPS configuration

In RAN1#106 e-meeting, it was agreed that one G-CS-RNTI is associated with the SPS-config if a SPS-config for MBS is configured in CFR. However, it is still open whether multiple G-CS-RNTIs associated with one SPS-config can be supported or not. There have been plenty of RNTIs defined in the current specification, e.g. C-RNTI, CS-RNTI, MCS-C-RNTI, G-RNTI, GS-RNTI, etc. Assigning multiple RNTI values for the same functionality will further consuming the finite RNTI values. It will impact not only the Rel-17 UEs but also the Rel-15/16 UEs considering the co-existence scenarios. From this perspective, any intention to allocate redundant RNTI values for the same purpose should be fully verified. However, we don’t see the motivation of supporting multiple G-CS-RNTIs associated with one SPS-config. Furthermore, it was already agreed that multiple SPS configuration can be configured for MBS for RRC_CONNECTED UE supporting MBS which is captured in the following agreement achieved in RAN1#104bis e-meeting:

Agreement: For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total. - It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively.

With the above agreement, the reasoning of supporting multiple G-CS-RNTIs associated with one SPS-config is even more questionable as the flexibility of MBS SPS transmission can be perfectly supported. Last but not least, the calculation of HPN for SPS PDSCH may be carefully investigated if multiple G-CS-RNTIs associated with one SPS-config is supported. Considering the late stage and there are still many essential issues on group scheduling, we don’t prefer to open the door for endless discussion on this trivial issue.

Proposal 5: Do not support multiple G-CS-RNTIs associated with one SPS-config.

Similar to dynamic MBS transmission, there are different views on whether to support PTM transmission scheme 1 based retransmission and PTP transmission scheme based transmission simultaneously for SPS MBS transmission in the same MBS group. As aforementioned, we don’t think it is necessary.

Proposal 6:  Do not support PTM scheme 1 based retransmission and PTP scheme based retransmission simultaneously for SPS MBS transmission in the same MBS group.

In RAN1#107 e-meeting, the following agreement on configuration of G-RNTI was achieved. However, how to configure G-CS-RNTI is still open. For CS-RNTI, it is configured per serving cell group. It is straightforward to configure G-CS-RNTI per serving cell group as well. However, considering MBS is a new feature and the G-RNTI(s) is/are configured per serving cell already, we slightly prefer to use the same mechanism as G-RNTI when configure G-CS-RNTI.

Agreement: For multicast of RRC_CONNECTED UEs, the G-RNTI(s) is/are configured - Opt.2: per serving cell. - FFS G-CS-RNTI(s)

Proposal 7: For multicast of RRC_CONNECTED UEs, the G-CS-RNTI(s) is/are configured per serving cell.

Conclusion

In this contribution, we discuss the remaining issues on group scheduling.  Based on the discussion, we have the following observation：

Observation 1: There is no issue on differentiating the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Furthermore, we have the following proposals:

Proposal 1: If HARQ feedback is disabled, UE ignores the following information fields when interprets DCI format 4_2:

- HARQ process number

- Downlink assignment index

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

Proposal 2: When UE monitors MBS DCI and unicast DCI in the same slot or same span, and the number of expected DCI is larger than the cap defined by the UE capability:

- UE processes DCI in CSS firstly.

  - The CSS with lower SS ID has higher priority.

- If the number of detected multicast DCI does not exceeds UE capability, UE continues PDCCH monitoring in USS.

Proposal 3:  Do not support PTM transmission scheme 2.

Proposal 4:  Do not support PTM scheme 1 based retransmission and PTP scheme based retransmission simultaneously for dynamic MBS transmission in the same MBS group.

Proposal 5:  When a UE is configured with both unicast SPS and MBS SPS, the following mechanisms can be considered to differentiate the unicast PDCCH with CS-RNTI is used to deactivate MBS SPS or unicast SPS:

- Option 1: Use 1 bit in the de-activation DCI scrambled with CS-RNTI to indicate whether the target SPS is MBS SPS or not.

- Option 2: Use HPN value {8-15} in the de-activation DCI scrambled with CS-RNTI to indicate the target MBS SPS index.

Proposal 6: Do not support multiple G-CS-RNTIs associated with one SPS-config.

Proposal 7:  Do not support PTM scheme 1 based retransmission and PTP scheme based retransmission simultaneously for SPS MBS transmission in the same MBS group.

Proposal 8: For multicast of RRC_CONNECTED UEs, the G-CS-RNTI(s) is/are configured per serving cell.

Reference

  - Chairman notes, RAN1#107 e-meeting

Appendix

Agreement

DCI format 4_2 doesn’t include the following fields:

  - Scell dormancy indication

  - BWP indicator

DCI format 4_2 includes the following field (configurable):

  - MCS/NDI/RV for TB2

    - Support of this field is subject to UE capability

Agreement

DCI format 4_2 includes ‘ZP CSI-RS trigger’ field.

Agreement

For DCI size alignment of DCI format 4_2, the size of DCI format 4_2 is configured by RRC signaling for RRC_CONNECTED UEs (similar as the configuration for the size alignment among DCI format 2_0/2_1/2_4/2_5/2_6).

Conclusion

For multicast of RRC_CONNECTED UEs, the value range of sps-ConfigIndex in SPS-Config-Multicast is {0-7}, and sps-ConfigIndex in sps-Config and SPS-Config-Multicast cannot be configured with the same value.

The TP below for Clause 10.1 in TS 38.213v17.0.0 is endorsed.

----------------- Start of TP ----------------

10.1	UE procedure for determining physical downlink control channel assignment

<Unchanged text is omitted>

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, or MCS-RNTI, MCCH-RNTI, G-RNTI, or G-CS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH reception and PSSCH transmission on a same serving cell.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1.2.2 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

5.1.2.2	Resource allocation in frequency domain

<Unchanged text is omitted>

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, DCI format 4_0 or DCI format 4_1, then downlink resource allocation type 1 is used.

If the scheduling DCI is configured to indicate the downlink resource allocation type as part of the 'Frequency domain resource assignment' field by setting a higher layer parameter resourceAllocation in PDSCH-Config to 'dynamicSwitch', for DCI format 1_1 or setting a higher layer parameter resourceAllocationDCI-1-2 in PDSCH-Config to 'dynamicSwitch' for DCI format 1_2 or setting a higher layer parameter resourceAllocation in PDSCH-Config-Multicast to  'dynamicSwitch' for DCI format 4_2, the UE shall use downlink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the downlink frequency resource allocation type as defined by the higher layer parameter resourceAllocation in PDSCH-Config for DCI format 1_1 or by the higher layer parameter resourceAllocationDCI-1-2 for DCI format 1_2 or by the higher layer parameter resourceAllocation in PDSCH-Config-Multicast for DCI format 4_2.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1.2.3 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

<Unchanged text is omitted>

The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of prb-BundlingTypeDCI-1-2 instead of prb-BundlingType as well as vrb-ToPRB-InterleaverDCI-1-2 instead of vrb-ToPRB-Interleaver. The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of prb-BundlingType given by PDSCH-Config-Multicast as well as vrb-ToPRB-Interleaver given by PDSCH-Config-Multicast.

A UE may assume that precoding granularity is  consecutive resource blocks in the frequency domain.  can be equal to one of the values among {2, 4, wideband}.

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous PRBs and the UE may assume that the same precoding is applied to the allocated resource associated with a same TCI state or a same QCL assumption.

<Unchanged text is omitted>

----------------- End of TP ----------------

Agreement

For DMRS of GC-PDSCH,

  - For GC-PDSCH scheduled by a DCI format 4_0/4_1, the UE assumes dmrs-AdditionalPosition = ‘pos2’, similar as that of DCI format 1_0.

  - For GC-PDSCH scheduled by a DCI format 4_2, the UE assumes dmrs-AdditionalPosition in DMRS-Config if configured in PDSCH-Config-Multicast, similar as that of DCI format 1_1.

  - Adopt the following TP for Clause 5.1.6.2 in TS 38.214:

----------------- Start of TP ----------------

5.1.6.2	DM-RS reception procedure

<Unchanged text is omitted>

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB.

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in PDSCH-Config-Multicast instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in PDSCH-Config.

When receiving PDSCH scheduled by DCI format 1_0, 4_0, 4_1 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A and type B, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

When receiving PDSCH scheduled by DCI format 1_1 by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or DCI format 4_2 by PDCCH with CRC scrambled by G-RNTI or G-CS-RNTI,

-	the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Clause 7.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength given by DMRS-DownlinkConfig.

-	if maxLength is set to 'len1', single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0', 'pos1', 'pos2' or 'pos3'.

-	if maxLength is set to 'len2', both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0' or 'pos1'.

-	and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Clause 7.4.1.1.2 of [4, TS 38.211].

<Unchanged text is omitted>

When receiving PDSCH scheduled by DCI format 1_0, 4_0, 4_1, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

<Unchanged text is omitted>

----------------- End of TP ----------------

Agreement

For PDSCH scheduled by a DCI format 4_1/4_2, the UE assumes phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB configured in PDSCH-Config-Multicast.

  - Adopt the following TP for Clause 5.1.6.3 in TS 38.214:

----------------- Start of TP ----------------

5.1.6.3	PT-RS reception procedure

<Unchanged text is omitted>

The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 4_1 or DCI format 4_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB in PDSCH-Config-Multicast.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

5.1	UE procedure for receiving the physical downlink shared channel

<Unchanged text is omitted>

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 4_0, 4_1, 4_2 or 1_2 decode the corresponding PDSCHs as indicated by that DCI. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH. The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6]. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration m, with N=13 for m=0, N=13 for m=1, N=20 for m=2, and N=24 for m=3.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1.3.2 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

5.1.3.2	Transport block size determination

<Unchanged text is omitted>

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1, format 4_0, format 4_1, format 4_2 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or SI-RNTI, if Table 5.1.3.1-2 is used and , or a table other than Table 5.1.3.1-2 is used and , the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 7.3.1.6 in TS 38.211v17.0.0 is endorsed.

----------------- Start of TP ----------------

7.3.1.6	Mapping from virtual to physical resource blocks

<Unchanged text is omitted>

-	for PDSCH transmissions scheduled with DCI format 1_0 in any common search space in bandwidth part i with starting position NBWP,istart, other than Type0-PDCCH common search space in CORESET 0 and common search space associated with G-RNTI or G-CS-RNTI, the set of NBWP,initsize virtual resource blocks 0,1,…,NBWP,initsize-1, where NBWP,initsize is the size of CORESET 0 if CORESET 0 is configured for the cell and the size of initial downlink bandwidth part if CORESET 0 is not configured for the cell, are divided into Nbundle virtual resource-block bundles in increasing order of the virtual resource-block number and virtual bundle number and the set of NBWP,initsize physical resource blocks NstartCORESET, NstartCORESET+1,…,NstartCORESET+NBWP,initsize-1 are divided into Nbundle physical resource-block bundles in increasing order of the physical resource-block number and physical bundle number, where Nbundle=NBWP,initsize+NBWP,istart+NstartCORESET mod LL, L=2 is the bundle size, and NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1.3.1 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

<Unchanged text is omitted>

elseif the higher layer parameter mcs-Table given by PDSCH-Config-Multicast is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by PDSCH-Config-Multicast is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

<Unchanged text is omitted>

----------------- End of TP ----------------

Agreement

For RRC_CONNECTED UEs receiving broadcast MCCH/MTCH, the Type0B-PDCCH CSS set configured by searchSpace-Broadcast in pdcch-Config-MCCH/pdcch-Config-MTCH follows the same prioritization rule for search space set overbooking procedure as CSS set(s) configured by searchSpace-Multicast.

Agreement

Regarding the number of DCIs that a UE can process in a slot or span, multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

Agreement

For multicast RRC_CONNECTED UEs, rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RB symbol level granularity,

  - The procedure for PDSCH scheduled by PDCCH with DCI format 4_1 is similar as that of DCI format 1_0 and the procedure for PDSCH scheduled by PDCCH with DCI format 4_2 is similar as that of DCI format 1_1, by applying the parameters of rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config-Multicast.

  - rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config for unicast do not apply for GC-PDSCHs.

  - rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs.

Agreement

PDSCH processing capability 2 is not applied to PDSCH scheduled by PDCCH with DCI format 4_0/4_1/4_2.

Agreement

Regarding the size of DCI format 4_2 for multicast of RRC_CONNECTED UE,

  - the size is configured per CFR for all G-RNTIs (included in cfr-Config-Multicast).

  - the value range of the size is {[1]..140} (the same as for DCI format 2_6)

- Agreement

- From RAN1 perspective, it is feasible for UE in RRC_CONNECTED state to receive MBS broadcast on an activated SCell as long as UE has capability of supporting MBS broadcast on SCell. From RAN1 perspective, if a UE is to receive MBS broadcast on SCell,

  - The capability of supporting MBS broadcast on SCell is separate capability from the one of CA for unicast.

  - The UE is not required to monitor DCI formats associated with SI-RNTI, P-RNTI, RA-RNTI in SCell.

  - Overbooking for SCell is not supported.

  - MBS broadcast reception on SCell can be supported only for RRC_CONNECTED UEs only with self-scheduling.

  - Type0-PDCCH CSS set is only configured on the primary cell of the MCG.

  - Configuring the search space on SCell for PDCCH monitoring of MBS DCI formats is via unicast RRC signaling.

  - The UE capability is expected to be defined by RAN2.

    - E.g. the total number of component carriers for receiving broadcast on SCell may be subject to UE capability

  - The UE is not required to receive broadcast on PCell and SCell simultaneously

- Agreement

- From RAN1 perspective, it is feasible for UE in RRC_CONNECTED state to receive MBS broadcast on non-serving cell, which is up to UE implementation and transparent to the network.

  - It is assumed in RAN1 that UE receiving MBS broadcast on non-serving cell does not have any impact to operation on serving cell(s), e.g., does not require UE to obtain the related configuration from the serving cell, does not require the network to guarantee the scheduling doesn’t exceed UE’s capability on serving cell, etc.

  - RAN1 assumes that receiving MBS broadcast on non-serving cell could be on the same or on a different band, but on a different carrier frequency than a UE’s serving cell

  - No RAN1 spec impact and no optimization is pursued in Rel-17 for MBS broadcast reception on non-serving cell.

  - The UE capability(ies), if any, is(are) expected to be defined by RAN2.

Agreement

Regarding the size of DCI format 4_2 for multicast of RRC_CONNECTED UE,

  - the value range of the size is {20..140}

The TP below for Clause 5.1.4.1 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

5.1.4.1	PDSCH resource mapping with RB symbol level granularity

<Unchanged text is omitted>

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2 instead of rateMatchPatternGroup1 and rateMatchPatternGroup2.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1, and the procedures for PDSCH scheduled by DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2 by applying the parameters of rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config-Multicast.

<Unchanged text is omitted>

----------------- End of TP ----------------

The TP below for Clause 5.1.4.2 in TS 38.214v17.0.0 is endorsed.

----------------- Start of TP ----------------

5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList. The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config-Multicast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

<Unchanged text is omitted>

----------------- End of TP ----------------