3GPP TSG RAN WG1 Meeting #108-e	R1-2201941

E-meeting, Feb. 21 – Mar. 03, 2022

Agenda Item:	8.2.2.2

Source:	Xiaomi

Title:	Maintenance on multi-TRP Inter-cell Operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss some remaining issues with respective to mTRP inter-cell operation.

Multi-TRP inter-cell operation

Rate matching

Agreement about additional rate matching behavior is shown as following [1]:

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

In Rel-15/16, it is specified that the PDSCH from serving cell should be rate matched around serving cell SSB in TS 38.214 Clause 5.1.4. And this additional rate matching behavior, namely UE shall assume that the PRBs containing SS/PBCH block transmission resources configured in CSI-ResourceConfig with physical cell ID different from that of serving cell are not available for PDSCH which is indicated with TCI state associated with non-serving cell PCI, should be clarified correspondingly.

Proposal 1: Adopt the following TP to TS 38.214 Clause 5.1.4

============================ Unchanged part omitted =========================== 5.1.4 PDSCH resource mapping When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. When UE is configured with [NumberOfAdditionalPCI], the UE shall assume that the PRBs containing SS/PBCH block transmission resources configured in CSI-ResourceConfig with physical cell ID different from that of serving cell are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. ============================ Unchanged part omitted ===========================

PCI association with CORESETPoolIndex

This is an agreement with respective to the association between PCI and CORESETPoolIndex [2]:

Agreement

- For inter-cell mTRP , one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex

- FFS: The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

In 107-e meeting, some related CRs was endorsed in the draft CR [3], one of them is about the association between PCI and CORESETPoolIndex in the procedure for UE to receive the physical downlink shared channel.  However, we do not think this is necessary because RAN1 was already assumed that there is only one physical layer configuration and that is applied to all the PDCCH associated with TCI state that is associated with either serving cell PCI or another different PCI [4], which means there is no difference for UE to receive PDSCH from serving cell or non-serving except for the transmission beam. For the perspective of UE, it just needs to know that there will be multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain both, whether inter-cell mTRP or intra-cell mTRP is configured. Therefore, the corresponding CR should be removed.

Proposal 2: The following TP related to TS38.214 clause 5.1 is provided.

============================ Unchanged part omitted =========================== 5.1 UE procedure for receiving the physical downlink shared channel … If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of full/partially-overlapped PDSCHs in time, only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex. For a ControlResourceSet without coresetPoolIndex, the UE may assume that the ControlResourceSet is assigned with coresetPoolIndex as 0. When the UE is configured with [NumberOfAdditionalPCI], ControlResourceSets corresponding to different coresetPoolIndex values may be associated with different physical cell IDs via activated TCI states of the ControlResourceSets, where ControlResourceSets corresponding to one coresetPoolIndex can be associated with one physical cell ID and ControlResourceSets corresponding to another coresetPoolIndex can be associated with another physical cell ID. When the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, the following operations are allowed: ============================ Unchanged part omitted ===========================

In RAN1 107-e meeting, switching between inter-cell mTRP and intra-cell mTRP is discussed and most companies mentioned that the switching is already supported by MAC CE. Then, the active TCI states corresponding to different CORESETPoolIndex can be associated with either different physical cell ID or same physical cell ID.

Proposal 3: Adopt the following TP to TS 38.214 clause 5.1.5.

5.1.5 Antenna ports quasi co-location … If the UE is configured with [NumberOfAdditionalPCI] and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a CORESETPoolIndex, the activated TCI states corresponding to one CORESETPoolIndex can be associated with one physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another or the same physical cell ID. ============================ Unchanged part omitted ===========================

UL-related enhancement

Some UL-related enhancements were discussed before and we did not reach any agreement because some companies believed that this is out-of-scope and it should be addressed in Agenda 8.1.1. Actually, for inter-cell beam management in Agenda 8.1.1, Rel-17 Unified TCI framework is supported while inter-cell mTRP operation is based on Rel-15/16 TCI framework. In addition, the WID, as shown below, limits the scope of our discussion to QCL/TCI-related enhancements, not just QCL/TCI enhancements. Accordingly, UL-related enhancements to support inter-cell mTRP operation should be discussed in this agenda if necessary.

Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

Firstly, we need to decide whether to support UL transmission between UE and TRP associated with non-serving cell PCI. According to the WID, the discussion of QCL/TCI-related enhancements to enable inter-cell multi-TRP operation should be based on multi-DCI based multi-PDSCH reception, in which case non-ideal backhaul is assumed. Then, for inter-cell mTRP, UL transmission between UE and TRP associated with non-serving cell PCI should be supported to reuse the separate HARQ-ACK feedback mechanism.

Proposal 4: Support UL transmission between UE and TRP associated with non-serving cell PCI.

Secondly, if UL transmission between UE and TRP associated with non-serving cell PCI is supported, the spatial relation should be enhanced to indicated UE the UL beam of non-serving cell. For example, to do so, non-serving cell SSB is allowed to be used as QCL source RS for UL signal/channel between UE and non-serving cell.

Proposal 5: Enhancements related to spatial relation are needed to support UL transmission between UE and TRP associated with non-serving cell PCI.

Conclusion

According to the discussion above, we have the following proposals:

Proposal 1: Adopt the following TP to TS 38.214 Clause 5.1.4

Proposal 2: The following TP related to TS38.214 clause 5.1 is provided.

Proposal 3: Adopt the following TP to TS 38.214 clause 5.1.5.

Proposal 4: Support UL transmission between UE and TRP associated with non-serving cell PCI.

Proposal 5: Enhancements related to spatial relation are needed to support UL transmission between UE and TRP associated with non-serving cell PCI.

References

- RAN1 104-e meeting Chairman notes.

- RAN1 106-bis-e meeting Chairman notes.

- R1-2112483, “Introduction of further enhancements on MIMO for NR”, 3GPP TSG RAN1 Meeting#107-e, Nov. 2021, Nokia

- R1-2110631, “LS Reply on inter-cell beam management and multi-TRP in Rel-17”, 3GPP TSG RAN1 Meeting#106-bis-e, Oct. 2021