3GPP TSG RAN WG1 #108-e                                       	       R1-2201947

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.5.4

Source:	Xiaomi

Title:                     Remaining issues on latency improvements for both DL and DL+UL positioning method

Document for:	Discussion and Decision

Introduction

In RAN1- 107 e-meeting [1], some agreements on latency improvement can be seen as below:

Agreement

At least the following parameters for PRS processing window from the gNB to the UE are supported.

  - Starting slot

  - Periodicity

  - Duration/length

  - Cell and SCS information associated with the above parameters

Discuss during the maintenance phase on the necessity of other parameters including but not limited to

  - Processing type (associated with the corresponding UE capability 1A/1B/2)

  - Band/CC-ID as needed depending on each scenario on which the PRS processing window is applied

  - The above cell and SCS information to determine where/when the PRS processing window is applied

Note: Indication of processing type does not suggest UE indication of multiple capabilities among (1A/1B/2) is already supported, which is a separate discussion.

Note: Some of the parameters above may not be mandatory for a PRS processing window

Agreement

For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority.

For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority.

Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

Agreement

The following options are supported subject to UE capability for priority handling of PRS when PRS measurement is outside MG.

  - Option 1: UE may indicates support of two priority states.

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

    - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS

  - Option 2: UE may indicate support of three priority states

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

    - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS

      - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority.

    - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS

  - Option 3: UE may indicate support of single priority state

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

Note: SSB is a separate issue.

In this contribution, we focus on remaining issues on latency improvements for both DL and DL+UL positioning method.

Measurement gap-less PRS measurement

According to the agreement in RAN1-107 e-meeting, for UE determining DL PRS to be higher priority inside the PRS processing window, different UE capabilities are supported, including Cap.1A, Cap.1B and Cap.2. From our point of view, three capabilities means three groups of UE with different capability. That means each UE support only one capability. Thus we prefer UE support one of capabilities. So ‘processing type’ is not needed for PRS processing window.

While for ‘band/CC-ID’, it is needed for Cap.1B at least since “only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority.”  In addition, for Cap. 2, similar to Cap.1A, it should be applied to all DL CCs.

- Proposal 1: on parameters for PRS processing window from the gNB to the UE

- Not support ‘processing type’ since reporting only one UE capability is preferred.

- Support ‘band ID’ for Cap. 1B only.

While for handling for PRS priority, only priority between PRS and PDSCH/PDCCH/CSI-RS are agreed. But refer to SSB, we prefer to lower the PRS priority than SSB reception since it is necessary for time and frequency synchronization by SSB before PRS reception.

- Proposal 2: Prefer higher priority for SSB than PRS.

Conclusion

In this contribution, we discussed about remaining issues on latency improvements for both DL and DL+UL positioning method. Based on above discussion, we provide the following proposals.

- Proposal 1: on parameters for PRS processing window from the gNB to the UE

- Not support ‘processing type’ since reporting only one UE capability is preferred.

- Support ‘band ID’ for Cap. 1B only.

- Proposal 2: Prefer higher priority for SSB than PRS.

References

- RAN1-107 e-meeting chairman notes.