3GPP TSG RAN WG1 Meeting #108-e				R1-2201960

e-Meeting, February 21st – March 3rd, 2022

Source 	: CAICT

Title 	: Remaining issues on enhancements on HARQ to support NTN

Agenda Item	: 8.4.3

Document for	: Discussion / Decision

Introduction

For HARQ enhancements in Rel.17 to support NTN, there are still some remaining issues to be handled. In this contribution, we will further discuss these aspects.

Discussion

Type-2 HARQ codebook enhancements

For Type-2 HARQ-ACK codebook in NTN, the following agreements were reached in the previous meeting:

Agreement

The bit-fields related to the HARQ-ACK feedback (i.e., PRI, PUSCH-to-HARQ_feedback timing, DAI) are unchanged for the DCI of PDSCH with feedback-disabled HARQ process in Rel-17 with the same interpretation from UE as for feedback-enabled HARQ process

Note: The interpretation regarding the DAI for Type-2 codebook is up to the progress of codebook design.

Furthermore, the way of counting C-DAI and T-DAI has not been determined. Two options were discussed.

For the DCI of PDSCH with feedback-disabled HARQ processes, one of following Options is supported:

- Option-1: the C-DAI and T-DAI are the same of the C-DAI and T-DAI of the most recent DCI of PDSCH with feedback-enabled processes, despite they are not incremented.

  - For the codebook generation, the UE should use the DAI in DCI of feedback-disabled HARQ process to detect if a previous DCI of feedback-enabled HARQ processes has been missed. If so, the UE should chose the HARQ codebook size based on the DAI in DCI of the feedback-disabled HARQ process, and the feedback-enabled HARQ process detected to be missed should be NACK.

  - if all DCIs of PDSCH are associated with feedback disabled HARQ process,

    - Alt-1: The value of C-DAI and T-DAI set to VC-DAIDL=4 and VT-DAIDL=4

- Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

The stated benefit of option 1 is the DAI in feedback-disabled HARQ process DCI could be used to detect if a previous DCI of feedback-enabled HARQ processes is missed. To our opinion, the robustness of HARQ-ACK codebook size determination with the C-DAI and T-DAI in DCI for feedback-enabled processes is the same as in a territorial network and no further performance improvements are needed. In addition, the C-DAI and T-DAI in the DCI could be set to 1,2,3,4 currently. For the PDSCHs corresponding one PUCCH slot, if the first PUSCH is with HARQ-ACK feedback-disabled HARQ processes, the value “0” may be needed for the C-DAI and T-DAI of this scheduling DCI. Furthermore, if all DCIs of PDSCH are associated with feedback disabled HARQ process, Type-2 HARQ-ACK codebook could be not reported at all. With option 1, the value of C-DAI and T-DAI set to VC-DAIDL=4 and VT-DAIDL=4 is used as a special state of DCI to identify this case. This means VC-DAIDL=4 and VT-DAIDL=4 could not be used when there is at least one feedback-enabled HARQ processes within the scheduling window. This would introduce scheduling restriction. Therefore, it is observed much specification impacts are needed while the possible benefits would be overdesigned with option1. With option 2, marginal specification impacts are expected.

Proposal 1: For the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

SPS PDSCH

For Type-2 HARQ-ACK codebook in NTN, the following agreements were reached in the previous meeting:

Agreement

HARQ feedback for SPS activation may be additionally enabled by the network by RRC configuration.

- If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of whether HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation

- Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation,

  - FFS between Alt1 and Alt2

    - [Alt-1: UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process

    - Alt-2: UE follows the feedback-enabled/disabled configuration of the SPS PDSCH]

For DL SPS, there is no bit field for HARQ process ID indication in DCI and the HARQ process ID is calculated by the location of transmission resource in time domain. If more than one HARQ process is configured for one SPS, different HARQ process ID would be used within this SPS according to the PDSCH transmission occasion. If the HARQ-feedback configuration for these HARQ IDs is different, the data performance would vary within one service. Besides, HARQ process ID could be reused between dynamically scheduled PDSCH and SPS PDSCH which are possible for different services. If we restrict enable/disable configuration for one HARQ process ID is the same no matter it is used for dynamic PDSCH or SPS PDSCH, the flexibility would also be restricted. This would be much undesirable. Therefore, it is proposed disabling/enabling of HARQ process is per SPS PDSCH configuration. The disabling/enabling configuration of SPS PDSCH is independent with the HARQ process disabling/enabling configuration for dynamic PDSCH.

Proposal 2: Disabling/enabling of HARQ process is per SPS PDSCH configuration. The disabling/enabling configuration of SPS PDSCH is independent with the HARQ process disabling/enabling configuration for dynamic PDSCH.

Type-1 HARQ-ACK codebook

For Type-1 HARQ-ACK codebook in NTN, the following agreements were reached in the previous meetings:

Agreement

For Type-1 HARQ codebook, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

There is no misalignment between gNB and UE about the feedback-disabled HARQ process transmission since the corresponding scheduling DCI is transmitted by gNB and received by the UE. Then the corresponding NACK is pre-known by gNB and UE without any ambiguous. This provides performance improvements for HARQ-ACK codebook transmission which was stated before.

To our understanding, it is overdesigned with this agreement since the reliability of HARQ-ACK for NTN is better than TN. Considering the UE power efficiency, possible PUCCH transmit power reduction with the improved decoding performance could be considered. The ∆TF,b,f,ci in the PUCCH transmit power determination reflects the impacts of UCI bits in [2] as below.

……

-	∆TF,b,f,ci is a PUCCH transmission power adjustment component on active UL BWP b of carrier f of primary cell c

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1, ∆TF,b,f,ci=10log10NrefPUCCHNsymbPUCCH(i)+∆UCIi where

-	NsymbPUCCH(i) is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in clause 9.2.

-	NrefPUCCH=2 for PUCCH format 0

-	NrefPUCCH=Nsymbslot for PUCCH format 1

-	∆UCIi=0 for PUCCH format 0

-	∆UCIi=10log10OUCIi for PUCCH format 1, where OUCIi is a number of UCI bits in PUCCH transmission occasion i

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, ∆TF,b,f,ci=10log10K1⋅nHARQ-ACKi+OSRi+OCSIiNREi, where

-	K1=6

-	nHARQ-ACKi is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook. nHARQ-ACKiis the same as OACKi as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, nHARQ-ACKi=1 if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, nHARQ-ACKi=0

-	OSRi is a number of SR information bits that the UE determines as described in clause 9.2.5.1

-	OCSIi is a number of CSI information bits that the UE determines as described in clause 9.2.5.2

-	NREi is a number of resource elements determined as NRE(i)=MRB,b,f,cPUCCH(i)⋅Nsc,ctrlRB(i)⋅Nsymb-UCI,b,f,cPUCCH(i), where Nsc,ctrlRB(i) is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and ⋅Nsymb-UCI,b,f,cPUCCH(i) is a number of symbols excluding symbols used for DM-RS transmission, as defined in clause 9.2.5.2, for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, ∆TF,b,f,ci=10log102BPRE⋅K2-1, where

-	K2=2.4

-	BPRE(i)=OACKi+OSRi+OCSIi+OCRCiNRE(i)

-	OACKi is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook, or as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, OACK=1 if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, OACK=0

-	OSRi is a number of SR information bits that the UE determines as described in clause 9.2.5.1

-	OCSIi is a number of CSI information bits that the UE determines as described in clause 9.2.5.2

-	OCRCi is a number of CRC bits that the UE determines as described in clause 9.2

-	NREi is a number of resource elements that the UE determines as NRE(i)=MRB,b,f,cPUCCH(i)⋅Nsc,ctrlRB(i)⋅Nsymb-UCI,b,f,cPUCCH(i), where Nsc,ctrlRB(i) is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and Nsymb-UCI,b,f,cPUCCH(i) is a number of symbols excluding symbols used for DM-RS transmission, as defined in clause 9.2.5.2, for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c.

It is proposed the pre-known NACK is excluded from HARQ-ACK bits when determining ∆TF,b,f,ci when DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE. As s result, the benefits from pre-known NACK could be enjoyed while the UE power consumption efficiency could be improved.

Proposal 3: For Type-1 HARQ-ACK codebook, when DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, the pre-known NACK is excluded from HARQ-ACK bits when determining ∆TF,b,f,ci for PUCCH transmit power.

Conclusion

In this contribution, we discussed remaining issues about enhancements on HARQ in NTN. The following proposals are reached:

Proposal 1: For the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

Proposal 2: Disabling/enabling of HARQ process is per SPS PDSCH configuration. The disabling/enabling configuration of SPS PDSCH is independent with the HARQ process disabling/enabling configuration for dynamic PDSCH.

Proposal 3: For Type-1 HARQ-ACK codebook, when DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, the pre-known NACK is excluded from HARQ-ACK bits when determining ∆TF,b,f,ci for PUCCH transmit power.

- Reference

- R1-2112664 Summary#3 of AI 8.4.3 for HARQ in NTN

- 3GPP TS 38.213 V17.0.0, “Physical layer procedures for control”