3GPP TSG RAN WG1 #108-e		R1-2201978

e-Meeting, February 21st – March 3rd, 2022

3GPP TSG-RAN WG4 Meeting # 101-e 					 R4-2120652

# Electronic Meeting, November 01-12, 2021

Title:	Draft reply LS on UE capability for supporting single DCI transmission schemes for multi-TRP

Release:	Rel-16

Work Item:	NR_eMIMO-Perf

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

- 1. Overall Description

In Rel-16 RAN4 has defined performance requirements for multi-DCI and single DCI transmission schemes for multi-TRxP transmission in FR1. These requirements are applicable to UEs supporting the following UE capability.

| Test Case/ Transmission scheme | UE Capability |
| --- | --- |
| Multi-DCI non-overlapping | multiDCI-MultiTRP-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI SDM | singleDCI-SDM-scheme-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI Inter-Slot TDM | supportInter-slotTDM-r16, maxNumberTCI-states-r16=2, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI FDM Scheme A | supportFDM-SchemeA-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |

The requirements are defined with 2 active TCI states. The multi-DCI transmission scheme requirements are based on UE capability multiDCI-MultiTRP-r16 also includes maxNumberCORESETPerPoolIndex-r16, which is sufficient to indicate support of 2 active TCI states. For single DCI Inter-Slot TDM the capability of maxNumberTCI-states-r16=2 in supportInter-slotTDM-r16 covers support for 2 active TCI states. For single DCI FDM and SDM transmission schemes, the UE capability doesn’t include any additional UE capability for support of more than 1 active TCI state or any pre-requisite to support maxNumberActiveTCI-PerBWP other than n1.

Question: RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

Answer: Based on RAN4’s question, RAN1 would like to provide replies on RAN4 that either an explicit signaling of maxNumberActiveTCI-PerBWP other than n1 or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 with an appropriate conclusion, e.g., if a UE reporting supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 may also report maxNumberActiveTCI-PerBWP other than n1 although there is no explicit signaling.

- 2. Actions:

To: RAN4

ACTION: RAN1 respectfully asks RAN4 to take the above information into account in their further work.

- 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 	16 – 27, May, 2022		e-Meeting

TSG-RAN WG1 Meeting #110 	22 – 26, August, 2022		Toulouse, France