3GPP TSG RAN WG1 Meeting #108-e	R1-2201982

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft Reply LS to RAN4 on PEMAX for NR-V2X

Response to:	LS (R4-2120047)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1 (Samsung)

To:	RAN4

Cc:

Contact Person:

Name:

E-mail Address:

Attachments:

1. Overall Description:

RAN1 would like to provide replies on the LS from RAN4 in R4-2120047 for PEMAX for NR-V2X.

RAN4 asked feedback on the following issue #1:

Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

RAN1 reply to Issue 1: It is correct understanding that the parameter is used for both in-coverage and out-of-coverage. In addition, sl-MaxTransPower is for PEMAX,c.

In addition, RAN4 asked feedback on the following issue #2:

Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed: - Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu - Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell. - Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used. RAN4 would like to check which option is aligned with the RAN1 and RAN2 specification

RAN1 reply to Issue 2: All options are compatible with current RAN1 specifications.

2. Actions:

To RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account in their further work

3. Date of Next Meetings:

TSG RAN1 Meeting #109-e	16th – 27th May 2022				e-Meeting

TSG RAN1 Meeting #110		22th – 26th August 2022				Toulouse, FR