3GPP TSG WG1 Meeting #108-e	R1-2201989

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev |  | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UCI multiplexing in PUSCH with repetitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The specifications do not correctly implement the Rel-15 RAN1 agreement for determining the number of UCI coded modulation symbols in a PUSCH with repetitions as the formula uses a number of symbols for one repetition of the PUSCH transmission instead of the number of symbols for the PUSCH transmission. This error was corrected in Rel-17 for “TBoMS” PUSCH (Rel-15 RAN1 agreement is implemented) and should be corrected for type A and type B repetitions to avoid UCI BLERs as large as 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Scale by the number of slots Ns the numerator in the formula for determining UCI coded modulation symbols in a PUSCH transmission with repetitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification of RAN1 agreements. Unreliable UCI when multiplexed in a PUSCH transmission with repetitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.1.1, 6.3.2.4.1.2, 6.3.2.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR has isolated impact. If a UE implements the CR and the gNB does not, or if the gNB implements the CR and the UE does not, there will be different understanding between the UE and the gNB about the REs used for UCI and for data multiplexing in a repetition of a PUSCH transmission. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged text is omitted >>

###### 6.3.2.4.1.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows:

QACK'=minOACK+LACK∙βoffsetPUSCH∙Ns,A∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,   α∙l=0Nsymb,allPUSCH-1MscUCIl

where

-	 is the number of HARQ-ACK bits;

-	if , ; otherwise  is the number of CRC bits for HARQ-ACK determined according to Clause 6.3.1.2.1;

-	;

-	Ns,A=1 if the PUSCH transmission is without repetitions; otherwise, Ns,A is the number of repetitions if the PUSCH transmission is with type A repetitions;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise,  is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling;

-	 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

For HARQ-ACK transmission on an actual repetition of a PUSCH with repetition Type B with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as QACK', is determined as follows:

QACK'=minOACK+LACK∙βoffsetPUSCH∙Ns,B∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr,   α∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl

where

-	Msc,nominalUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ⋯, Nsymb,nominalPUSCH-1, in the PUSCH transmission assuming a nominal repetition without segmentation, and Nsymb,nominalPUSCH is the total number of OFDM symbols in a nominal repetition of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=MscPUSCH-Msc,nominalPT-RSl where Msc,nominalPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission assuming a nominal repetition without segmentation;

-	Msc,actualUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l , for l=0, 1, 2, ⋯, Nsymb,actualPUSCH-1, in the actual repetition of the PUSCH transmission, and Nsymb,actualPUSCH is the total number of OFDM symbols in the actual repetition of the PUSCH transmission, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=MscPUSCH-Msc,actualPT-RSl where Msc,actualPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the actual repetition of the PUSCH transmission;

-	Ns,B is the number of nominal repetitions;

-	and all the other notations in the formula are defined the same as for PUSCH not using repetition type B.

<< Unchanged text is omitted >>

###### 6.3.2.4.1.2	CSI part 1

For CSI part 1 transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows:

QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙Ns,A∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'

where

-	 is the number of bits for CSI part 1;

-	if , ; otherwise  is the number of CRC bits for CSI part 1 determined according to Clause 6.3.1.2.1;

-	;

-	Ns,A=1 if the PUSCH transmission is without repetitions; otherwise, Ns,A is the number of repetitions if the PUSCH transmission is with type A repetitions;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	QACKCG-UCI'=QACK' if HARQ-ACK is present for transmission on the same PUSCH with UL-SCH and without CG-UCI, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH as defined in clause 6.3.2.4.1.1 if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is no more than 2 bits, where  is the number of reserved resource elements for potential HARQ-ACK transmission in OFDM symbol , for , in the PUSCH transmission, defined in Clause 6.2.7; or

-	QACKCG-UCI'=QACK' if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmitted on the PUSCH as defined in clause 6.3.2.4.1.5; or

-	QACKCG-UCI'=QCG-UCI' if CG-UCI is present on the same PUSCH with UL-SCH and without HARQ-ACK, where QCG-UCI' is the number of coded modulation symbols per layer for CG-UCI transmitted on the PUSCH as defined in clause 6.3.2.4.1.4;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling.

For CSI part 1 transmission on an actual repetition of a PUSCH with repetition Type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as QCSI-part1', is determined as follows:

QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙Ns,B∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr,   α∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACKCG-UCI' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACKCG-UCI'

where

-	Msc,nominalUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ⋯, Nsymb,nominalPUSCH-1, in the PUSCH transmission assuming a nominal repetition without segmentation, and Nsymb,nominalPUSCH is the total number of OFDM symbols in a nominal repetition of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=MscPUSCH-Msc,nominalPT-RSl where Msc,nominalPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission assuming a nominal repetition without segmentation;

-	Msc,actualUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ⋯, Nsymb,actualPUSCH-1, in the actual repetition of the PUSCH transmission, and Nsymb,actualPUSCH is the total number of OFDM symbols in the actual repetition of the PUSCH transmission, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=MscPUSCH-Msc,actualPT-RSl where Msc,actualPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the actual repetition of the PUSCH transmission;

-	Ns,B is the number of nominal repetitions;

-	and all the other notations in the formula are defined the same as for PUSCH not using repetition type B.

<< Unchanged text is omitted >>

###### 6.3.2.4.1.3	CSI part 2

For CSI part 2 transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined as follows:

QCSI-2'=minOCSI-2+LCSI-2∙βoffsetPUSCH∙Ns,A∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'

where

-	 is the number of bits for CSI part 2;

-	if , ; otherwise  is the number of CRC bits for CSI part 2 determined according to Clause 6.3.1.2.1;

-	;

-	Ns,A=1 if the PUSCH transmission is without repetitions; otherwise, Ns,A is the number of repetitions if the PUSCH transmission is with type A repetitions;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	QACK/CG-UCI'=QACK' if HARQ-ACK is present for transmission on the same PUSCH with UL-SCH and without CG-UCI, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH as defined in clause 6.3.2.4.1.1 if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is 1 or 2 bits; or

-	QACK/CG-UCI'=QACK' if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmitted on the PUSCH as defined in clause 6.3.2.4.1.5; or

-	QACK/CG-UCI'=QCG-UCI' if CG-UCI is present on the same PUSCH with UL-SCH and without HARQ-ACK, where QCG-UCI' is the number of coded modulation symbols per layer for CG-UCI transmitted on the PUSCH as defined in clause 6.3.2.4.1.4;

-	 is the number of coded modulation symbols per layer for CSI part 1 transmitted on the PUSCH;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, .

-	 is configured by higher layer parameter scaling.

For CSI part 2 transmission on an actual repetition of a PUSCH with repetition Type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as QCSI-part2', is determined as follows:

QCSI-2'=minOCSI-2+LCSI-2∙βoffsetPUSCH∙Ns,B∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr,   α∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1'

where

-	Msc,nominalUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ⋯, Nsymb,nominalPUSCH-1, in the PUSCH transmission assuming a nominal repetition without segmentation, and Nsymb,nominalPUSCH is the total number of OFDM symbols in a nominal repetition of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the PUSCH assuming a nominal repetition without segmentation, Msc,nominalUCIl=MscPUSCH-Msc,nominalPT-RSl where Msc,nominalPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission assuming a nominal repetition without segmentation;

-	Msc,actualUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0, 1, 2, ⋯, Nsymb,actualPUSCH-1, in the actual repetition of the PUSCH transmission, and Nsymb,actualPUSCH is the total number of OFDM symbols in the actual repetition of the PUSCH transmission, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the actual repetition of the PUSCH transmission, Msc,actualUCIl=MscPUSCH-Msc,actualPT-RSl where Msc,actualPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the actual repetition of the PUSCH transmission;

-	Ns,B is the number of nominal repetitions;

-	and all the other notations in the formula are defined the same as for PUSCH not using repetition type B.

<< Unchanged text is omitted >>