3GPP TSG RAN WG1 #108-e			R1-2202000

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.2.4

Source: 	Samsung

Title: 	Maintenance on Rel-17 HST-SFN

Document for:	Discussion and Decision

Introduction

In Rel.17 NR FeMIMO WID, multi-TRP enhancements for HST-SFN are given as follows [1]:

Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

This contribution provides Samsung’s view on the highlighted topic above.

SFN transmission for multi-TRP

  - Default spatial relation and PL-RS for PUCCH, PUSCH, and SRS

In RAN1#106-e [3], the following agreement was made about the default spatial relation and PL-RS for PUCCH, PUSCH and SRS considering SFN PDCCH transmission.

Agreement If enhanced SFN PDCCH transmission scheme (scheme 1 or if TRP-based pre-compensation is supported in FR2) is configured, and if the CORESET with the lowest ID in the active DL BWP is indicated with two TCI states - If PL-RS and spatial relation information are not configured for PUCCH and enableDefaultBeamPL-ForPUCCH is configured in FR2 - For single-TRP PUCCH transmission, select the first TCI state of the CORESET as default beam and PL RS - If PUSCH scheduled by DCI format 0_0 and enableDefaultBeamPL-ForPUSCH0-0 is configured in FR2, and if PUCCH resource is not configured on active UL BWP in the cell or if spatial relation is not configured in any PUCCH resource on active UL BWP in the cell, - For single-TRP PUSCH transmission scheduled by DCI format 0_0, select the first TCI state of the CORESET as default beam and PL RS - If PL-RS and spatial relation information are not configured for SRS and enableDefaultBeamPL-ForSRS is configured in FR2 - For single-TRP SRS resource, select the first TCI state of the CORESET as default beam and PL RS - FFS other details, if any - These are UE optional features

Regarding SFN transmission of PDCCH from multi-TRP, the current default spatial relation and pathloss reference RS (PL-RS) for PUCCH, PUSCH, and SRS would be affected. As a result of the possibility of SFN transmission of PDCCH from multi-TRP, the existing method based on current specification to derive a default spatial setting and PL-RS for PUCCH, PUSCH, and SRS cannot directly be applied. This is a direct consequence of the existence of multiple TCI states associated to the SFNed CORESET with the lowest ID. Therefore, it is required to resolve this ambiguity.

However, the current specification considers the only case of a single default spatial relation and PL-RS for PUCCH, PUSCH, and SRS. In Rel-17, the multiple default spatial relations and PL-RSs can be also considered at least for multi-TRP based PUCCH and PUSCH repetition. For PUCCH or PUSCH repetition intended towards multi-TRP, if certain conditions are fulfilled, it is natural to select multiple default spatial relations so that each spatial relation can be mapped to the PUCCH or PUSCH repetitions towards a corresponding TRP similar with two default TCI states in Rel-16 PDSCH TDMed repetition. Otherwise, for example, if a single default PL-RS is selected based on only one TCI state (spatial relation from a certain TRP), then the computed PL may not be accurate to compensate PL in PUCCH or PUSCH repetitions towards the remaining TRPs. Similarly, if a spatial setting is selected based on a single TCI states, then the PUCCH transmissions towards the remaining TRPs may not be correctly beamformed. Therefore, it is essential to consider multiple default beams for multi-TRP based PUCCH or PUSCH repetitions.

Proposal 1. Support multiple default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

  - Default TCI for single-TRP PDSCH reception

When we consider default TCI for single-TRP PDSCH reception, the case when CORESET and PDSCH is overlapped can be also considered, which is captured at Clause 5.1.5 in TS38.214 highlighted as follows.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', - the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

Similar with the highlighted sentence above, we can consider the case when the CORESET, which is activated with one or two TCI states, is overlapped with the scheduled single-TRP PDSCH. This can be extended to the intra-band CA case as well. To make the specification as a complete one, our view is to consider the overlapped CORESET which is activated one or two TCI states and is located in same or different component carrier with the scheduled single-TRP PDSCH.

Proposal 2. Support to consider the case when the CORESET, which is overlapped with the scheduled single-TRP PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

  - Default TCI for Rel-17 SFN PDSCH reception

In RAN1#106-e [3], the following agreement was made to re-use the existing rule onto the default TCI for Rel-17 SFN PDSCH reception.

Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is a UE optional feature

Similar with determining the default TCI for single-TRP PDSCH reception as in the current specification, we also consider the case when a CORESET and the scheduled PDSCH is overlapped, which is captured at Clause 5.1.5 in TS38.214 highlighted as follows. Even in the reception for Rel-17 SFN PDSCH, the case for the CORESET, which is activated one or two TCI states, is overlapped and located in same or different component carrier with the scheduled Rel-17 SFN PDSCH should be also considered.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', … - If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)

Proposal 3. Support to consider the case when the CORESET, which is overlapped with the scheduled Rel-17 SFN PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

  - Remaining issues on default TCI for Rel-17 SFN PDCCH/PDSCH

We would like to deal with some remaining issues on default TCI considering Rel-17 SFN PDCCH/PDSCH.

The following two agreements were made in RAN1#106-e and RAN1#107-e, respectively, which are related to default beam operation for PDSCH when scheduling offset is less than the threshold timeDurationForQCL.

Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is a UE optional feature Agreement The agreement from RAN1#106b-e meeting is updated as follows When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e. Rel-16 MTRP PDSCH is not configured), for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL, - For DCI format 1_1/1_2, support both configurations with and without TCI state field. - [If enableTwoDefaultTCIStates is not configured,] for both cases with and without TCI state field, - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception - FFS : Whether above applies for TRP -based pre-compensation if TRP -based pre-compensation is agreed to be support in FR2 - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH - It is up to editor how to capture the above agreement

It seems that above agreements do not cover the scenario when PDSCH is RRC configured with SFN scheme and enableTwoDefaultTCIStates is not configured. In order to make complete specification, in such scenario, we support the similar default beam behavior with the second agreement above.

Proposal 4. When SFN PDSCH is configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL

- For DCI format 1_1/1_2, support both configurations with and without TCI state field.

- If enableTwoDefaultTCIStates is not configured, for both cases with and without TCI state field,

  - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception

    - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2

  - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Further, there was an agreement discussed in RAN1#107-e as follows:

Agreement When SFN PDSCH and SFN PDCCH are configured by RRC , for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET , UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET , UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN , the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured

We would like to discuss about the above FFS: if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, when time offset is equal to or larger than the threshold timeDurationForQCL, UE supports the configuration which there is no TCI field in the DCI scheduling PDSCH. Then, UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH, which is the same solution as the above possible agreement if there is one active TCI state for the CORESET although PDCCH is SFN configured.

Proposal 5. Support the possible agreement in principle and prefer to discuss additionally the following.

- Removing FFS and add UE behavior “TCI field is always present in DCI format 1_1 and 1_2” if the UE does not support the above.

- Consider the configuration if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, support the configuration which there is no TCI field in the DCI scheduling PDSCH when time offset is equal to or larger than the threshold timeDurationForQCL, and UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH.

Further, there was a proposal#4-7 discussed in RAN1#106b-e as follows:

Proposal #4-7: Support configuration of DCI format 1_0/1_1/1_2 without TCI state field for PDSCH transmission using SFN scheme in FR1 - Reuse default TCI states agreed for FR2, i.e., UE applies TCI state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies both QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, UE applies the one active TCI state of the CORESET when receiving the PDSCH

The intention of this proposal is to enable the configuration which there is no TCI field in the DCI scheduling PDSCH in FR1 as well. It seems that the above proposal #4-7 includes the case when PDSCH is SFN configured and both when PDCCH is SFN configured or not. Hence, in order to make complete specification, we also would like to discuss the case when PDSCH is not SFN configured. In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception. Also, if there is a single TCI state for the CORESET, the UE applies the one TCI state of the CORESET for the PDSCH reception, which is current specification.

Proposal 6. Support the proposal #4-7 in RAN1#106b-e, and prefer to discuss additionally the case when PDSCH is not SFN configured.

- In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception.

  - RLM RS selection considering CORESET activated with two TCI states

Based on the enhancement on Rel-17 HST-SFN, the number of activated TCI states can be extended as 2. The motivation on some enhancements, e.g., defining new default beam behaviors (for PUCCH, PUSCH, SRS, and PDSCH) BFD RS, and QCL prioritization rule, is to clarify whether the existing rule considering CORESET with a single TCI state is extended or not when two TCI states are activated in CORESET.

One more possible enhancement is RLM RS selection rule considering two activated TCI states for CORESET. In current specification, When RadioLinkMonitoringRS is not configured, there is a rule to select RLM RSs among the source RS of TCI state for CORESETs which have a single TCI. The selection priority depends on the periodicity of search space set first (the shortest periodicity has the highest selection priority) and if periodicities are same, then the index of CORESET can be used to determine a selection priority (the highest CORESET ID has the highest selection priority). Hence, if two TCI states are activated for all CORESETs or some CORESETs among all CORESETs, it should be studied whether/how to enhance RLM RS selection rule (e.g., the number of activated TCI states can be considered as an additional standard for selection priority).

Proposal 7. Study whether/how to enhance RLM RS selection rule considering CORESET activated with two TCI states.

Conclusion

In this contribution, the following observations and proposals are given:

Proposal 1. Support multiple default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

Proposal 2. Support to consider the case when the CORESET, which is overlapped with the scheduled single-TRP PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

Proposal 3. Support to consider the case when the CORESET, which is overlapped with the scheduled Rel-17 SFN PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

Proposal 4. When SFN PDSCH is configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL

- For DCI format 1_1/1_2, support both configurations with and without TCI state field.

- If enableTwoDefaultTCIStates is not configured, for both cases with and without TCI state field,

  - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception

    - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2

  - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Proposal 5. Support the possible agreement in principle and prefer to discuss additionally the following.

- Removing FFS and add UE behavior “TCI field is always present in DCI format 1_1 and 1_2” if the UE does not support the above.

- Consider the configuration if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, support the configuration which there is no TCI field in the DCI scheduling PDSCH when time offset is equal to or larger than the threshold timeDurationForQCL, and UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH.

Proposal 6. Support the proposal #4-7 in RAN1#106b-e, and prefer to discuss additionally the case when PDSCH is not SFN configured.

- In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception.

Proposal 7. Study whether/how to enhance RLM RS selection rule considering CORESET activated with two TCI states.

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- Chairman’s notes, RAN1#105-e

- Chairman’s notes, RAN1#106-e