3GPP TSG RAN WG1 Meeting #108-e                                                                     R1-2202001

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.3

Source: 	Samsung

Title: 	Maintenance on Rel-17 SRS

Document for:	Discussion and Decision

- Introduction

In RAN#86, the Rel-17 WID of further enhancements on MIMO for NR is approved [1]. In the approved WID, a particular point is about SRS enhancements in terms of flexibility, coverage and capacity, targeting both FR1 and FR2. The detailed scope of the SRS enhancement is given as follows.

3. Enhancement on SRS, targeting both FR1 and FR2:

  - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

  - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

  - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

Previous RAN1 agreements on these SRS enhancements are given in Section 6.1.

In this contribution, we summarize companies’ views on the issues with wide interest after the two rounds of discussion in RAN1#106e.

- Flexibility enhancements

  - SRS triggering offset

The followings are agreements for flexible SRS triggering offset.

Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets (FFS: across all CCs or across a CC/BWP) - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset

Collision handling

To trigger aperiodic SRS more flexibly, RAN1 agreed the flexible SRS triggering offset based on the available slot ‘t’. After triggering the aperiodic SRS, gNB can determine that the triggered aperiodic SRS is outdated but the other SRS transmission is required (e.g., SRS resource set for different usage with the already triggered SRS resource set). In this case, we can consider two options: one option is to define new dropping rule and the other is to rely on implementation (gNB’s scheduler). So far, we found no critical issues for collision handling for flexible aperiodic triggering.

Proposal 1: No support new dropping rule in Rel-17.

  - Flexible DCI format

Re-purpose

Based on the agreement of using DCI 0_1/0_2 to trigger SRS without data and without CSI request, the following schemes to repurpose unused fields in these DCI fields to indicate SRS parameters dynamically. These repurpose filed can be used to have more flexible configuration of aperiodic SRS transmission. However, since the original intention of adopting flexible triggering is to distribute the triggering DCI so that we failed to see additional benefits over using flexible ‘t’ offset.

| Categories | Detailed alternatives |
| --- | --- |
| CAT-A (Time-domain parameters) | A-1: Indication of available slot position, i.e., the t values |
|  | A-2: Indication of slot offset |
|  | A-3: Indication of SRS symbol-level offset |
|  | A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting |
| CAT B (Frequency-domain parameters) | B-1: Indication of a group of CCs for SRS transmission |
|  | B-2: Indication of frequency domain resource in a BWP for SRS transmission |
|  | B-3: Indication of whether DL/UL BWP is applied for SRS transmission |
|  | Do not support this category |
| CAT C (Power control parameters) | C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ |
|  | C-2: Indication of open loop power control parameter e.g., p0. |
|  | Do not support this category |
| CAT D (Spatial-domain parameters, i.e., indication of SRS port and beamforming) | Re-purpose CSI-RS/TPMI indication to indicate SRS spatial-domain parameters |
|  | Do not support this category |
| CAT E (Extend the number of DCI codepoints for aperiodic SRS trigger states) | Extend the number of DCI codepoints for aperiodic SRS trigger states |
| New functionalities | Re-purpose to indicate set usage |

Proposal 2: No repurpose of DCI field(s) is supported for SRS parameter indication in Rel-17.

Group-common DCI

The group common DCI has potential benefit of less DCI size budget compared with UE specific DCI when AP-SRS are triggered for multiple UEs. DCI format 2_3 can be reused to flexibly trigger AP-SRS resource sets without PUSCH for other purpose than carrier switching with introduction of a new RNTI. Each UE can extract its own information block from the CRC scrambled DCI format 2-3 with this new RNTI. The starting position of each block needs to be configured by a higher layer command configuration specifically designed and configured for flexible AP-SRS triggering. Each UE information block can contain SRS request codepoint and slot offset values. Since more than one aperiodic SRS can be triggered with a single SRS request codepoint, indication of different slot offsets in one DCI can be done either through a bitmap structure or through individual indication with a specific assigned number of bits for each triggered set. With bitmap structure, the transmission order of triggered AP-SRS sets can be according to a pre-determined order or SRS set ID. With individual indication, the corresponding slot offsets bits of different SRS sets are sorted according to a pre-determined order or SRS set ID.

Proposal 3: Group-common DCI can be used for triggering of AP-SRS with UE specific usage

- With new RNTI where indication of all triggered AP-SRS sets’ slot offsets in one DCI

- Bitmap structure or through individual indication with a specific assigned number of bits for each triggered set

  - Flexible antenna switching

Flexible antenna switching:

Due to channel variation and the cost of RRC reconfiguration, gNB always prefers to configure the highest possible antenna switching configuration and the intention of introducing combined capability goes amiss. To address this issue, gNB may either configure multiple resource sets following Rel. 15/16 or for periodic/semi-persistent SRS antenna switching scenario specifically, one SRS set can be configured with multiple SRS resources within that set with different or same number of SRS ports. UE selects the correct resource set based on a dynamic indication of specific configuration through MAC CE. This solution, however, may not be efficient in terms of resource overhead. An alternative is to introduce a new optional RRC parameter conditionally only for antenna switching usage that would allow RRC configured downgraded version of SRS resource set. With this new additional RRC parameter, the gNB can then use MAC CE to dynamically select/activate a downgraded configuration.

Proposal 4: Support flexible antenna switching via a dynamic indication of specific SRS resource configuration for aperiodic SRS

- The indication can be done via MAC CE that allows RRC reconfigured version of SRS resource sets followed by a MAC CE

- Coverage and capacity enhancements

The followings are agreements for coverage and capacity enhancements and this is section, we discuss remaining details.

Agreement For comb-8 SRS in Rel-17, the maximum number of CSs is 6. - FFS: Whether a maximum number of 12 CSs is supported Agreement For the detailed pattern of khopping when start RB location hopping across legacy FH periods is enabled, support the following - For PF = 2, khoppingkhopping = {0, 1} - For PF = 4, khoppingkhopping = {0, 2, 1, 3} - Note: khopping={x0,⋯xPF-1} means khopping=xn mod PF for the (n+1)-th legacy FH period, where n = {0, 1, 2, 3, …} Working assumption To support 4 ports with Max CS = 6, - Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively. - Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively. - Note: n_CS and k_TC are the configured CS and comb offset values. - Note: This working assumption can be revisited if Max CS = 12 is agreed.

Dynamic signaling to determine PF and kF

It has been agreed that RRC signaling is used to indicate PF and kF. The following is to discuss whether more dynamic signaling, e.g., MAC CE or DCI can also be used to update these two values.

Proposal 5: Do not support to use MAC CE or DCI based update for PF and kF

CS allocation to support 4 ports with Max CS = 6 for Comb-8

In RAN1#106b-e, the maximum number of 6 CSs for Comb-8 was agreed. Further, it was concluded that there is no consensus in RAN1 to support maximum CS = 12 for Comb-8. The only remaining issue for Comb-8 is CS allocation to support 4 ports with maximum CS = 6 which has been discussed for several meetings and has been made as working assumption in RAN1#107-e as above. Hence, we support to confirm the working assumption for CS allocation to support 4 ports with maximum CS = 6 for Comb-8.

Proposal 6: Confirm the working assumption in RAN1#107-e for CS allocation to support 4 ports with maximum CS = 6 for Comb-8.

TP for clarification that no new sequence including length is introduced for RPFS

To support RPFS (RB-level partial frequency sounding), the following agreement has been made in RAN1#104-e.

Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

Although it was agreed that no new sequence length is introduced, it was not captured in the current Rel-17 specification. If the agreement is not captured in current Rel-17 specification, it is unclear whether sequence lengths which have not been supported in Rel-15/16 SRS can be supported by Rel-17 or not. That is, if there is no description, new sequence lengths can be supported at least for length larger than 30. To restrict not to introduce new sequence lengths for SRS, we support to adopt the following TP.

Proposal 7: Adopt the following TP to reflect the restriction not to support new sequence length as in the above agreement.

- 6.4.1.4.3 Mapping to physical resources <Unchanged text omitted> When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. If FreqScalingFactor is configured, the UE is not expected that Msc,bSRS is not an integer multiple of 6. <Unchanged text omitted>

- Conclusion

In this contribution, the following proposals are given:

- Proposal 1: No support new dropping rule in Rel-18.

- Proposal 2: No repurpose of DCI field(s) is supported for SRS parameter indication in Rel-17.

- Proposal 3: Group-common DCI can be used for triggering of AP-SRS with UE specific usage

  - With new RNTI where indication of all triggered AP-SRS sets’ slot offsets in one DCI

  - Bitmap structure or through individual indication with a specific assigned number of bits for each triggered set

- Proposal 4: Support flexible antenna switching via a dynamic indication of specific SRS resource configuration for aperiodic SRS

  - The indication can be done via MAC CE that allows RRC reconfigured version of SRS resource sets followed by a MAC CE

- Proposal 5: Do not support to use MAC CE or DCI based update for PF and kF

- Proposal 6: Confirm the working assumption in RAN1#107-e for CS allocation to support 4 ports with maximum CS = 6 for Comb-8.

- Proposal 7: Adopt the following TP to reflect the restriction not to support new sequence length as in the above agreement.

- 6.4.1.4.3 Mapping to physical resources <Unchanged text omitted> When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. If FreqScalingFactor is configured, the UE is not expected that Msc,bSRS is not an integer multiple of 6. <Unchanged text omitted>