3GPP TSG RAN WG1 #108-e	        		             R1-2202012

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.4.2

Source: 	Samsung

Title: 	Enhancements on UL time and frequency synchronization for NTN

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the remaining issues on UL time and frequency synchronization for NTN [1].

Discussions

N_TA for initial access

In RAN1#107-e the following working assumption was agreed regarding NTA update based on TA Command field in msg2/msgB at initial access:

Working assumption: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: - Option 1: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB

There has been some concerns that with the assumption above if a UE does not delay its initial transmission as least by the amount of +Te_NTN there may be some performance impacts due to undesired interference caused by the previous PRACH occasion or UL PUSCH/PUCCH transmission.  For that there are proposals to apply some additional delay in the equation captured in the working assumption to cope with this issue.  While we appreciate the existence of such issue and its consequences to impact the performance, we do not see any reason why we cannot leave this to the network.  Network can easily handle such an issue by including any required delay in NTA,common  broadcasted in the cell.  This way network will have the flexibility to apply any amount of desired delay in NTA at initial access and handle the issue of interference properly.

Proposal 1: Confirm the following working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ,

where, TA is the TAC field in msg2/msgB.

Open/Closed Loop TA control

NTA,UE-specific may be relatively static in certain scenarios, e.g. communication with a GEO satellite. In those instances, the gNB may only need to employ closed-loop TA control, reducing the computational burden on a UE to update NTA,UE-specific (which can be set to zero or set to its initial value). This motivates supporting closed-loop TA control (along with open-loop TA control and the combination of open&closed loop TA control, depending on the gNB configuration).

Proposal 2: Each of the following options are supported based on the gNB configuration:

- Closed-loop TA control

- Open-loop TA control

- Combination of open&closed-loop TA control

Common TA indication

Once RAN1 has completed the above-mentioned normative works, if RAN1 can identify any of the above-mentioned scenarios, then RAN1 should support GNSS-challenged UEs to enhance the robustness of NTN. In those scenarios:

- GNSS-challenged UEs can utilize a common TA value to perform coarse timing and frequency pre-compensation for uplink synchronization.

- The gNB should account for larger timing uncertainties in the received PRACH; for example, it can provision a longer guard time.

Also, in the previous RAN1#103-e meeting, it was agreed that in NTN, the network may broadcast a common timing offset value (while the details of this common timing offset are FFS). This common timing offset value can be set to the common TA value.

The common TA value can be indicated based on the altitude of the satellite and the additional residual propagation delay due to the slanted beam between the satellite and the ground.

Observation 1: The common TA, Tc, can be divided into the minimum common TA, Tc0, and a residual common TA, TcRes. The minimum common TA, Tc0, can be derived by UE from satellite ephemeris (or simply altitude) information without additional signalling.

Further enhancement

The minimum common TA value can be calculated as Tc0=h/c, where h is the altitude of the satellite and c is the speed of light.

The common TA value can then be determined as:

Tc=s∙hc+ TcRes		(2)

where s can be indicated to be 2 (or a larger value), given the Rel-17 NR NTN emphasis on transparent satellite payloads. Thus, TcRes will account for the total propagation delay from the service link and the access link. In a future release, if evolved NTN considers regenerative satellite payloads, then s can be indicated to be 1 if the common TA value only includes the propagation delay from the service link.

Proposal 3: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information.

When a UE can precisely estimate, and apply, a UE-specific TA for PRACH transmission, the residual TA value would be insignificant; it would fall within the TA value signaling range in the RAR. However, for GNSS-challenged UEs, the residual TA value may differ significantly from the indicated common TA value; if so, then it cannot be signaled to the UE using the TA value signaling range in the RAR:

- In 38.213, the maximum value ofis 3846, yielding which is 1,476,864 for 15 kHz SCS.

- Multiplying NTA by the NR basic time unit, i.e., Tc = 0.509 ns, yields 0.751 ms as the maximum TA that can be indicated in the RAR; this corresponds to a maximum cell size of 225 km.

- The typical satellite beam footprint size is 100 – 1000 km for LEO and 200 – 3500 km for GEO, which may exceed the above-mentioned maximum cell size.

One solution is to extend the current TA value signaling range in the RAR for NTN. Another solution is to signal several common TA values; for example, one cell can be divided into three zones as shown in Figure 3.

Figure 3. Multiple common TA values for multiple zones

For the central zone, the reference point for the common TA value is the center of the cell. For the surrounding zones 1 and 2, the reference points can be chosen as points A and B, respectively. Thus, a total of three common TA values can be utilized by the UEs; the common TA value that a particular UE utilizes can either be signaled by the satellite or derived by the UE if it knows its own location.

Proposal 4: Multiple reference points and common TA values should be considered for extremely large cells

Enhancements to TA maintenance phase

In the previous meetings, companies discussed the mechanics of the TA maintenance phase, including the possibility of supporting a combination of open and closed control loops to that end. The UE can utilize the TA variation rate when performing TA maintenance.

The TA variation rate and Doppler shift are proportional to each other; this is shown in Figure 5 for the basic UE-satellite configuration in Figure 4.

Figure 4. Basic UE-satellite configuration

Figure 5. TA variation rate and Doppler shift for basic UE-satellite configuration

The TA variation rate, rTA, and the Doppler shift, fD, due to the motion of the satellite are given by

rTA=vccosω,

fD=fcvccosω		(3)

where v is the speed of the satellite, i.e., v=GMER+h, c is the speed of light, ω is the angle between the satellite velocity and the LOS from the satellite to the UE, and fc is the carrier frequency. The UE can then use (3) to determine either 1) the Doppler shift if the TA variation rate is indicated or 2) the TA variation rate if the Doppler shift is indicated. The UE can also determine both the TA variation rate and the Doppler value based on the satellite ephemeris indication if the BS also indicates either ω or cosω.

Thus, the TA variation rate and the Doppler shift can be jointly signaled to the UE by indicating one of the following:

- TA variation rate rTA

- Doppler shift fD

- ω or cosω

- Frequency offset value to be compensated

Observation 2: The gNB jointly indicates the TA variation rate and the Doppler shift.

Observation 3: Based on the indicated TA variation rate r_TA (and the current TA), the UE can autonomously adjust its TA.

Observation 4: Based on the indicated Doppler shift f_D (and the compensated frequency offset), the UE can determine the residual Doppler shift and pre-compensate its UL transmission.

The gNB can also use a UE-specific DCI format to indicate the TA variation or the Doppler. The following options can be considered:

- A DCI format associated with the UE-specific search space can be re-used, where the interpretation of the DCI format can be to indicate the TA variation or the Doppler if all remaining fields/bits not used for the indication have predetermined values. For instance, DCI format 0_0 or 1_0 can be reused.

- A new DCI format can be introduced for UE-specific indication, where the CRC is scrambled by the C-RNTI. The size of the new DCI format (which differs from the size of DCI formats scheduling PDSCH/PUSCH) can be L1 bits. L2 out of L1 bits can be used to indicate the TA variation or the Doppler, and the remaining (L1-L2) bits can be reserved (e.g. set to ‘0’). To fulfill the budget for a maximum of three DCI formats with C-RNTI having different sizes, it is assumed that the DCI formats scheduling PDSCH/PUSCH have two sizes (and size-matching is performed when necessary). Admittedly, this approach will limit possible schedulable DCI formats to UE.

A group-common (GC) DCI format with a corresponding RNTI (TDI-RNTI) can also be used to indicate the TA variation and/or Doppler to a set of UEs located in a certain area in the cell that are experiencing a similar TA variation rate and/or Doppler shift.

The gNB can also use the TAC MAC CE in a scheduled PDSCH reception to indicate the TA variation or Doppler. For example, the gNB can extend the size of the TAC MAC CE to indicate the TA variation or Doppler; this is shown in Figure 6.

Figure 6. Extension of TAC MAC CE for indication of TA variation and/or Doppler.

The PDSCH carrying the TAC MAC CE can be scheduled by either a UE-specific DCI or a group-common DCI with a TDI-RNTI.

Proposal 5: The gNB signals common TA drift rate to enable autonomous TA update at UE.

Proposal 6: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa.

UL frequency synchronization

Figure 5. The gNB indicates the additional UL frequency offset value.

In Figure 7, DL center frequencies at DL at gNB, satellite, and UE are shown. The UE can calculate frequency offset ∆f_2 that occurs in service link based on the position information of the satellite. The frequency offset ∆f_1 that occurs in feeder link can be pre-compensated by the gNB for DL. The other approach could be for the gNB to indicate ∆f_1 to UEs. Since a UE in Rel-17 enables to calculate ∆f_2 based on the GNSS information, the gNB may not need to indicate the additional offset for the UE. The UE pre-compensates the frequency offset ∆f_2 before it performs PRACH transmission based on SIB and GNSS information. After RRC connection, the UE can still continue to pre-compensate ∆f_2 before uplink transmission. However, similar to TA adjustment, the additional indication of frequency offset ∆f_2 could be used for fine tuning for UL frequency synchronization

Proposal 7: The gNB indicates the additional UL frequency offset value for the pre-compensation at UE side.

Conclusions

In this contribution, we discussed several enhancements on UL time and frequency synchronization for NTN.  Our proposals and observations are summarized as follows.

Observation 1: The common TA, Tc, can be divided into the minimum common TA, Tc0, and a residual common TA, TcRes. The minimum common TA, Tc0, can be derived by UE from satellite ephemeris (or simply altitude) information without additional signalling.

Observation 2: The gNB jointly indicates the TA variation rate and the Doppler shift.

Observation 3: Based on the indicated TA variation rate r_TA (and the current TA), the UE can autonomously adjust its TA.

Observation 4: Based on the indicated Doppler shift f_D (and the compensated frequency offset), the UE can determine the residual Doppler shift and pre-compensate its UL transmission.

Proposal 1: Confirm the following working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ,

where, TA is the TAC field in msg2/msgB.

Proposal 2: Each of the following options are supported based on the gNB configuration:

- Closed-loop TA control

- Open-loop TA control

- Combination of open&closed-loop TA control

Proposal 3: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information.

Proposal 4: Multiple reference points and common TA values should be considered for extremely large cells

Proposal 5: The gNB signals common TA drift rate to enable autonomous TA update at UE.

Proposal 6: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa.

Proposal 7: The gNB indicates the additional UL frequency offset value for the pre-compensation at UE side.

- References

- RP‑210908, “Revised WID: Solutions for NR to support non-terrestrial networks (NTN).”