3GPP TSG RAN WG1 #108-e                               	R1-2202020

e-Meeting, Feb 21st ~ Mar 3rd, 2022

Agenda Item:		8.6.1.1

Source:				Samsung

Title:				UE complexity reduction

Document for:	    Discussion

Introduction

In RAN1 #107-e, the following agreements were made for bandwidth reduction techniques of RedCap UEs:

Agreement - In Rel-17, up to 1 separate initial UL BWP for RedCap can be configured. Agreement - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation (except for standalone use for RRM measurement) based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement - Send an LS to RAN2 and RAN4 to inform them about relevent RAN1 agreement on FR1 and corresponding agreement on FR2, as well as the working assumption, and ask them whether the working assumption reasonable or not: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - Indicate in the LS that RAN1 does not expect any futher RAN1 specification impact from the above working assumption. - Also include the following RAN1 agreement in the LS as background information: - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement - When the frequency hopping for the RedCap PUCCH resources (for HARQ feedback for Msg4/MsgB) is deactivated, - Each PUCCH resource is mapped to a single PRB. - What side[(s)] of the RedCap UL BWP center frequency to which PUCCH resources are mapped is[/are] configurable by the network, including SIB-configurable [additional] offset (with no more than [4] candidate values) from edge using the existing equations for determining the PRB index of the PUCCH transmission as a starting point. - RedCap and non-RedCap can be configured with the same or different PUCCH resource set indices (see TS 38.213 Table 9.2.1-1). Agreement - For a separate initial DL BWP for RedCap UEs, - The supported bandwidths for the separate initial DL BWP for RedCap UEs can have any values up to the maximum UE bandwidth (as in legacy operation). R1-2112801 [Draft] LS on use of NCD-SSB in initial DL BWP used for paging for RedCap UE Ericsson Decision: As per email decision posted on Nov 20th, the draft LS is endorsed. Final version is approved in R1-2112802

This contribution discusses the support of RedCap UE with bandwidth reduction.

Discussion on bandwidth reduction

There are several open issues remaining related to bandwidth reduction for RedCap UEs. Summarized as below:

- Separate initial DL BWP related issues

  - Issue 1: Should a separate SIB-configured initial DL BWP for RedCap always be configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UEs bandwidth?

  - Issue 2: DCI size of DCI format1_0 in CSS

- Measurement for RedCap UEs

  - Measurement for paging

  - Measurement for msg1/A retransmission

- RedCap UEs supporting band n79

Separate initial DL BWP related issues

Issue 1: Should a separate SIB-configured initial DL BWP for RedCap always be configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UEs bandwidth?

According to the agreement in RAN1#106-e meeting, the initial DL BWP for non-RedCap UEs can’t be used for RedCap UE if the bandwidth exceed the maximum RedCap UEs bandwidth. Thus, if separate SIB-configured initial DL BWP is not configured in this scenario, there are two alternatives,

- Alt1: RedCap UEs are not allowed to access the cell

- Alt2: RedCap UEs using CORESET#0 as initial DL BWP

To further analysis the alternatives, consider cases in below.

- Case 1: separate initial UL BWP is not configured, and initial UL BWP for non-RedCap UEs is smaller than maximum RedCap UEs bandwidth.

If the initial UL BWP for non-RedCap UEs is not wider than maximum BW of RedCap UE, RedCap UEs can reuse initial UL BWP for non-RedCap UEs and using CORESET#0 as initial DL BWP during and after RA, which is Alt2. As shown in Figure 1 below, it is fine if CORESET#0 bandwidth and initial UL BWP is within maximum RedCap UEs bandwidth, but if it doesn’t, RF retuning between UL and DL is needed. This will bring challenges to UE implementation for UL and DL processing time after random access, for RedCap UEs without non-initial DL BWP configured after random access. To avoid the RF retuning, additional restriction on CORESET#0 frequency position shall be added in this case, which brings further specification impact.

    - (b)

Figure 1

- Case 2: separate initial UL BWP is configured

As shown in Figure 2, similar as that in case 1, Alt2 will bring UE implementation complexity or impact on specification. Besides, as agreed in RAN1#106bis-e meeting, “For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs.”, but if there is no SIB configured initial DL BWPs or CORESET#0 is used as initial DL BWP for RedCap UEs, the agreement above needs to be further clarified.

    - (b)

Figure 2

Observation#1: When separate SIB-configured initial DL BWP is not configured for RedCap UE and the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, Alt2 (RedCap UE using CORESET#0 as initial DL BWP) either brings UE implementation complexity or impact on specification.

Based on the above analysis, we have below proposal considering both implementation complexity and specification impact from RAN1 point of view.

Proposal#1: For a cell that allows RedCap UEs to access, a separate SIB-configured initial DL BWP for RedCap UEs shall always be configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UEs bandwidth.

Issue 2: DCI size of DCI format1_0 in CSS

According to 38.212, for DCI format 1_0 monitored in a CSS, bits of ‘Frequency domain resource assignment’ in DCI is determined by

  - the size of CORESET 0 if CORESET 0 is configured for the cell; and

  - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

When a RedCap UE receives PDSCH scheduled with DCI format 1_0 in CSS within a separate initial DL BWP, ‘Frequency domain resource assignment’ in DCI may indicate a frequency allocation based on bandwidth which exceed the initial DL BWP. As shown in Figure 3, bandwidth of CORESET#0 may be larger than the used CORESET# (not CORESET#0), and it may exceed the initial DL BWP.

Figure 3

The case above also happens to legacy UEs. For example, when a legacy UE receives PDSCH scheduled with DCI format 1_0 in CSS which associated with CORESET# (not CORESET#0). Network shall only allocate the bandwidth within the active DL BWP in this scenario.

Besides, if restrict frequency resource to CORESET# instead of CORESET#0 in this scenario, DCI bits will be saved and therefore benefit for PDCCH coverage. However, it will increase the type of DCI size and impacts DCI budget for UE’s blind detection.

With analysis above, there is no potential issue to keep the same rule as legacy UEs in this scenario.

Observation#2: RedCap UEs can determine the size of DCI format1_0 in CSS as that for legacy UEs. Thus, no specification change is needed.

Measurement for RedCap UEs

Measurement for paging

As agreed in RAN1#107-e meeting, LS is sent from RAN1 to RAN2 and RAN4 to inform them about relevant RAN1 agreement. And RAN2 reply LS in R2-2201759 as below.

Regarding the following working assumption for FR1 and FR2 related to a separate initial DL BWP: “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective” - RAN2 agreed that “A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB.” Therefore, using an NCD-SSB for such purposes is not considered further.

For a separate initial DL BWP (not include CD-SSB and the entire CORESET#0),

- According to the LS reply from RAN2, in RRC_IDLE/RRC_INACTIVE mode, RedCap UEs monitor the CSS for paging configured in initial DL BWP of non-RedCap UEs which associated with CD-SSB.

- In RRC_CONNECTED mode, if paging is configured in the separate initial DL BWP, then NCD-SSB can be used for paging monitoring. It is supported already for the non-initial DL BWP of R15 UEs. Thus, no specification change is needed for RedCap UEs.

Observation#3: For paging monitoring, RedCap UEs follow the same rule as legacy UEs, and no specification change is needed.

Proposal#2: Agree on the following conclusion:

From RAN 1 perspective, for a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging for RRC_CONNECTED mode, RedCap UEs follow the same rule as legacy UEs.

Measurement for msg1/A retransmission

As agreed in RAN1 #107-e meeting, RAN1 sent LS to RAN2/RAN4 in R1-2112802. And regarding the item below in the RAN1 LS, RAN2 sent LS to inform RAN1 in R2-2201760 about the agreement in RAN2-116bis-e meeting.

- For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

  - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

RAN2 agreement in RAN2-116bies-e meeting

From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

If a RedCap UE in idle/inactive mode is configured with a separate initial DL BWP which not contain SSB, then when the RedCap UE performs new RSRP measurement before Msg1/A retransmission, it needs to measure SSB, then switch back to separate initial UL BWP for Msg1/A retransmission, illustrated in Figure 4 .

Figure 4

Currently, higher layers of UE can indicate to the physical layer to transmit a PRACH, and if requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception. Generally, it takes longer time to prepare Msg1/A retransmission comparing with scenarios that no measurement or having SSB within separate initial DL BWP.

As agreed in RAN2, it is up to UE implementation whether to perform new RSRP in this scenario, however, with the time restriction in RAN1, it is not feasible to perform the measurement. To make it feasible for UE implementation in this scenario, there are three alternatives:

- Alt1: RedCap UEs not follow current time restriction for PRACH transmission, i.e.,  msec

- Alt2: Extend time restriction for RedCap UEs by adding delta to current time restriction, i.e.,  +delta msec

- Alt3: RedCap UEs keep current time restriction and trigger higher layer indication after measurement

Observation#4: Even though it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission, it is not feasible with current time restriction.

Alternatives are analyzed as below.

- Alternative 1

For Alt1, without time restriction, RedCap UEs which implement the measurement can determine the time interval according to its capability and implementation. And it has limit impact on specification.

- Alternative 2

For Alt2, the added time ‘delta’ shall be predefined. It can be spilt into BWP switch time and measurement time, with formula:

Delta = 2*BWP switch time + measurement time.

- BWP switch time

Currently, depending on UE capability bwp-SwitchingDelay, UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1 for DCI/timer based BWP switch delay on a single CC. BWP switch time in the formula above can be determined by the same table. For example, for RedCap UEs with type1 capability in 15KHz subcarrier spacing, the BWP switch time is 1ms.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

- Measurement time

Measurement time can refer to the measurement gap in specification. According to 38.306, parameter ‘supportedGapPattern’ indicates the optional supported measurement gap pattern, which is start from Gap pattern ID 2. And the Gap pattern ID 0/1 are mandatory for UEs. Thus, from the gap pattern configuration table below, 6ms can work for all types of UE.

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

- Alternative 3

Currently, higher layers of UE can indicate to the physical layer to transmit a PRACH,

- If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or

- If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or

- If the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or

- If the higher layers do not identify the RAPID associated with the PRACH transmission from the UE.

For Alt3, lower layer of RedCap UEs can indicate high layer after the new measurement, then with new measurement indication, high layer can further indicate PRACH transmission. In this way, RedCap UEs can keep current time restriction and do the new measurement with enough processing time. Correspondingly, an additional condition shall be added to current triggers: if higher layers receive new measurement indication from lower layer, higher layers of UE can indicate to the physical layer to transmit a PRACH. Moreover, it may bring UE complexity on higher layer and lower layer interface with the new added indication.

Observation#5: Extending time restriction or adding trigger to higher layer indication after measurement for RedCap UEs brings larger speciation impact and UE complexity.

With analysis above, Alt1 is preferred considering speciation impact and UE complexity.

Proposal#3: RedCap UEs configured with a separate initial DL BWP (if it does not include SSB), not follow the time restriction on PRACH transmission/re-transmission as legacy UE does.

RedCap UE supporting band n79

In RAN#93-e meeting, below agreement is conclueded in RP-212634 for RedCap UE supporting band n79.

- Band n79 can be supported by RedCap UEs. RedCap WI will add the RedCap specific requirements for n79 (e.g. related to 10 and 20MHz with 1Rx) - Adding channel bandwidths to band n79 for NR operation shall be specified prior to specifying the RedCap requirements for n79

In RAN#94-e, the updated channel bandwidth (CBW) support to the band n79 was approved for the new scenarios (e.g., RedCap) based on RAN4’s agreements. After RAN#94-e meeting, RAN4 identified a backward compatible issue due to the minimum bandwidth change of n79 and sent LS in R4-2202286 with potential solutions to RAN1 for further input.

- Identified issue from RAN4 LS

Since the band n79 newly introduces {10, 20, 30, 70, 90} MHz to existing channel bandwidths {40, 50, 60, 80, 100} MHz, the minumum channel bandwidth of n79 has been changed from 40 MHz to 10MHz. In TS 38.213, upon detection of SS/PBCH block, UE determines CORESET#0 from MIB by looking up the table for controlResourceSetZero. The table is determined according to subcarrier spacing of SSB, subcarrier spacing of PDCCH, and minimum channel bandwidth of the frequency band where UE located. For example, a new UE supporting 10 MHz the minimum channel bandwidth for n79 will have a different table (table 13.4 in TS 38.213) for CORESET#0 configuration with a legacy UE still supporting 40 MHz (table 13.6 in TS 38.213). Then, how to determine the table in this scenario can be an issue that needs further clarifications.

Observation#6: With new introduced minimum channel bandwidth of n79, there is backward compatible issue for CORESET#0 configuration table determination.

- Potential solutions from RAN4 LS

RAN4 has come up with four potential solutions,

- Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration.

- Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13.6 to legacy UE and new UE for CORESET#0 configuration.

  - Rationale: Legacy UE not supporting channel bandwidth lower than 40MHz will always look at table 13.6, this table shall then be the common one.

- Alt-1b: Add narrower channel bandwidth to n79, and indicate:

  - A UE synchronizing on a GSCN with step 4 shall look at the table 13.6.

    - Rationale: This enables new and legacy to connect in band n79 using the same CORESET#0 configuration.

  - A UE synchronizing on a GSCN with step 1 but not step 4 shall look at the table 13.4.

    - Rationale: Table 3.4 offers more flexibility on CORESET#0 configuration

- Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

Even though the final solution is still under discussion, RedCap UEs can follow the new UE’s behavior in the final solution for CORESET#0 table determination. Thus, RedCap UEs can co-exist with legacy UE automatically.

Observation#7: Solution of backward compatible issue on band n79 will be discussed in R1-2201973. And RedCap UEs can co-exist with legacy UE automatically with the same solution.

Proposal#4: RedCap UEs follow the same solution for backward compatible issue on band n79.

Conclusion

This paper discussed the remaining issues related to bandwidth reduction for RedCap UEs. The following observations were made:

Observation#1: When separate SIB-configured initial DL BWP is not configured for RedCap UE and the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, Alt2 (RedCap UE using CORESET#0 as initial DL BWP) either brings UE implementation complexity or impact on specification.

Observation#2: RedCap UEs can determine the size of DCI format1_0 in CSS as that for legacy UEs. Thus, no specification change is needed.

Observation#3: For paging monitoring, RedCap UEs follow the same rule as legacy UEs, and no specification change is needed.

Observation#4: Even though it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission, it is not feasible with current time restriction.

Observation#5: Extending time restriction or adding trigger to higher layer indication after measurement for RedCap UEs brings larger speciation impact and UE complexity.

Observation#6: With new introduced minimum channel bandwidth of n79, there is backward compatible issue for CORESET#0 configuration table determination.

Observation#7: Solution of backward compatible issue on band n79 will be discussed in R1-2201973. And RedCap UEs can co-exist with legacy UE automatically with the same solution.

Based on the observation and the analysis, the following proposals were made:

Proposal#1: For a cell that allows RedCap UEs to access, a separate SIB-configured initial DL BWP for RedCap UEs shall always be configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UEs bandwidth.

Proposal#2: Agree on the following conclusion:

- From RAN 1 perspective, for a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging for RRC_CONNECTED mode, RedCap UEs follow the same rule as legacy UEs.

Proposal#3: RedCap UEs configured with a separate initial DL BWP (if it does not include SSB), not follow the time restriction on PRACH transmission/re-transmission as legacy UE does.

Proposal#4: RedCap UEs follow the same solution for backward compatible issue on band n79.