3GPP TSG RAN WG1 #108	                      R1-2202024

# e-Meeting, February 21th – March 3rd, 2022

Agenda Item:		8.7.1.2

Source:				Moderator (Samsung)

Title:					Moderator summary#1 on TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion/Decision

# Introduction

This document provides summary of contributions [1-20] submitted to agenda item 8.7.1.2 for RAN1#108-e meeting, and corresponding technical discussion in the meeting. According to the proposals in contributions [1-23], the remaining issues for supporting TRS/CSI-RS occasion(s) for idle/inactive UEs can be divided into the following parts, wherein the details are captured in Section 2 to 6, respectively:

- Section 2: TRS availability indication content

  - Issue 1-1: TP for CRs of TS 38.213

  - Issue 1-2: Clarification on default assumption

  - Issue 1-3: Whether/how to restrict consistent TRS availability indication from PEI and paging PDCCH

  - Issue 1-4: Whether to support PEI based TRS availability indication only for RS resources with same QCL references

- Section 3: TRS validity duration

  - Issue 2-1: TP for CRs of TS 38.213

  - Issue 2-2: Clarifications on reference point

- Section 4: TRS resources configuration

  - Issue 3-1: TP for CR of TS 38.214

  - Issue 3-2: Maximum number of TRS resource sets

  - Issue 3-3: Whether/how to reduce configuration overhead

- Section 5: Others

The issues in this document are colour coded with High Priority, Medium Priority, or Low Priority.

Per chairman’s instruction, this document will be used for the following email discussion:

[108-e-R17-PowSav-02] Email discussion for maintenance on TRS/CSI-RS occasions for idle/inactive UEs – Qiongjie (Samsung) - 1st check point: February 25 - Final check point: March 3

For the first round discussion, please kindly provide your comments on moderator proposals or questions tagged ‘[1RD]’ before Feb, 22th, Tuesday, UTC 16:59.

# TRS availability indication content

The following were agreed regarding TRS availability indication:

From RAN1#107bis-e: Agreement Confirm the following working assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI. Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently.

In contributions [1-20], proposals regarding remaining issues for TRS availability indication content are captured in table below:

| Huawei, HiSilicon |  |
| --- | --- |
| ZTE, Sanechips | A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle. ...................................... Text Proposal for TS38.214............................................................. 5.1.6.1.1 CSI-RS for tracking ... For each [TRS-ResourceSet] the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter [indBitID]. A UE does not expect to receive inconsistent TRS availability indication from DCI format 2-7 and DCI format 1-0 with CRC scrambled by P-RNTI within a default paging cycle. ................................. End of Text Proposal for TS38.214............................................................... - Proposal 3: The following TP for CR of TS 38.213 should be adopted. .................................................... Text Proposal for TS38.213........................................................... 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. *** Unchanged text is omitted *** .................................... End of Text Proposal for TS38.213..................................... |
| Vivo |  |
| TCL |  |
| Spreadtrum | Proposal 1: For PEI PDCCH based availability indication, the bitmap can have small bit size, e.g. L1 availability indication at an occasion can provide availability information only for RS resources with QCL references to be the same as for the L1 availability indication occasion. |
| CATT | Proposal 1: If DCI format 2_7 is configured, the size and association between bitmap and TRS resources sets of TRS availability indication field of DCI format 2_7 should be same as that of the paging DCI. Proposed TP for TS 38.213 #### 10.4B Indication of TRS resources ------------------------- Unchanged text is Omitted --------------------------------------------------------- A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 and or a DCI format 1_0 with CRC scrambled by P-RNTI includes the a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap, if DCI format 2_7 is configured. ------------------------- Unchanged text is Omitted ---------------------------------------------------------. Proposal 3: The flowing TP should be adopted to clarify the meaning of value of '0' for a bit of the bitmap. Proposed TP for TS 38.213 #### 10.4B Indication of TRS resources ---------------------------------------------------- Unchanged text is Omitted --------------------------------------------------------- A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by indicates that the UE keeps the existing assumption of the availability or unavailability of the corresponding TRS resource set(s). ---------------------------------------------------- Unchanged text is Omitted ----------------------------------------------------------. |
| OPPO | ********************* Start of text Proposal************************************* ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an Id i which indicate the association to the ith bit within the bitmap. The UE can be additionally provided a multiple, by validityDuration, of a number of frames provided by defaultPagingCycle; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] which is the first frame within the default DRX cycle that a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets is received, where T is provided by defaultPagingCycle. ********************** End of text Proposal*************************************** |
| Sony |  |
| Intel | Proposal 1: The default assumption, i.e., before UE receives L1 availability indication after camping on a cell, is that the configured group(s) of TRS resource set(s) are unavailable. |
| Xiaomi | Proposal 1: If L1 indication is configured, before UE receive a first L1 indication after entering a cell, UE should assume the TRS/CSI-RS for idle is not available. And related TP text can be as follows: TP#1 for TS 38.214 Clause 5.1.6.1.1 ============================= Unchanged part omitted ========================================= 5.1.6.1.1 CSI-RS for tracking A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. <omitted text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a [TRS-ResourceSetConfig]. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of two periodic NZP CSI-RS resources in one slot or by a [TRS-ResourceSet] of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter [TRS-Resource] in a [TRS-REsourceSet] with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the [TRS-ResourceSet] and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is not restricted by initial DL BWP. - [UE is not expected to receive TRS occasions outside the initial DL BWP.] - the periodicity and slot offset for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a [TRS-ResourceSet], is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a [TRS-ResourceSet]. - the UE does not expect the [TRS-ResourceSet] to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by [TRS-ResourceSet] to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a [TRS-ResourceSet] applies to all resources in a [TRS-ResourceSet]. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a [TRS-ResourceSet], is a SS/PBCH block, applies to all resources in a [TRS-ResourceSet]. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. Immediately after UE camped on the serving cell, if L1 TRS availability indication is configured via a [TRS-ResourceSetConfig], UE assumes no TRS resource is available before detecting the first L1 TRS availability indication on the serving cell. ============================= Unchanged part omitted ========================================= Proposal 2: If L1 TRS availability indication is configured both in paging DCI and PEI, for UE monitoring PEI, UE would acquire L1 indication in PEI and ignore L1 indication in paging DCI. And related TP text can be as follows: TP#2 for TS 38.214 Clause 5.1.6.1.1 ============================= Unchanged part omitted ========================================= 5.1.6.1.1 CSI-RS for tracking A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. <omitted text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a [TRS-ResourceSetConfig]. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of two periodic NZP CSI-RS resources in one slot or by a [TRS-ResourceSet] of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter [TRS-Resource] in a [TRS-REsourceSet] with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the [TRS-ResourceSet] and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is not restricted by initial DL BWP. - [UE is not expected to receive TRS occasions outside the initial DL BWP.] - the periodicity and slot offset for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a [TRS-ResourceSet], is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a [TRS-ResourceSet]. - the UE does not expect the [TRS-ResourceSet] to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by [TRS-ResourceSet] to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a [TRS-ResourceSet] applies to all resources in a [TRS-ResourceSet]. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a [TRS-ResourceSet], is a SS/PBCH block, applies to all resources in a [TRS-ResourceSet]. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. If L1 TRS availability indication is configured both in paging DCI and PEI, for UE monitoring PEI, UE would acquire L1 indication in PEI and ignore L1 indication in paging DCI. ============================= Unchanged part omitted ========================================= Proposal 3: If L1 TRS availability indication is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable. And related TP text can be as follows: TP#3 for TS 38.214 Clause 5.1.6.1.1 ============================= Unchanged part omitted ========================================= 5.1.6.1.1 CSI-RS for tracking A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. <omitted text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a [TRS-ResourceSetConfig]. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a [TRS-ResourceSet] consists of two periodic NZP CSI-RS resources in one slot or by a [TRS-ResourceSet] of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter [TRS-Resource] in a [TRS-REsourceSet] with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the [TRS-ResourceSet] and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is not restricted by initial DL BWP. - [UE is not expected to receive TRS occasions outside the initial DL BWP.] - the periodicity and slot offset for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a [TRS-ResourceSet], is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a [TRS-ResourceSet]. - the UE does not expect the [TRS-ResourceSet] to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by [TRS-ResourceSet] to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a [TRS-ResourceSet] applies to all resources in a [TRS-ResourceSet]. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a [TRS-ResourceSet], is a SS/PBCH block, applies to all resources in a [TRS-ResourceSet]. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. If L1 TRS availability indication is configured via [SIB], and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable. ============================= Unchanged part omitted ========================================= |
| CMCC |  |
| Panasonic | Proposal 1: UE default assumption of TRS availability should be described as "may" instaed of "shall", if either of SIB configuration or L1 indication is temporarily absent or unreliable in below cases: - A UE just camps on a cell or received an updated TRS configuration, before the UE receives a L1 TRS availability indication. - During the system modification period and before the new SIBs become valid, a UE can not acquire the up to date TRS configuration but still stores previous TRS configuration. Proposal 4: UE assumes the TRS availability indication consistent between PEI and associated paging DCI. If UE detects inconsistent indication, this is treated as error case and no need to specify. |
| Samsung |  |
| Apple |  |
| InterDigital | Conclusion 2: If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE can assume the configured group(s) of TRS resource set(s) are unavailable. |
| Sharp | Proposal 2: TRS availability indications in PEI-DCI can be inconsistent with the indication in paging DCI Proposal 3: Support providing availability indication via PEI for TRS resources with the same QCL references |
| LG | Proposal 2: UE assumes the indicated validity durations are temporally unavailable from the start of modification period where SI acquisition procedure is applied until it acquires a SIB1 message - If a value tag of the SI for TRS configuration is changed from the stored version - the validity durations indicated before the current modification period are unavailable - UE acquires SIB-X to update TRS configuration - UE can apply L1 based availability indication based on updated TRS configuration from the current modification period - Otherwise, if a value tag of the SI for TRS configuration is not changed from the stored version - UE can use stored version of SI for TRS configuration - UE can re-assume TRS for the indicated validity durations, if available |
| Ericsson |  |
| Qualcomm |  |
| MediaTek |  |
| Nokia | Observation: If PO and associated PEI fall to different default paging cycles, the L1 availability indication could be different in the paging DCI and PEI. Observation: Having different L1 indication value in paging DCI and associated PEI would not cause any UE ambiquity as UE can determine the availability assumption based on the latest L1 availabilty indication with value ‘1’. |
| Nordic |  |
| Lenovo |  |

According to the above proposals, the remaining issues regarding TRS availability indication include:

- Issue 1-1: TP for CRs of TS 38.213

- Issue 1-2: Clarification on default assumption

- Issue 1-3: Whether/how to restrict consistent TRS availability indication from PEI and paging PDCCH

- Issue 1-4: Whether to support PEI based TRS availability indication only for RS resources with same QCL references

## 2.1 <1st round discussion>

### Issue 1-1: TP for CR of TS 38.213

The proposed TPs for CRs of TS 38.213 regarding TRS availability indication based on existing agreements are collected in the table below.

| Index | Proposed TP | From |
| --- | --- | --- |
| 1 | ............................. Text Proposal for TS38.213.................................. 10.4B Indication of TRS resources (text omitted) A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. *** Unchanged text is omitted *** ................ End of Text Proposal for TS38.213..................................... | ZTE |
| 2 | #### 10.4B Indication of TRS resources -------------- Unchanged text is Omitted -------------------------------- A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 and or a DCI format 1_0 with CRC scrambled by P-RNTI includes the a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap, if DCI format 2_7 is configured. --------------- Unchanged text is Omitted ------------------------- | CATT |
| 3 | #### 10.4B Indication of TRS resources --------------------------------- Unchanged text is Omitted --------------------------------------------------------- A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by indicates that the UE keeps the existing assumption of the availability or unavailability of the corresponding TRS resource set(s). ---------------------------------------------------- Unchanged text is Omitted ---------------------------------------------------------- | CATT |
| 4 | ********************* Start of text Proposal************************************* ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an Id i which indicate the association to the ith bit within the bitmap. The UE can be additionally provided a multiple, by validityDuration, of a number of frames provided by defaultPagingCycle; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] which is the first frame within the default DRX cycle that a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets is received, where T is provided by defaultPagingCycle. ********************** End of text Proposal*************************************** Proposal*************************************** | OPPO |

[1RD]

Please a) provide your views on whether or not to support any of the TPs above.

| TP | Companies |
| --- | --- |
| #1 | - Yes: Nokia, Nordic, vivo (partially), LGE,Sharp,ZTE,Sanechips, intel (with update), MediaTek, , Ericsson1, IDCC - No: CATT, Samsung,Xiaomi |
| #2 | - Yes: CATT, Nordic, Qualcomm, Apple, vivo (partially), LGE,ZTE,Sanechips(partially), intel, CMCC, Panasonic - No: ,Xiaomi, MediaTek (support Xiaomi’s revision) |
| #3 | - Yes: CATT, Qualcomm,Xiaomi, intel, Huawei, HiSilicon, CMCC, Ericsson1 (with updates), IDCC - No: Nordic, vivo |
| #4 | - Yes: Nokia, Nordic, Apple (only PEI-RNTI part) ,Xiaomi, LGE, intel, Huawei/HiSilicon(Only PEI-RNTI part), CMCC, IDCC - No: CATT, vivo |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | For #2; no strong view as such, but isn’t this covered in 38.212 (by ” - TRS availability indication – 1, 2, 3, 4, 5, or 6 bitsif TRS-ResourceSetConfig is configured; 0 bits otherwise.” |
| CATT | For TP#1, the texts of associated TRS validity should remain. For TP#4, most of changes are not necessary. |
| Nordic | For #3 if “ignored” is not liked, we could say “is discarded by the UE” |
| Apple | For #1 and #3, we think the TPs are not essential. Either way is correct and there is no ambiguity. But we can go with the majority view. For #4, we think only “PEI-RNTI” change is needed, and all the other parts do not seem necessary. |
| Samsung | For TP#1, we think the texts should not be removed. Otherwise it’s ambiguous whether or not UE can be indicated with new TRS availability indication before the expiration/end of validity duration associated with previous L1 based signalling for TRS availability indication. For TP#4, we don’t agree with the first two modifications. 1) RNTI for DCI format 38.213 is removed in latest CR by editor with justification. 2) The description of the association between each bit and TRS resource set is clear in the original CR. For the last modification on reference point for validity duration, we are generally OK. It’s also under discussion for Issue 2-1. |
| vivo | For the text removed by TP#1, the intention seems to capture the following agreements. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI. And TP#2 seems also trying to capture the above agreements. In our understanding, the above agreements should be captured properly. Hence, we suggest the following TP based on TP#1 and TP#2. ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 if configured or ,and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames.. |
| Xiaomi | For TP #1, Form our view, the deleted part is a common understanding and it would good to have it to make the spec more clear. but if majority think it is redundant and want to delete it, we can also accept it. For TP #2, We understand the intention is to say that if DCI 2-7 is not configured, than TRS availability indication would not be configured in DCI 2-7. And we can agree with the intention, but the current wording in TP #2 seems saying that, if DCI 2-7 is not confogured, than TRS availability indication would not be provided in DCI 1-0 as well. We prefer a different wording as the following, -------------- Unchanged text is Omitted -------------------------------- A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI and a DCI format 2-7, if DCI format 2_7 is configured, includes the a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. --------------- Unchanged text is Omitted ------------------------- For TP #4, TP #2 and TP #4 should be merged. |
| LGE | For TP#3, we don’t have strong view, but it seem like TP is not essential. For TP#4, we tend to agree with Apple’s view. Especially we have concern with “within the default DRX cycle” . |
| ZTE, Sanechips | For #1, in our understanding, we think the description that “A value of '0' for a bit of the bitmap is ignored by the UE/” is sufficient, and the last sentence can be removed. Because even without the last sentence, the following can be inferred. 1) No matter whether there is an on-going validity time duration or not, if UE receives an indication of “1”, the associated resource set is valid from the SFN determined by the formula till the multiple of the number of frames; 2) If the TRS resources are not valid, it is still invalid after reception of “0”. If there is an on-going validity time duration, the duration will not change if the indication of “0” is ignored by UE. Moreover, with the last sentence, - it is unclear whether it refers to each bit or a whole L1 signaling. - the possibility of indicating “0” during the on-going validity time duration is precluded. For #2, the version provided by vivo is preferred. For the current TP#2, it may imply that the indication can be carried by DCI format 1-0 and PEI, only when PEI is configured. Which is not aligned with the previous agreements. For #3, okay to clarify the implication of“A value of '0' for a bit of the bitmap is ignored by the UE” or the proposed update. For #4, in our opinion, the ‘PEI-RNTI’ is removed by the editor in the latest CR. Other revisions seem not needed. For example, description of the TRS availability indication filed is clear in TS 38.331 CR shown as following. indBitID The index of the associated bit in TRS availability indication field in DCI. Each TRS resource set is configured with an ID i for the association with i-th indication bit in TRS availability indication field in DCI.. |
| Intel | For TP# 1, the intention seems to capture that UE can receive new L1 availability indication before the validity duration triggered by a previous L1 indication expires. However, we are not sure the following part is generally applicable “…….where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. “ The “time” gap can be smaller than validity duration. We are not sure whether we had agreement that time between PEI and paging DCI is always smaller than validity duration. Without this part, we think description is clear as follows. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames. where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. The proposed change above does not impose any restriction and UE is allowed to receive two L1 indications within an active validity duration. These two L1 indications can be paging DCIs (if PEI is not configured) or PEI followed by paging DCI. Support TP # 2, 3, 4 |
| Huawei, Hisilicon | TP#2: We can understand the intention of TP#2. But we agree with Nokia that 212 is already clear enough. TP#3: The updated description seems clearer and is copied&pasted from the agreement. We are fine with it. For TP#4, the last part we are open to discuss it in issue 2-1. |
| CMCC | TP#1: this TP is also related to issue 1-3. TP#3: It is better to clarify the UE behavior of receiving value ‘0’. |
| Ericsson1 | #2 : not needed since it is covered by 38.212. #3 : suggested update “indicates that the UE keeps the existing assumption of the availability or unavailability of TRS in the corresponding TRS resource set(s).” #4 : This has overlap with Issue 2-1 and prefer to discuss together. Some changes are not needed such as adding PEI-RNTI, “which is the first frame”. |
| Moderator1 | On TP#1: - [CATT, Samsung, Xiaomi] have concerns to remove the texts. Without the texts it’s ambiguous “whether the availability indication is transmitted [only once] during the validity duration”. - To follow up with Intel’s comment, the text doesn’t consider time between PEI and paging DCI. DCI format 2_7 or DCI format 1_0 is just a DCI format that can provide the TRS availability indication. The intention of the text is to reflect the following agreement Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication On TP#2: vivo and Xiaomi suggested a minor revision to clarify “if configured” only applies to DCI format 2_7. On TP#3: no strong objection reason. The wording can be slightly revised to match with indication of “1”. On TP#4: - the first two parts are not necessary as commented by [CATT, Samsung, Apple, ZTE, Ericsson]. - For the last part regarding reference point for validity duration determination, it can be discussed in Section 3. Given above, the TPs are merged as follows: [2RD] Proposal 1-1 (v1): Support the following TP for CR of TS 38.213. ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 if configured or and a DCI format 1_0 with CRC scrambled by P-RNTI includes a the TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE indicates that the UE keeps existing assumption of the availability or unavailability of associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## *** Unchanged text is omitted ***. |

### Issue 1-2: Clarification on default assumption

According to the contributions, default assumptions for TRS availability were discussed for the following cases:

- C1: before UE receives L1 availability indication after camping on a cell

- C2: the TRS resource configuration is changed by SI update procedure

- C3: During the system modification period and before the new SIBs become valid

Companies views about default assumptions include

- Alt1: UE assumes all TRS resource set(s) are unavailable

  - Intel(C1), Xiaomi (C1, C2), LG (C3)

- Alt2:  UE “may” or “can” assume all TRS resource set(s) are unavailable

  - Panasonic, InterDigital

The issue was discussed in RAN1#107bis-e meeting. The following conclusion based on Alt2 (with majority support in last meeting) is suggested by FL for continuous discussion in this meeting.

[1RD] Conclusion 1-2 (v1) If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE can assume the configured group(s) of TRS resource set(s) are unavailable. Note: During the modification period before the new SIBs become valid, stored TRS configurations and the associated availability indication still apply.

Please a) provide your views on whether or not to support Conclusion 1-2 (v1) below:

| Support | Companies |
| --- | --- |
| Yes | Nokia, CATT, Qualcomm, Apple, Samsung, vivo, Xiaomi, Sharp, intel, Huawei, HiSilicon, DOCOMO, CMCC, Panasonic(with revision), IDCC |
| No | MediaTek (no need) |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | Maybe “can assume are unvailable” ” cannot not assume to be available”? |
| CATT | Agree with Nokia. If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE can not assume that the configured group(s) of TRS resource set(s) are unavailable. |
| Nordic | CATT wording is OK |
| Qualcomm | We think the original wording is better than Nokia’s update. There is no need to preclude UE blind detection of the TRS although there is no such a requirement for UE either. |
| Xiaomi | In fact we are not sure what does “During the modification period before the new SIBs become valid” refers. Does it refer to the time duration that paging DCI has indicated the SI will be changed in the next modification period and the next modification period is not coming yet? |
| LGE | Before making a decision regarding default assumption on configuration and associated availability, we would like to share our understanding on this issue. To acquire the updated TRS configuration, UE need to decode SIB1 and SIB-X. In case that the proposed conclusion is accepted, it should be noted that different UE may have different version of TRS configuration since there is possibility that UE fails to decode SIB1 or SIB-X. Hence, different UE may have different understanding on L1 availability indication that received during a modification period where UE performs SI update procedure for the TRS configuration. Is it ok to allow such an inconsistent interpretation for TRS availability between UEs? If it is allowed, it seems like gNB shall support both TRS configurations during the modification period. This means that resource overhead due to the TRS transmission can be doubled |
| ZTE, Sanechips | Not sure why we need to repeat this discussion. I think the same intention is pretty clear based on the previous agreements. As to the SI update procedure, we think it is a common issue, which does not require special handling for TRS. |
| Intel | Support. Also fine with proposal from Nokia. |
| Huawei, HiSilicon | Agree with Qualcomm and that is the intention of original description. A bullet should be added to capture that there is no spec impact. |
| MediaTek | The following agreement already clarifies the UE assumption when network doesn’t indicate TRS is available. So we don’t think the repeated conclusion is needed. Note that it is also our understanding current specification doesn’t preclude UE to perform blind detection while it is network that cannot assume UE can always do the blind detection. Agreements: (RAN1#104-e) For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).. |
| Panasonic | Agree with Qualcomm the current main text is okay and no need to restrict possible UE blind detection. Regarding the note, we think it is better to be revised to “…can/may still apply.” |
| Ericsson1 | Given below agreement, can it be clarified why this conclusion is needed? Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). |
| Moderator1 | [Nokia, CATT, Nordic, Intel] suggested modification “the UE can not assume that the configured group(s) of TRS resource set(s) are unavailable.” But it’s not acceptable to [QC, HW, Panasonic]. The intention of the conclusion is a) to end the discussion on default assumption, and b) not to preclude UE blind detection of the TRS by implementation although there is no such a requirement for UE either. However, MTK and Ericsson pointed out that we have agreement about UE default assumption. No further conclusion is needed. So, we can drop the discussion. |

### Issue 1-3: Whether to restrict consistent TRS availability indication from PEI and paging PDCCH

Proposals about “whether/how to avoid inconsistent availability information in Paging PDCCH and PEI PDCCH” include the following alternatives:

- Alt1: A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle

  - ZTE

- Alt2: the UE ignores TRS availability indication in DCI format 1_0 with CRC scrambled by P-RNTI.

  - Xiaomi

- Alt3: The availability information can be inconsistent. No any UE ambiguity.

  - Sharp, Nokia

- Alt4: If UE detects inconsistent indication, this is treated as error case and no need to specify.

  - Panasonic

The issue was discussed in RAN1#107bis-e meeting. For the first round discussion in this meeting, two options from both sides are provided for continuous discussion.

- Option 1 is a conclusion based on Alt3 and Alt4 without any specific change.

- Option 2 is based on Alt1 and Alt2.

[1RD] Option 1: Conclusion 1-3 (v1) A UE can receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle. Option 2: Proposal 1-3 (v1) A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle. - If a UE receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle, the UE ignores TRS availability indication in paging PDCCH.

Please a) provide your views on which option to support for resolving issue 1-3 below:

| Support | Companies |
| --- | --- |
| Option 1: Conclusion 1-3 (v1) | Nokia, Samsung, LGE, Sharp, DOCOMO, MediaTek, Panasonic, Ericsson1 |
| Option 2: Proposal 1-3 (v1) | CATT, Nordic, Qualcomm (with update), vivo (with some modification), Xiaomi, ZTE,Sanechips (with update), intel (without sub-bullet), Huawei, HiSilicon(with update), CMCC |

and, b) provide additional comments, if any, below:

| Company | Comments |
| --- | --- |
| Nokia | Discussion for option 1 and 2 has relation to proposal 2-2b. |
| CATT | The UE behavior would be unspecified if they receive different TRS availability indication from DCI format 2_7 and 1_0 in the same DRX cycle. |
| Nordic | A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH received in PO associated with the PEI. - Note: If a UE receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle, the UE ignores TRS availability indication in paging PDCCH. |
| Qualcomm | The bullet “If a UE receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle, the UE ignores TRS availability indication in paging PDCCH” is unnecessary in proposal 2. Proposal 2 main body should be guaranteed by network. |
| Apple | Before we discuss the two options, the meaning of “inconsistent TRS availability indication” needs to be clarified first. Based on how availability indication is interpreted, we do not think there is any inconsistency between any two indications, because each one is interpreted independently. It would be helpful if an example of inconsistency can be provided, and why it is considered as inconsistency. |
| Samsung | Our understanding of “inconsistent TRS availability indication” is different bitmap value. Since we agreed that UE can receive new indication before expiration of validity duration for previous TRS availability indication in last meeting, we don’t see any issue for UE to receive PEI based availability indication and paging PDCCH based availability indication with different value. |
| Vivo | In principle, we agree with the intention of the main-bullet of Option 2. However, we do not see the need for the sub-bullet since the case provided in the sub-bullet has been addressed as an error case by the main-bullet. Additionally, from our understanding, PEI and its associated paging DCI may transmit in different paging cycles, hence we suggest the following revision. Option 2: Proposal 1-3 (revised by vivo) A UE does not expect to receive inconsistent TRS availability indication from PEI and its associated paging PDCCH within a default paging cycle. - If a UE receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle, the UE ignores TRS availability indication in paging PDCCH. |
| Xiaomi | We support Option2. Option1 has a very strong negative impact on UE power consumption, and it eliminate all the power saving gain of PEI. If inconsistent TRS availability indication from PEI and paging DCI is allowed, UE does not able to know whether the same or a different TRS availability indication has been indicated in paging DCI or not , so UE has to decode both PEI and paging DCI every DRX cycle. this is basically a huge waste of UE power and we currently see no benefit at all |
| LGE | We have similar view with Samsung. When “1” is indicated by the L1 signal, new validity duration can be started from the reference point, and existing validity durations that UE already has are not impacted. When “0” is indicated by the L1 signal, there is no change for the validity duration. Thus we do not see any issue due to the inconsistency of TRS availability indication between Paging PDDCH and the associated PEI. |
| Sharp | UE can interpret the availability of TRS resources when it receives both or either of two type indications and have no ambiguous regardless of the indications are consistent or inconsistent. we think the restriction is not necessary |
| ZTE, Sanechips | We agree with other companies that the sub-bullet in the option 2 is not needed. As to the association between paging DCI and PEI, we don’t think it is needed. |
| Huawei, HiSilicon | We are generally fine with the main bullet of option 2. However, we think the indication should be consistent not only within a default paging cycle and also within a validity duration. And we think the sub-bullet is not needed. For the case of inconsistent (which is actually an error case), it can be up to UE implementation. Option 2: Proposal 1-3 (v1) A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle a validity duration. - If a UE receive inconsistent TRS availability indication from PEI and paging PDCCH within a default paging cycle, the UE ignores TRS availability indication in paging PDCCH. Furthermore, if “0” is always ignored and “1” means “available”, can anybody explain in in which case the indication is inconsistent considering this value design? Although we are fine with option2 above, we are wondering whether we need to discuss this proposal in general. |
| DOCOMO | We have similar view with Samsung and LG. We don't think there is a issue with the inconsistency in the TRS availability indication. |
| MediaTek | Based on the current specification, the validity durations of TRS resources will only be prolonged. In our understanding, UE will follow the latest indication as the updated information for TRS validity duration even the latest indication doesn’t provide the same content as the previous one. To avoid ambiguity, we also suggest to change ‘inconsistent indication’ to ‘different indication content’ |
| CMCC | If Option 1 is adopted, the interpretation of TRS availability indication value ‘0’in paging DCI may be different between UEs supporting PEI and UEs not supporting PEI |
| Ericsson1 | Consistent/inconsistent terminology is not very clear. Since each DCI has its own reference point and validity time duration, we do not see need for linking reception/indication in one DCI with that of another DCI. We do not support Option 2. |
| Moderator1 | On Proposal 1-3, - [8] companies object it, as they don’t see issue to allow inconsistent or different indication content from PEI and paging PDCCH. - To follow up with Xiaomi’s comment, For UE configured with PEI, UE is not required to monitor paging PDCCH for TRS availability indication in every DRX cycle. Whether or not UE needs to decode paging PDCCH is indicated by PEI. So, moderator suggests to continue check conclusion 1-3 with the intention to end the discussion for this issue. Conclusion 1-3 is further updated to clarify “inconsistent”, and association between PEI and paging PDCCH. Conclusion 1-3 (v2) A UE can receive different inconsistent TRS availability indication content from PEI and associated paging PDCCH within a default paging cycle |

### Issue 1-4: Whether to support PEI based TRS availability indication only for RS resources with same QCL reference

[Spreadtrum, Sharp] proposed to support TRS availability indication based on PEI PDCCH in beam-selective manner. The issue was discussed in previous meetings. The majority consider it as non-essential based on the feedbacks in previous discussions.

In the first round discussion in this meeting, two options from both sides are provided for first round discussion. Option 1 is the latest proposal discussed in last meeting to support the additional design, and Option 2 is to conclude no support for the additional design and end of the discussion.

[1RD] Option 1: Proposal 1-4 (v1): Support to only include one indication bit in TRS availability indication field in DCI for-mat 2_7, which is associated with the same QCL reference (SSB index) of the monitoring occasion for the DCI format 2_7, in addition to the support of the same TRS availability indication field size as that in paging DCI. Option 2: Conclusion 1-4 (v1) No consensus to support - PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion

Please a) provide your views on which option to support for resolving issue 1-4 below:

| Support | Companies |
| --- | --- |
| Option 1: Proposal 1-4 (v1) | Apple, Sharp(with modification), Huawei, HiSilicon |
| Option 2: Conclusion 1-4 (v1) | CATT, Qualcomm, Samsung, vivo, Xiaomi,ZTE, Sanechips, intel, DOCOMO, MediaTek, CMCC, Panasonic, IDCC |

and, b) provide additional comments, if any, below:

| Company | Comments |
| --- | --- |
| Nokia | Option 1 is not clear if we have multiple resources sharing same QCL source, while the associated L1 availability ID/bit is different |
| Nordic | We have been supporting Option 1, but since we are in maintenance, Option 2 is the way to go. |
| Apple | We have always been supporting Option 1. We would certainly welcome if companies are willing to consider it. |
| Samsung | The additional design for PEI based indication is non-essential. |
| vivo | Not support to introduce new definitions on TRS availability field. The bit filed length in PEI DCI should be same as that in paging DCI to limit spec impact. |
| Xiaomi | Since we have agreed a more flexible way to indicate availability for both TRS with the same and different QCL,we don’t see it is essential to support Option1. |
| LGE | Our preference is option 1, but we can compromise to majority view. |
| Sharp | For Nokia’s question, we can modify the proposal with bits for each resources sharing same QCL: Support to only include one indication bit(s) in TRS availability indication field in DCI for-mat 2_7, which is associated with the same QCL reference (SSB index) of the monitoring occasion for the DCI format 2_7, in addition to the support of the same TRS availability indication field size as that in paging DCI. |
| Huawei, HiSilicon | We support Option 1 which provides technical benefits. |
| Ericsson1 | Not clear what has changed from the last time this was discussed (i.e. RAN1#107-e). |
| Moderator1 | The majority don’t think the additional design is essential. Since we are at the CR phase, moderator suggests to agree on Conclusion 1-4 to end the discussion for this issue. Conclusion 1-4 (v1) No consensus to support - PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion |

# TRS validity duration

The following were agreed regarding validity time for L1 based availability indication:

From RAN1#107-e: Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration

In contributions [1-20], proposals regarding the validity time for L1 based availability indication are captured in table below:

| Huawei, HiSilicon | - The description of “SFN of the first PF from the current default DRX cycle where UE receives the availability indication” in the agreement may cause different referent point of validity durations for different UEs sharing the same PO/PEI-occasion, and needs further clarification to avoid ambiguity between gNB and UE. - It violates the motivation to introduce validity duration of TRS availability if the ambiguity of the reference point caused by detected MOs on UE is left to gNB implementation handling. - It is beneficial to further reduce the maximum number of configurable TRS resource sets for FR1 to reduce UE memory size for low cost and power consumption, especially for RedCap UEs. - Clarify that “the current default DRX cycle where UE receives the availability indication” in RAN1#107 refers to the default DRX cycle that includes the first MO of a PO where UE receives the availability indication, at least for the case when L1 availability indication signaling is received in paging DCI. - Adopt text proposal 1 in section 2.1 ------------------------------------------ Start of Text Proposal 1------------------------------------------ < Unchanged parts are omitted > ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from (SFN+PF_offset)modT=0 [17, TS 38.304] that corresponds to the frame that includes the first MO of a paging occasion, where the paging occasion includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. < Unchanged parts are omitted > -------------------------------------------- End of Text Proposal 1-----------------------------------------. Proposal 3: Select one of the alternative to align the validity duration for PEI DCI and paging DCI in RAN1#108: - Alt.1: The validity duration(s) of L1 availability indication field in a PEI DCI refer to the validity duration(s) of L1 availability indication field in the associated paging occasions of the PEI DCI. - Alt.2: The validity duration(s) of L1 availability indication field in a PEI DCI refers to the validity duration starts from the SFN of default DRX cycle that includes the first MO of the PEI occasion where UE receives the availability indication. |
| --- | --- |
| ZTE, Sanechips | For PEI based availability indication, the reference time of the start of validity duration has no ambiguity issue according to current specification and observation. |
| Vivo | Observation 1: PO for a UE can be located in the first frame of a DRX cycle, and PEI is offset to PO in frame level, which lead to the frame for PEI may be in previous DRX cycle of the associated paging PDCCH. Proposal 1: For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI, i.e., UE assumes the L1 availability is detected in the associated paging DCI when determine the starting time of validity duration. |
| TCL |  |
| Spreadtrum |  |
| CATT | Proposal 2: The validity duration should be a number of paging cycles provided by defaultPagingCycle. Proposed TP for TS 38.213 #### 10.4B Indication of TRS resources ---------------------------------------------------- Unchanged text is Omitted --------------------------------------------------------- … The UE can be additionally provided a multiple, by validityDuration, for a number of paging cycles frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. ---------------------------------------------------- Unchanged text is Omitted ---------------------------------------------------------. Proposal 4: PEI based availability indication should have same reference point with that of its associated paging DCI. Proposed TP for TS 38.213 #### 10.4B Indication of TRS resources ---------------------------------------------------- Unchanged text is Omitted --------------------------------------------------------- A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, or its associated DCI format 2_7, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. ---------------------------------------------------- Unchanged text is Omitted ----------------------------------------------------------. |
| DOCOMO | Proposal 1: Regarding the reference point, Option1 or Option3 should be supported. |
| OPPO |  |
| Sony |  |
| Intel | Observation 1: There is no critical need to change the existing agreement regarding reference point definition. |
| Xiaomi |  |
| CMCC |  |
| Panasonic | Observation 1: In the case that PEI and associated POs locate in different paging cycle, if reference point is not aligned for the L1 availability indications in the PEI and paging DCI, below issues happen: - gNB may skip the PEI transmission, which is harmful for UE power saving gain. - Inconsistency between paging DCI and associated PEI based TRS availability indication may be required, which needs additional standardization efforts and potential specification impact. - The UEs whose POs are within the first POnumPerPEI POs may lose power saving gain due to not being able to utilize the TRS. Proposal 3: For both PEI and associated paging based TRS availability indication, the reference point for start of the validity duration is SFN of the first PF of the current default DRX cycle corresponding to the first MO of PO. |
| Samsung | Proposal 1: support the following TP#1 for TS 38.213. Proposed TP # 1 for TS 38.213 ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE.. |
| Apple |  |
| InterDigital |  |
| Sharp |  |
| LG |  |
| Ericsson | - For PEI based availability indication of TRS in TRS occasions, the reference point is the SFN of the first PF associated with the default paging cycle for which UE receives PEI. - Adopt TP1 related to 38.213 subclause indication of TRS resources (updated text in blue). <begin TP1> ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of TRS in associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame default DRX cycle that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. <end TP1> |
| Qualcomm | Observation 1: The validity duration of TRS availability indication with current agreement has the following inconsistency problems - TRS availability indication received on different beams may result in different validity durations for same TRS - UEs of different POs associated with same PEI-O may have different understanding of validity duration depending on whether the UE receives the availability indication from PEI or from paging PDCCH - If a UE receives both PEI and paging PDCCH in same PF, it may have to unnecessarily extend the validity duration by one default paging cycle Proposal 1: Clarify in the agreement that “the current default DRX cycle” is the one that overlaps with the DRX cycle logically associated with the UE’s PEI or PO. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Note: Qualcomm and Huawei have concern on Alt a. |
| MediaTek |  |
| Nokia |  |
| Nordic |  |

According to the above proposals, the remaining issues regarding validity duration for L1 based TRS availability indication include:

- Issue 2-1: TP for CRs of TS 38.213

- Issue 2-2: Clarifications on reference point

## 3.1 <1st round discussion>

### Issue 2-1: TP for CRs of TS 38.213

The proposed TPs for CRs of TS 38.213 regarding TRS validity duration based on existing agreements are collected in the table below.

| Index | Proposed TP | From |
| --- | --- | --- |
| 1 | #### 10.4B Indication of TRS resources ---------------------------------------------------- Unchanged text is Omitted --------------------------------------------------------- … The UE can be additionally provided a multiple, by validityDuration, for a number of paging cycles frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. ---------------------------------------------------- Unchanged text is Omitted --------------------------------------------------------- | CATT |
| 2 | ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE.. | Samsung |
| 3 | <begin TP1> ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of TRS in associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame default DRX cycle that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. <end TP1> | Ericsson |

[1RD]

Please a) provide your views on whether or not to support any of the TPs above.

| TP | Companies |
| --- | --- |
| #1 | - Yes: Nokia, CATT, Nokia, Qualcomm,Xiaomi, LGE,Sharp, CMCC - No: Samsung, vivo |
| #2 | - Yes: Qualcomm, Samsung, vivo (with some modification), intel - No:Nokia (see comment), CATT, Nordic, LGE, , Ericsson1(see comment) |
| #3 | - Yes: Qualcomm, ZTE, Sanechips, intel, Ericsson1, IDCC - No: Nokia (see comment), CATT, Nordic, vivo, LGE |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | Both #2 and #3 seem to have a partial overlap with TP#4 in Issue 1-1“which is the first frame within the default DRX cycle”. |
| Nordic | each PEI belongs to PO, and the default DRX cycle is the one where PO is. This is related also to PEI AI, where it is discussed that PEI should not be associated with PFs in different paging cycles. |
| Qualcomm | For #1, it should be “for a number of default paging cycles frames provided by defaultPagingCycle” #2 and #3 together are good. I.e., both “first” and “default” should be added to the spec text. |
| Apple | For #1, we are fine either way, and also fine with QC’s suggestion. #2 and #3 overlaps with the a previous TP, and it is better to discuss together. It may be good to have a consolidated TP that includes all the necessary clarifications. |
| Samsung | For #1, defaultPagingCycle indicates a number of radio frames according to 38.331 as below: defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on So, we think the original CR is correct. |
| vivo | The TP#2 is reasonable, and we try to add a ‘default’ in the following based on our agreement to avoid ambiguity. ## 10.4B Indication of TRS resources <Unchanged text is Omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the default DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. <Unchanged text is Omitted> |
| Xiaomi | TP #2/#3 should be discussed together with Issue 1-1. |
| LGE | For #1, we prefer QC’s revision. For #2, as we commented in Issue 1-1, we have concern on “within the default DRX cycle” |
| Sharp | TP#2/3 can be suspended waiting for the conclusion for other issues |
| ZTE, Sanechips | For #1, we are okay with or without the revisions. TP #2,3, and the TP is issue 1-1 and be discussed together. For TP #2,3, we think updating it as ‘the frame of the default DRX cycle’ as it is suggested in TP#3 is clear. The reference point of the frame is clearly clarified by the formula. |
| Intel | For TP # 1, we think it has relation to the description of TP # 1 in issue 1 -1, where validity duration is expressed in multiple number of frames. Hence, these TPs should be consistent in terms of definition and reference to validity duration. |
| Huawei, HiSilicon | For #1, we are fine with it. However, if we decides to change something, it should be “default paging cycles.”. For #2 and #3, in our understanding, the motivation of the two TPs are the same, i.e. change from ‘a frame including L1 indication’ to ‘a DRX cycle including L1 indication’. However, we think the reason why the description in the spec is not clear is that the agreement itself is not clear enough. Specifically, the word “where” in the agreement (“where UE receives the availability indication”) is not clearly defined. So we suggest to first discuss issue 2-2. |
| CMCC | TP#2 and TP#3 are related to the discussion of issue 2-2. |
| Ericsson1 | #1: defaultPagingCycle provides a number of frames. #2 : Given the agreement reflects default DRX cycle, using simply “DRX cycle” can become ambiguous. #3 : Support, below is the agreement from RAN1#107-e. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration |
| Moderator1 | On TP#1: - In current CR, “a number of frames provided by defaultPagingCycle “corresponds to the time unit of one default paging cycle. validityDuration is a multiple of default paging cycle. However, the TP indicates validityDuration is a multiple of a number of paging cycles, which seems to be wrong. On TP#2 and #3: - both “first” and “default paging cycle” are supported by the majority to clarify the reference point is the SFN of the first PF of the default paging cycle where UE receives the indication based on existing agreement. Given above, the TPs are merged as follows: [2RD] Proposal 1-1 (v1): Support the following TP for CR of TS 38.213. ## 10.4B Indication of TRS resources *** Unchanged text is omitted *** A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the default DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## *** Unchanged text is omitted ***. |

### Issue 2-2: Clarification on reference point

The proposals regarding clarification on reference point for validity duration can be summarized into the following options as discussed in last meeting. In this meeting, we resume the discussion.

[1RD] Option 1 Proposal 2-2a (v1) The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first PDCCH MO of the PO/PEI-O where UE receives the availability indication. Option 2 Proposal 2-2b (v1) For paging PDCCH based TRS availability indication, the reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first PDCCH MO of the PO/PEI-O where UE receives the availability indication. For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI Option 3 No change for the following agreement Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication

Please a) provide/update your views on which option you support for resolving issue 2-2 below:

| Support | Companies |
| --- | --- |
| Option 1 Proposal 2-2a (v1) | Huawei/HiSilicon, DOCOMO, QC, Nokia(alt), Apple (can accept), LGE, MediaTek |
| Option 2 Proposal 2-2b (v1) | Vivo (with some modification), CATT, Panasonic, Qualcomm (with modification), LGE, Ericsson1(with modification), IDCC |
| Option 3 | ZTE, DOCOMO, Intel, Nokia(alt), Apple, Samsung,Xiaomi, CMCC |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | Would be OK with option 1 or 3. |
| CATT | The abnormal condition is that large PEI frame offset might have the results of PEI in different DRX cycle to that of PO at DRX cycle N. However, PEI is defined as the association of PO. Even PEI is received in DRX cycle N-1 for the early indication of PO at DRX cycle N, the TRS availability received from PEI should also be assumed is for the DRX cycle N (not DRX cycle N-1). |
| Nordic | I am not the English speaker, but to me the wordings are still ambiguous. Option 1 should be “first frame is the first frame of a DRX cycle in which paging PDCCH is received Option 2 should be “first frame is the frame containing the first paging PDCCH MO of the default paging cycle in which paging PDCCH is received PEI location should not impact the reference frame. |
| Qualcomm | We made the following modification to proposal 2-2b for the same reasons as mentioned by CATT for paging PDCCH. I.e., even paging PDCCH is received within the duration of DRX cycle N+1 for PO associated with DRX cycle N, the TRS availability received from paging PDCCH should also be assumed is for the DRX cycle N (not DRX cycle N+1). Please note in TS 38.304, there is spec text “NOTE 1: A PO associated with a PF may start in the PF or after the PF.” Proposal 2-2b (v1) For paging PDCCH based TRS availability indication, the reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first PDCCH MO of the PO/PEI-O where UE receives the availability indication for which the UE receives the paging PDCCH. For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI |
| Apple | For PEI-based availability indication, we think it is critical that the availability indicaiton applies to the time duration between PEI and PO, so that the available TRS can be used for the current PO reception. |
| Samsung | Similar as Issue 1-2, since we agreed that UE can receive new indication before expiration of validity duration for previous TRS availability indication in last meeting, we think PEI based availability indication and paging PDCCH based availability indication can be processed independently by UE. No need to align the reference point for them. |
| vivo | We think there is some redundancy in the description of the current proposal 2-2b. Our original intention is to clarify the case that when TRS availability indication presents in both PEI DCI and paging DCI, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI. Hence, we think the first paragraph in proposal 2-2b belongs to option-1, and we suggest to discuss these two issues separately. Option 2 Proposal 2-2b (revised by vivo) When TRS availability indication presents in both PEI DCI and paging DCI, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI. |
| Xiaomi | Current agreement can work, no need to revert previous agreement. |
| LGE | We are fine with either Option 1 or Option 2. |
| ZTE, Sanechips | According to the current agreement/CR, there is no ambiguity of reference point for the TRS indication between gNB and UE . |
| Intel | Current agreement works, so no need to change it. |
| Huawei, HiSilicon | Our first preference is Option 1. As we explained under Issue 2-1, the previous agreement is not clear enough. It should be clarified. We understand the motivation of the second bullet of proposal 2-2b (v1) is to make the L1 indication more useful in some cases. Though it does not resolve all the problematic scenarios, the reasonable enhancement is acceptable for us. If it is hard to get the companies’ views converged, we suggest the moderator to discuss the reference point for paging DCI and PEI separately. According to our observation, majority view on paging DCI is easier to be converged. |
| Ericsson1 | Support revised proposal from Qualcomm. For example, it could so happen that PEI-O is in an SFN between last PF of DRX cycle N and first PF of DRX cycle N+1 – then a bit unclear which cycle it belongs to. NW may be able to handle such cases (e.g. consider both N/N+1 assumption for reference point when indicating availability, or simply set the bitmap to all zeros), however reducing ambiguity could be beneficial for UE. |
| IDCC | Support revised proposal from Qualcomm. |
| Moderator1 | One the first debating point is about the default DRX cycle used to determine the reference point - Alt1: the default DRX cycle is associated with the PDCCH MO whether UE receives the availability indication, i.e. previous agreement. - Alt2: the default DRX cycle is associated with the first PDCCH MO of the PO/PEI-O where UE receives the availability indication. Another debating point is whether or not to support different reference point for PEI: - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated PO. For the second debating point, the majority (Option 1 + Option 3) don’t support the new design of reference point for PEI based availability indication. Also, Apple points out the new design precludes the use case, where PEI indicates TRS resources that can be used for associated PO reception. Since current design still work for PEI, moderator suggests to conclude no consensus to support the new design, and end the discussion. Conclusion 2-2(v1) No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI. For the first debating point, the majority (Option 1 + Option 2) prefer Alt2. However, several companies think current spec works fine without ambiguity of reference point between gNB and UE. So, moderator suggests to further check the issue of current spec before modifying the existing agreement. Proposal 2-2 (v2) The current default DRX cycle used for determining the reference point for start of validity duration for an availability indication is down-selected from the following in RAN1#108-e: - Alt1: the current default DRX cycle is associated with the PDCCH MO whether UE receives the availability indication - Alt2: the current default DRX cycle is associated with the first PDCCH MO of the PO/PEI-O where UE receives the availability indication. |

# TRS resources configuration

The following were agreed in RAN#107bis-e regarding configuration of TRS resources for idle/inactive UEs:

From RAN1#107bis-e: Agreement Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is based on configburation parameter with applicable value {2, 4} Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS Agreement The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set.

In contributions [1-20], proposals regarding the TRS resources configuration are captured in table below:

| Huawei, HiSilicon | Observation 3: It is beneficial to further reduce the maximum number of configurable TRS resource sets for FR1 to reduce UE memory size for low cost and power consumption, especially for RedCap UEs. Proposal 5: Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1 |
| --- | --- |
| ZTE, Sanechips | The number of configured TRS resource sets can be larger than the number of actual transmitted SSB. - Proposal 4: The following TP for CR of TS 38.214 should be adopted. ...................... Text Proposal for TS38.214.............................................................................. 5.1.6.1.1 CSI-RS for tracking *** Unchanged text is omitted *** - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. *** Unchanged text is omitted *** .................... End of Text Proposal for TS38.214....................................................................... |
| Vivo |  |
| TCL |  |
| Spreadtrum | Proposal 2: A dedicated SIBx can be used for TRS reconfiguration. |
| CATT | Proposal 5: The following TRS resource set configuration parameters: startingRB, nrofRBs, periodicity and TRS resources number could be common for all the TRS resources sets. |
| DOCOMO | Proposal 2: The common configuration parameter among all TRS resource sets should be supported to reduce the SIB overhead for TRS/CSI-RS for idle/inactive mode UE. |
| OPPO |  |
| Sony |  |
| Intel | Proposal 2: The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. |
| Xiaomi |  |
| CMCC |  |
| Panasonic | Proposal 2: The maximum number of configured TRS resource sets is 64 and no need to limit by the number of actual transmitted SSBs in ssb-PositionsInBurst. |
| Samsung |  |
| Apple | Proposal 1: The maximum number of configure TRS resource sets is 16 for FR1. Proposal 2: Introduce common parameters startingRB and nrofRBs for all configured TRS resource sets to reduce the signaling overhead. - Note: startingRB and nrofRBs can still be separately configured per TRS resource set if needed. |
| InterDigital | Conclusion 1: The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. |
| Sharp | Proposal 1: Adopt TP1 for TS 38.214 ---------------------<begin TP1 for TS 38.214>-------------------------------------------------------------------- 5.1.6.1.1 CSI-RS for tracking <omit unchanged text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a TRS-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more NZP CSI-RS set(s), where a TRS-ResourceSet NZP CSI-RS set consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources.The UE does not expect to receive a configuration with four resources when the TRS symbols in the second slot are indicated as uplink by tdd-UL-DLConfigurationCommon. - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. -----------------<end TP1>------------------------------------------------------------------------------- |
| LG |  |
| Ericsson | - The following parameters are configured as ‘optional’ for one or more TRS resource set(s). - startingRB’, nrofRBs’, frequencyDomainAllocation, ’powerControlOffsetSS’, ‘scramblingID’, firstOFDMSymbolInTimeDomain - For a TRS resource set, when the parameter value is explicitly configured, follow the configured parameter value, otherwise follow the parameter value from the first configured TRS resource set. - Adopt TP2 related to 38.214 subclause CSI-RS for Tracking. <begin TP2> ##### 5.1.6.1.1 CSI-RS for tracking <omitted unchanged text> Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: <omitted unchanged text> - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. <omitted unchanged text> <end TP2> |
| Qualcomm |  |
| MediaTek |  |
| Nokia | Observation: RAN1#104bis-e agreement on initial BWP not restrictin TRS-ResourceSet configuration is not correctly captured in TS38.214 Proposal: Adopt following TP to 38.214 Section 5.1.6.1.1: ##### 5.1.6.1.1 CSI-RS for tracking [text omitted] - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.. |
| Nordic | Proposal-1: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64. |
| Lenovo |  |

According to the above proposals, the remaining issues regarding TRS resources configuration include:

- Issue 3-1: TP for CR of TS 38.214

- Issue 3-2: Maximum number of TRS resource sets

- Issue 3-3: Whether/how to reduce configuration overhead

## 4.1 <1st round discussion>

### Issue 3-1: TP for CR of TS 38.214

The proposed TPs for CRs of TS 38.214 regarding configuration of TRS resources based on existing agreements are collected in the table below.

| Index | Proposed TP | From |
| --- | --- | --- |
| 1 | ...................... Text Proposal for TS38.214.............................................................................. 5.1.6.1.1 CSI-RS for tracking *** Unchanged text is omitted *** - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. *** Unchanged text is omitted *** .................... End of Text Proposal for TS38.214....................................................................... | ZTE |
| 2 | ---------------------<begin TP1 for TS 38.214>-------------------------------------------------------------------- 5.1.6.1.1 CSI-RS for tracking <omit unchanged text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a TRS-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more NZP CSI-RS set(s), where a TRS-ResourceSet NZP CSI-RS set consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources.The UE does not expect to receive a configuration with four resources when the TRS symbols in the second slot are indicated as uplink by tdd-UL-DLConfigurationCommon. - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. -----------------<end TP1>------------------------------------------------------------------------------- | Sharp |
| 3 | ##### 5.1.6.1.1 CSI-RS for tracking <omitted unchanged text> Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: <omitted unchanged text> - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. <omitted unchanged text> <end TP2> | Ericsson |
| 4 | ##### 5.1.6.1.1 CSI-RS for tracking [text omitted] - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. | Nokia |

[1RD]

Please a) provide your views on whether or not to support any of the TPs above.

| TP | Companies |
| --- | --- |
| #1 | - Yes: Nokia, Nordic, Qualcomm, Apple, Samsung,Xiaomi, LGE, ZTE,Sanechips, intel, Huawei, HiSilicon, MediaTek, CMCC, Ericsson1, IDCC - No: CATT |
| #2 | - Yes: Qualcomm, Apple (2nd part) ,Xiaomi,Sharp - No: Nokia (please see comment), CATT, Nordic, Ericsson1 |
| #3 | - Yes: Nokia, Nordic, Qualcomm, Apple, Samsung,Xiaomi, LGE,Sharp, ZTE,Sanechips, intel, Huawei, HiSilicon, MediaTek, CMCC, Ericsson1, IDCC - No: |
| #4 | - Yes: Nokia (proponent), CATT, Nordic, Qualcomm (with modification), Apple (support QC’s modification), Samsung,Xiaomi, LGE, ZTE,Sanechips, intel, Huawei, HiSilicon, MediaTek(support QC’s modification), CMCC, Ericsson1, IDCC - No: |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | For #2, - the text seems to be aligned with earlier text for CONNECTED mode: “with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.” - For the second change proposal for the UL, need to check if this is already captured somewhere. |
| Qualcomm | Update the text of #4 to: Bandwidth of the NZP CSI-RS resource configured by TRS-ResourceSet is not restricted by initial DL BWP. |
| Huawei, HiSilicon | For #2, we think we should use ‘TRS resource set(s)’ to replace “NZP CSI-RS set(s)” instead. |
| Ericsson1 | #2: First change is not needed since TRS resource set is correct usage. 2nd change : similar comment as Nokia. |
| Moderator1 | For TP#2, several companies think the following aligned with connected mode, should be already captured by spec. - The UE does not expect to receive a configuration with four resources when the TRS symbols in the second slot are indicated as uplink by tdd-UL-DLConfigurationCommon. No objection for TP#1, TP#3 and TP#4 with justification. So the TP#1, #3, #4 are merged for further check. ##### 5.1.6.1.1 CSI-RS for tracking <omitted text> A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a TRS-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more NZP CSI-RS set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. <omitted text>. |

### Issue 3-2: Maximum number of TRS resource sets

Proposals on “whether to reduce maximum number of TRS resources sets” include the following alternatives:

- Alt 1: The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

- ZTE, Intel, Panasonic, InterDigital

- Alt 2: The maximum number of configure TRS resource sets is 16 for FR1

- Apple

- Alt 3: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

- Nordic

In the first round, FL suggests to check all the alternatives.

[1RD] Option 1: Conclusion 3-1 (v1) The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. Option 2: Proposal 3-1a(v1) The maximum number of configure TRS resource sets is 16 for FR1 Proposal 3-1b(v1) The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

Please a) provide your views on which option to support for resolving issue 3-1 below:

| Support | Companies |
| --- | --- |
| Option 1 Conclusion 3-1 (v1) | vivo,Xiaomi, Sharp, ZTE,Sanechips, Panasonic |
| Option 2 Proposal 3-1a (v1) | Xiaomi, Huawei, HiSilicon |
| Option 2 Proposal 3-1b (v1) |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | Don’t support any of the proposals. There are no requirements for the UE in relation to the said TRS occasions, thus it would not seem appropriate restrict the configuration either. |
| CATT | We don’t need further agrements |
| Nordic | As commented in our contribution - UE must be able to comprehend all 64 configurations in order to pick the best TRS - given that TRS availability is dynamic, the best TRS may be made unavailable, and UE may want to go for 2nd best. Nokia says UE can pick subset of 16, but how should UE know at the time of configuration which TRS resource sets gNB will make available? We can compromise for any of above proposals. |
| Apple | We need to be reasonable when we set the upper bound. Having an unrealistic upper bound only increases UE complexity without any gain. We would appreciate if any company can provide a realistic use case of 64 TRS configurations for FR1, instead of just saying any further discussion is not needed. With this said, we would be fine with any proposals that address the concern to some extent. |
| LGE | We don’t need further discussion. |
| Huawei, HiSilicon | Our proposal is categorized to Issue 3-3. Actually we have the same proposal with Apple in our Tdoc. Restricting the maximum number of TRS resource sets to 16 is beneficial for reducing the memory requirement for UE. |
| Panasonic | We are also okay with no consensus. |
| Ericsson1 | Do not see need for any of the proposals. |
| Moderator1 | The majority don’t support either P3-1a or P3-1b. To end the discussion, the following conclusion is provided for further check. Conclusion 3-2 (v2): No consensus to support: - Reduced maximum number of configured TRS resource sets for FR1. |

### Issue 3-3: Whether/how to reduce configuration overhead

The following were proposed to reduce TRS resources configuration overhead in SIBx:

P1: Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1

- HW

P2: The common configuration parameter among all TRS resource sets should be supported to reduce the SIB overhead for TRS/CSI-RS for idle/inactive mode UE.

- DOCOMO

- CATT: startingRB, nrofRBs, periodicity and TRS resources number

- Apple: startingRB and nrofRBs

- Ericsson1 : startingRB’, nrofRBs’, frequencyDomainAllocation, ’powerControlOffsetSS’, ‘scramblingID’, firstOFDMSymbolInTimeDomain

[1RD]

Please a) Provide your views on whether to support or further discuss the proposals above:

| Proposals | Companies |
| --- | --- |
| P1 | - Yes: Nordic, Apple,Xiaomi, Huawei, HiSilicon - No: Nokia, LGE, Ericsson1 |
| P2 | - Yes: CATT, Nordic, Apple,Sharp, DOCOMO, CMCC, Panasonic, Ericsson1 - No: |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | Neither of the proposed solutions limits the total/max configuration overhead. P1 is a configuration issue and it is up to network to choose the number of resources. As I read P2, it would still allow configuring e.g. startingRB also per TRS resource set, thus it would offer opportunity to reduce the overhead if configuration allows. |
| Nordic | Any of parameters could be common for all TRS resource sets. If configured common, it should not be configured in the TRS resource set. |
| Apple | P1 should be discussed in issue 3-1, right? |
| Samsung | In general, we don’t see any need to continue RRC related design. Whether or how to reduce configuration overhead can be up to RAN2 decision. |
| Huawei, HiSilicon | P1 can be discussed under Issue 3-2. |
| Ericsson1 | RAN2 cannot judge without RAN1 input which parameters could be made common across TRS resource sets to reduce SIB overhead (while still allowing the flexibility to allow different values per TRS resource set as per current framework). |
| Moderator1 | [8] companies support P2 for configuration overhead reduction. Since no objection, a proposal is drafted accordingly for further discussion in next round. Proposal 3-3 (v1) Support common configuration parameter(s) among all TRS resource sets for idle/inactive mode UE - The common configuration parameters include startingRB, nrofRBs, frequencyDomainAllocation, powerControlOffsetSS, scramblingID, firstOFDMSymbolInTimeDomain, periodicity, numberOfresources |

# Others

In addition to the main topics in Section 2-4, some other issues or design aspects have been discussed by a few companies, and the corresponding proposals are captured below.

| HW | Proposal 2: Adopt the following TP2 not to impact connected mode UE’s rate matching behavior due to TRS resource set(s) configured in SIB. ------------------------------------------ Start of Text Proposal 2------------------------------------------ < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured via [TRS-ResourceSetConfig]; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. ------------------------------------------ End of Text Proposal 2------------------------------------------ < Unchanged parts are omitted >. Proposal 4: Conclude that it is RAN1 understanding that Idle/inactive mode UE is not required to monitor all the TRS resource set(s) configured in SIBx by gNB. Proposal 7: Conclude that it is up to UE implementation whether to utilize TRS resources for time/frequency tracking configured in SIB when UE is in CONNECTED mode. |
| --- | --- |
| CMCC | Proposal 1. TRS availability indication field is treated as scheduling information for Paging when interpreting the Short Message indicator field in DCI format 1_0 with P-RNTI. The TP suggestion for TS 38.212 section 7.3.1.2.1 is as the following: <Unchanged text is omitted> Table 7.3.1.2.1-1: Short Message indicator Bit field; Short Message indicator. 00; Reserved. 01; Only scheduling information for Paging and TRS availability indication (if any) is present in the DCI. 10; Only short message is present in the DCI. 11; Both scheduling information for Paging, TRS availability indication (if any) and short message are present in the DCI. <Unchanged text is omitted> |
| MediaTek | Observation 1: For TRS availability indication by PEI for upcoming PO in current paging cycle, a proper time gap should be accommodated for UE to warm up for TRS processing. Observation 2: Connected mode UE can report requirement of a 3-ms time gap between DCP and the start of the associated DRX on-duration. It is reasonable a longer time gap is reserved for idle/inactive mode UE to realize lower power consumption. Proposal 1: For PEI-based TRS availability indication, an application delay of [5]-ms is introduced. Proposal 2: Adopt the following text proposal to Section 10.4B of TS 38.213: <Unchanged parts are omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. The UE is not expected to receive the indicated TRS resources set within application delay of 5 ms after TRS availability indication provided by DCI format 2_7. <Unchanged parts are omitted>. |
| CATT |  |
| Sharp | Proposal 4: Support zero-power PDSCH punching for REs overlapping with TRS resources configured in SIB |
| LG | Observation 1: According to the section 5.1.6.1 in the TS 38.214, idle/inactive UE can assume that it is not expected to receive TRS and SIB1 message in the overlapping PRB in the OFDM symbols where SIB1 is transmitted. Observation 2: Rel-15/16 UE in idle/inactive state does not have prior information on TRS. Therefore, the Rel-15/16 UE is not able to deal with the overlap between indicated TRS and broadcast PDSCH which occurs in the idle/inactive state. Observation 3: Rel-17 UE in idle/inactive state can have prior information on TRS. Therefore, it is necessary to deal with the overlap between indicated TRS and broadcast PDSCH for the Rel-17 idle/inactive UEs. Proposal 1: For REs that are configured for a TRS occasion(s) for idle/inactive UEs and indicated actual TRS transmission via L1 signaling - The UE expects TRS transmission in the REs even if they are overlapped with scheduled PDSCH - The UE expects PDSCH REs, which overlap with actual TRS transmission, are punctured after PDSCH RE mapping (i.e. zero power PDSCH RE) Proposal 3: Consider following options for the TRS configuration validation and associated availabilities for the eDRX UE: - Opt 1: UE shall monitor SIB1 to check the value tag of SI for TRS configuration before the PTW every eDRX cycles - Opt 2: UE cannot assume any validity duration before it monitors the first PO within a PTW - Opt 3: L1 signal (i.e. paging DCI and PEI) conveys bit field for indicating SI update for TRS configuration |

## 5.1 <1st round discussion>

### Issue 4-1: Others

Proposals for other issues with need of spec change are collected in the table below.

| Index | Proposals | From |
| --- | --- | --- |
| 1 | Proposal: Adopt the following TP2 not to impact connected mode UE’s rate matching behavior due to TRS resource set(s) configured in SIB. ------------------------------------------ Start of Text Proposal 2----------------------------------------- < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured via [TRS-ResourceSetConfig]; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. ------------------------------------------ End of Text Proposal 2------------------------------------------ < Unchanged parts are omitted > | HW |
| 2 | Proposal: TRS availability indication field is treated as scheduling information for Paging when interpreting the Short Message indicator field in DCI format 1_0 with P-RNTI. The TP suggestion for TS 38.212 section 7.3.1.2.1 is as the following: <Unchanged text is omitted> Table 7.3.1.2.1-1: Short Message indicator Bit field; Short Message indicator. 00; Reserved. 01; Only scheduling information for Paging and TRS availability indication (if any) is present in the DCI. 10; Only short message is present in the DCI. 11; Both scheduling information for Paging, TRS availability indication (if any) and short message are present in the DCI. <Unchanged text is omitted> | CMCC |
| 3 | Proposal: The UE is not expected to receive the indicated TRS resources set within application delay of 5 ms after TRS availability indication provided by DCI format 2_7. | MTK |
| 4 | Proposal: For REs that are configured for a TRS occasion(s) for idle/inactive UEs and indicated actual TRS transmission via L1 signaling - The UE expects TRS transmission in the REs even if they are overlapped with scheduled PDSCH - The UE expects PDSCH REs, which overlap with actual TRS transmission, are punctured after PDSCH RE mapping (i.e. zero power PDSCH RE) | LG, sharp |
| 5 | Proposal: Consider following options for the TRS configuration validation and associated availabilities for the eDRX UE: - Opt 1: UE shall monitor SIB1 to check the value tag of SI for TRS configuration before the PTW every eDRX cycles - Opt 2: UE cannot assume any validity duration before it monitors the first PO within a PTW - Opt 3: L1 signal (i.e. paging DCI and PEI) conveys bit field for indicating SI update for TRS configuration | LG |

[1RD]

Please a) provide your views on whether or not to support any of proposals above.

| Proposals | Companies |
| --- | --- |
| #1 | - Yes: Nokia, Nordic, Qualcomm (with modification), vivo, LGE, Huawei, HiSilicon, CMCC - No: CATT, intel, Ericsson1 |
| #2 | - Yes: Nokia, Qualcomm (with modification), Huawei, HiSilicon(needs further modification), CMCC - No: CATT, Nordic, Apple, LGE, intel |
| #3 | - Yes: MediaTek - No: Nokia, CATT, Nordic, Qualcomm, Apple, LGE, ZTE,Sanechips, intel, CMCC, Ericsson1 |
| #4 | - Yes: LGE,Sharp - No: Nokia, CATT, Nordic, Qualcomm, Apple, ZTE,Sanechips, intel, Huawei, HiSilicon, Ericsson1 |
| #5 | - Yes: LGE - No: Nokia, CATT, Nordic, Qualcomm,, ZTE,Sanechips, intel, CMCC, Ericsson1 |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia | For #4, to IDLE/Inactive mode UEs, this would be different behavior than for legacy i.e. for paging message no NZP-CSI related rate matching is assumed. For #5, not sure if additional changes are needed. I.e. SI update could have also been related to paging configuration, thus SI change procedure should account the eDRX. |
| Qualcomm | For #1, it is necessary to make similar clarification to that for measurement object. It should be clarified that this only applies to - if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured via [TRS-ResourceSetConfig] in SIBx (for idle and inactive mode UE); For #2, we think the proposal should be updated to “restrict TRS availability indication in paging PDCCH to only when short message indicator is not ‘00’” This is to avoid legacy UE that does not support idle/inactive TRS unnecessarily decoding paging PDCCH. It is reasonable to assume that if no UE is paged, there is no need for any UE to receive additional TRS for paging PDSCH decoding or short message decoding. Since UE can use TRS for short message reception, we think CMCC proposal should be extended to also indicate TRS availability when short message is to be received. For #3, the application delay is functionally unnecessary but will force UE to do additional handling of the delay gap. For #4, the issue either does not exist or has been handled by network already because the TRS is a configured to some connected mode UE already. No additional clarification or design is needed. For #5, RAN1 is not officially aware of eDRX. RAN1 should require RAN2 to judge whether specific design is needed. |
| Apple | For #1, we think the main clarification point is whether connected UEs even need to monitor these TRS configurations or not. If not, then we do not need to worry about it. For #2, it is not clear why this is necessary. TRS availability indication can be present in any paging DCI message. For #3, it is not necessary. If needed, UE can apply application delay by itself, but there is no need to specify. For #4, we agree with the 1st sub-bullet in principle but we don’t see the need to agree on anything. The 2nd sub-bullet is new UE behavior, and such scenario should be avoided by the network. For #5, it is not entirely clear to us the impact of eDRX. This needs further discussion and/or guidance from RAN2. |
| LGE | For #4, our point is to clarify the UE assumption when overlap is occurred. It is obvious that UE could not receive PDSCH data and TRS element simultaneously at the same RE, so UE assumption when overlap is occurred shall be clearly described in the spec. So far, this issue has not been discussed enough due to the priority. But we would like to emphasize again that, 1) overlaps between reference signals and other physical signals/channels should be handled by the spec to avoid ambiguity problem and 2) overlap between broadcast PDSCH and TRS resource would not be avoided by the network when PO is densely deployed. For #5, recall that in issue 1-2, it is common understanding that the validity duration associated with the previous TRS configuration would be expired when TRS configuration is changed. Also, according to the agreement in a previous RAN1 meeting, length of the validity duration could be configured up to 22 minutes. Let’s consider the case that configured eDRX cycle is shorter than the configured validity duration, and TRS configuration has been changed after a PTW where UE received the L1 availability indication. In general, UE cannot know whether the SI has been updated before it checks short message in paging DCI, SIB1, or SIB-X for TRS configuration, and so far we have been assumed that TRS can be used for time/frequency synchronization before PO. Our suggestion is to avoid undesired UE behavior that eDRX UE uses unavailable TRS resources for t/f synchronization. |
| Sharp | For #4, we think the second bullet is necessary to provide TRS signal without interference for idle/inactive UEs |
| ZTE,Sanechips | For issue #2, if the indication needs to be carried by short message indicator, “00” is better. For issue#1, we tend to agree with Apple that we need to first discuss whether RRC_connected UE read SIB for TRS. |
| intel | Agree with Apple comment above for # 1 |
| Huawei, HiSilicon | For#1, the intention is to avoid rate matching of the TRS resource set configured in SIB. It should be captured somewhere. Regarding apple’s and intel’s comments, we think whether connected mode UE can use TRS occasions of IDLE mode UE is UE implementation. Regardless whether connected UE can use it, it should be clear on rate matching behavior of connected mode UEs. For#2, we agree that except the reserved state, TRS availability field can be in paging DCI. So, TRS availability should be also added for “01” bit field. If we do this change, it seems the description in 212 needs to be also updated. For#3, we are flexible on it. For#4, it is gNB implementation. For#5, we can wait RAN2 for more details for eDRX. |
| CMCC | For #2, From our perspective, we think the TRS availability indication should be treated as scheduling information for Paging, because the TRS availability indication field uses the reserved bits in DCI format 1_0 not the short message field. For #4, we are ok to discuss it. |
| Ericsson1 | #1 : proposal is not needed i.e. even without the TP, spec seems to be clear. #2 : It seems that in all cases, the TRS availability bitfield would be present in the DCI and hence could be used for indicating availability. |

# Proposals for GTW handling

## 7.1 <GTW on 02/23>

The following proposals are suggested for GTW handling on Feb, 23th, Wednesday.

Proposal 1-1 (v1): Support the following TP for CR of TS 38.213. ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 if configured or and a DCI format 1_0 with CRC scrambled by P-RNTI includes a the TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE indicates that the UE keeps existing assumption of the availability or unavailability of associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## *** Unchanged text is omitted *** Proposal 3-1 (v1): Support the following TP for CR of TS 38.214. ##### 5.1.6.1.1 CSI-RS for tracking <omitted text> Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. <omitted text> Conclusion 1-4 (v1) No consensus to support - PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion Conclusion 3-2 (v2): No consensus to support: - Reduced maximum number of configured TRS resource sets for FR1. Conclusion 2-2(v1) No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI. Proposal 2-2 (v2) The current default DRX cycle used for determining the reference point for start of validity duration for an availability indication is down-selected from the following in RAN1#108-e: - Alt1: the current default DRX cycle is associated with the PDCCH MO where UE receives the availability indication, i.e. previous agreement. - Alt2: the current default DRX cycle is associated with the first PDCCH MO of the PO/PEI-O where UE receives the availability indication. Proposal 2-1 (v1): Support the following TP for CR of TS 38.213. ## 10.4B Indication of TRS resources *** Unchanged text is omitted *** A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the default DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## *** Unchanged text is omitted *** Conclusion 1-3 (v2) A UE can receive different TRS availability indication content from PEI and associated paging PDCCH.

# Conclusion

The following agreements were made in this meeting.

# References

[1]R1-2200945	Remaining issue on assistance TRS occasions for IDLE/inactive mode	Huawei, HiSilicon

[2]R1-2201102	Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

[3]R1-2201131	Remaining issues of TRS for RRC idle and inactive UEs	ZTE, Sanechips

[4]R1-2201281	Further discussion on RS occasion for idle/inactive UEs	OPPO

[5]R1-2201371	Remaining issues of TRS/CSI-RS for paging enhancement	CATT

[6]R1-2201485	Discussion on TRS/CSI-RS occasion for idle/inactive Ues	NTT DOCOMO, INC.

[7]R1-2201552	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

[8]R1-2201642	Remaining issues on enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs	Panasonic

[9]R1-2201706	Discussion on the remaining details of TRS/CSI-RS occasions in idle/inactive time	Intel Corporation

[10]R1-2201778	Remaining issues on indication of TRS configurations for idle/inactive UEs	Apple

[11]R1-2201866	Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs	CMCC

[12]R1-2201918	Remaining issues on TRS/CSI-RS configuration and indication for idle/inactive UEs	Xiaomi

[13]R1-2202023	Maintenance on TRS/CSI-RS occasion(s) for idle/inactive UEs	Samsung

[13]R1-2202069	Maintenance on TRS/CSI-RS Information for idle/inactive UEs	MediaTek Inc.

[14]R1-2202149	Remaining issues on idle and inactive TRS	Qualcomm Incorporated

[15]R1-2202195	Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs	Sharp

[16]R1-2202217	Maintenance for TRS occasion provisioning for Idle/Inactive UEs	Ericsson

[17]R1-2202248	Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs	InterDigital, Inc.

[18]R1-2202329	Open issues on n TRS information forto IDLE/INACTIVE mode UEs	Nokia, Nokia Shanghai Bell

[19]R1-2202347	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

[20]R1-2202385	On TRS design for idle/inactive UEs	Nordic Semiconductor ASA

# Appendix: Previous Agreements

## RAN1#102-e

Agreements: - New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE. Agreements: The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. - Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share). - Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. - Note: Always-on TRS/CSI-RS transmission by gNodeB is not required - At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported - FFS for other RS types - FFS: Whether UE blind detection is required or not. Agreements: Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as: - AGC, time/frequency tracking - FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication Observation: It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.) Agreements: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling - FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.) - FFS for other signalling candidates (e.g., pre-configuration, etc.) - FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.) Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreement: - Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s). Agreements: - SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s). - Up to RAN2 to decide which SIB is to be used. - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2 Send an LS to RAN2 informing the above agreements, and - To further add that RAN1 is working on the detailed physical layer design Agreement: - Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs. Agreements: - Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting) - Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence Agreements: - Discuss further based on the following alternatives and down-select at RAN1#104-e: - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE. - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - The condition can be, e.g., existence of paging. - Alt 4. Combination of the above alternatives. - FFS for details - FFS for UE behavior when the availability is not informed. - Other techniques are not precluded. - Companies encourage to provide sufficient information for the proposal, e.g., - how to achieve power saving gain - how to minimize impact on NW how to minimize extra UE implementation complexity - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs - Proposals should be consistent with the WID objective. Conclusion: - TRS/CSI-RS based PEI is discussed in AI 8.7.1.1. - PEI functionality is not further discussed under AI 8.7.1.2. - Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Update on 1/28 email: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#105-e: - Alt1: same as initial BWP - Alt2: configurable parameter Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only) Update on 1/31: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). Conclusion From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs. Agreements: The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e: - Alt-1: within initial DL BWP - Alt-2: is not restricted by initial BWP - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. Agreements: To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives: - Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS - Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO - FFS details - FFS details - Other alternatives are not precluded Conclusion: Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104b-e

Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling To further check on 4/19 Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded

## RAN1#105-e

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

## RAN1#106-e

Agreement Support at least one of the following alternatives - Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion. - Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion Note: The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication. Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only. Agreement L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where - the time duration can be determined based on at least one from the following (to be down-selected): - Alt-1: configured by higher layer - Alt-2: a predefined/configured window - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s) - Alt-4: until when the UE receives another availability indication - A combination of alternatives or other alternatives is not precluded. - the reference point can be determined as at least one from the following (to be down-selected): - Alt-1: start of next PO or DRX cycle - Alt-2: time location where UE receives the indication - Note: the time location is subject to application delay if agreed - Alt-3: start of current PO or DRX cycle where UE receive the indication - Alt-4: a time location which is configured by higher layer - A combination of alternatives or other alternatives is not precluded. Agreement For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

## RAN1#106bis-e

Conclusion No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available. - support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set - Bitmap size is up to X bits - X = [6] for paging PDCCH based L1 availability indication. - FFS X for PEI DCI based L1 availability indication - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured) - for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources - FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion - FFS: indication of unavailability Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling Agreement Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:  a TRS resource set can be configured to include o a set of TRS resources up to two consecutive slots,  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol. o at least common configuration parameters:  a QCL reference  firstOFDMSymbolInTimeDomain,  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’  FFS  scramblingID,  a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported  Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

## RAN1#107-e

Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Note: Qualcomm and Huawei have concern on Alt a Agreement For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support - time unit is one default paging cycle, - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s): Agreement Confirm the following working assumption Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. - Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

## RAN1#107bis-e

Agreement Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is based on configuration parameter with applicable value {2, 4} Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS Agreement One or more scrambling IDs is configured for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources. - Otherwise, each TRS resource is provided with a scrambling ID. Agreement The applicable values:{64, 128, 256, 512} are supported for the validity duration configured by higher layer. - Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX, it is up to UE implementation whether to assume the validity duration length is no larger than 10.24s Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently. Note: CATT has concern on the power saving gain based on this agreement. Agreement Confirm the following working assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI.