3GPP TSG RAN WG1 #108                      	                                     R1-220XXX

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:		8.15.7

Source:			Samsung

Title:			UE features for UE power saving enhancements

Document for:		Discussion and decision

# Introduction

This contribution includes comments for Rel-17 UE features for supporting UE power saving enhancements, as drafted in [1].

# 2. Comments to FG 29-1: TRS resources for idle/inactive UEs

In last RAN1 meeting, a LS was sent to RAN2 to check RAN2’s view about whether/how to separate the capability for UE subgroup indication. If a separate FG for component 2 is introduced, we think it’s necessary to clarify component 1 and component 2 are based on indication in DCI format 2_7; otherwise no change on the description of the components are needed.

Proposal 1: Support the following modification on the description of components for FG 29-1 if a separate FG for component 2 is introduced,

- 1. Support paging early indication in DCI format 2_7

- 2. Support UE subgroup indication in DCI format 2_7

# 2. Comments to FG 29-2: TRS resources for idle/inactive UEs

One remaining issue for FG 29-2 is whether/how to separate the capability for receiving L1 indication for TRS availability. We don’t see the need to separate the capability for receiving L1 indication for TRS availability, as UE doesn’t need to support DCI format 2_7 only for receiving TRS availability indication.

However, we think it’s necessary to clarify that the TRS availability indication can be provided in DCI 1_0 and/or DCI 2_7 if supported; otherwise it’s ambiguous whether or not UE needs to support DCI format 2_7 in order to support the entire feature of FG 29-2.

Proposal 2: Support the following modification on the description of components for FG 29-2:

- Support reading TRS configuration from SIB

- Support receiving L1 indication for TRS availability via DCI format 1_0

- Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

# 4. Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Support the following modification on the description of components for FG 29-1 if a separate FG for component 2 is introduced,

- 1. Support paging early indication in DCI format 2_7

- 2. Support UE subgroup indication in DCI format 2_7

Proposal 2: Support the following modification on the description of components for FG 29-2:

- Support reading TRS configuration from SIB

- Support receiving L1 indication for TRS availability via DCI format 1_0

- Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

# 5. Reference

[1] R1-2200678, Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.)