3GPP TSG RAN WG1 #108-e	        		             R1-2202047

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.12

Source: 	Samsung

Title: 	UE features for NR MBS

Document for:	Discussion and Decision

Introduction

This contribution considers UE features list for Rel-17 MBS.

Discussions

In [1], the preliminary RAN1 UE features list for Rel-17 was discussed.

Mandatory/optional and DCI format

For FG 33-1, this feature should be optional. Instead, it is proposed to note that this is a basic FG for MBS. If this is Optional without capability signaling, then “need for the gNB to know” should be no. Also, for the component 5, the DCI format should be 4_0.

| Features | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH - FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG | Optional without capability signalling This is a basic FG for MBS. |

Also, DCI format 0_1 and 1_1 in FG 33-2 and 33-6-1 should be replaced with DCI format 4_1 and 4_2, respectively.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. |

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. |

Pre-requisite

For 33-3-2/3, pre-requisite can be either 33-1 or 33-2, but should not be both.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1 or 33-2 |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1 or 33-2 |

Adding G-CS-RANTI for FG 33-6-1

For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1. |

FG 33-2d

Regarding which 33-2c (PTM reTx) or 33-2d (PTP reTx) to be merged 33-2a, we prefer to have 33-2d as a separate FG and use it as a pre-requisite of the capability to indicate whether a UE can support combining a PTM initial transmission and a PTP retransmission in case of different circular buffer.

Conclusions

This contribution considered the UE features list for Rel-17 MBS. The above changes are proposed.

- References

[1] RAN1 Chair’s Notes for RAN1#107b-e, Jan, 2022.

[2] R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e,” Moderator (NTT DOCOMO, INC.), Jan. 2022.