3GPP TSG RAN WG1 Meeting #108-e  	                          R1-2202052

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda Item: 8.13.1

Source: MediaTek Inc.

Title: On Cross-Carrier Scheduling from sSCell to P(S)Cell

Document for: Discussion and decision

- Introduction

In this contribution, we focus on the discussions for Rel-17 remaining issues of cross-carrier scheduling from SCell to P(S)Cell for NR.

- Discussion about Type A UE

2.1 Separation grid for search space sets on P(S)Cell and search space sets on sSCell

According to the RAN1 #105e [1] agreement as shown below:

Agreement

Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell

- For Type A UE

  - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - …

It can be seen that the designated search space sets on P(S)Cell and search space sets on sSCell are not overlapping by [slot/symbol] for Type A UE. Since the blind decoding budget in 38.213 [3] is defined in the unit of slot, to minimize the spec impact on BD/CCE handling and to enable potential backward compatibility of Type A UE to Rel-15/16 UE, it is more reasonable to set the separation grid on slot.

Proposal 1: For Type A UE, since the blind decoding budget in 38.213 [3] is defined in the unit of slot, to minimize the spec impact on BD/CCE handling, and to enable potential backward compatibility of Type A UE to Rel-15/16 UE:

- the designated search space sets on P(S)Cell and search space sets on sSCell are not overlapping by slot

- Overbooking handling on P(S)Cell

According to 38.213 [3] 10.1, the concept of overbooking is to define a “per scheduled cell BD limit for USS set: MPDCCHuss” for P(S)Cell

so that some USS CORSETs for P(S)Cell may be dropped to ensure the total PDCCH BD does not exceed the “per scheduled cell BD limit”  minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ for P(S)Cell.

In RAN1 #106-bis-e, it is agreed that

Agreement

Option A is supported in Rel-17

- At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1),[and at least when UE is not provided monitoringCapabilityConfig for any cell]

  - Option A

    - …

    - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ   is used for P(S)Cell overbooking procedure

For overbooking procedure described in 38.213 [3] 10.1, the following text are used to determine the dropping order of USS CORSETs for P(S)Cell:

- Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .

RAN1 agreed that PDCCH overbooking on sSCell USS set(s) is not allowed (RAN1 #104-bis-e), and α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure (RAN1 #106-bis-e), so only the P(S)Cell’s USS on P(S)Cell should be accounted for dropping in overbooking procedure using α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ.

Observation 1: RAN1 agreed that PDCCH overbooking on sSCell USS set(s) is not allowed (RAN1 #104-bis-e), and α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure (RAN1 #106-bis-e), so only the P(S)Cell’s USS on P(S)Cell should be accounted for dropping in overbooking procedure using α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ.

We hence have the following proposal:

Proposal 2: Adopt the following TP to 38.213 [3] 10.1 in Rel-17

- Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .  For cross-carrier scheduling from sSCell to P(S)Cell

  - considers only PDCCH candidates for monitoring for USS set on P(S)Cell for same cell scheduling

  - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ  is used for P(S)Cell overbooking procedure instead of minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ

  - PDCCH overbooking on sSCell USS set(s) is not allowed

- Application delay of the scaling factor α

In the final FL summary [4] of 8.13.1 in RAN1 #107e, application delay of the scaling factor α is discussed in FL proposal 9v3-1:

Proposal 9v3-1

- Following is supported

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X1 of P(S)Cell when sSCell deactivation is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X2 when sSCell BWP switch to dormant BWP is triggered in slot n of P(S)Cell

  - X1 = [BWP switching delay specified for P(S)Cell SCS]; X2 = [BWP switching delay specified for P(S)Cell SCS];

It can be seen that the FL proposal 9v3-1 discusses the application delay of not applying the scaling factor α to P(S)Cell (under sSCell deactivation or sSCell switching to dormant BWP). There is no need to discuss the sSCell part since sSCell just stops monitoring PDCCH.

At the same time, the application delay of applying the scaling factor α to P(S)Cell due to sSCell activation or sSCell switching to non-dormant BWP, should also be defined. There is no need to discuss the sSCell part since the (1- α) scaling should be ready when the sSCell activation or sSCell switching to non-dormant BWP is finished.

Observation 2: FL proposal 9v3-1 in the FL summary [4] (RAN1 #107e) discusses the application delay of not applying the scaling factor α to P(S)Cell (under sSCell deactivation or sSCell switching to dormant BWP). There is no need to discuss the sSCell part since sSCell just stops monitoring PDCCH.

Observation 3: Based on FL proposal 9v3-1, the application delay of applying the scaling factor α to P(S)Cell due to sSCell activation or sSCell switching to non-dormant BWP, should also be defined. There is no need to discuss the sSCell part since the (1- α) scaling should be ready when the sSCell activation or sSCell switching to non-dormant BWP is finished.

According to 38.133 [5] 8.2.2.2.2 “Interruptions at SCell activation/deactivation”, and 8.2.2.2.5 “Interruptions due to Active BWP switching Requirement” (spec contents copied in the appendix), P(S)Cell would be interrupted during sSCell activation/deactivation and sSCell switching into/out dormant BWP. This interruption may have impact on the X1/X2 values defined in FL proposal 9v3-1. This would require RAN4’s input.

Observation 4: According to 38.133 [5] 8.2.2.2.2 “Interruptions at SCell activation/deactivation”, and 8.2.2.2.5 “Interruptions due to Active BWP switching Requirement” (spec contents copied in the appendix), P(S)Cell would be interrupted during sSCell activation/deactivation and sSCell switching into/out dormant BWP. This interruption may have impact on the X1/X2 values defined in FL proposal 9v3-1. This would require RAN4’s input.

Considering Observations 2/3/4, we have the following proposal:

Proposal 3: Following is supported

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X1 of P(S)Cell when sSCell deactivation is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X2 when sSCell BWP switch to dormant BWP is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is applied for PDCCH monitoring on P(S)Cell from slot n+Y1 of P(S)Cell when sSCell activation is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is applied for PDCCH monitoring on P(S)Cell from slot n+Y2 when sSCell BWP switch to non-dormant BWP is triggered in slot n of P(S)Cell

Send an LS to RAN4 to let them determine the values of X1/X2/Y1/Y2 considering the interruption time defined in 38.133 [5] 8.2.2.2.2 “Interruptions at SCell activation/deactivation” and 8.2.2.2.5 “Interruptions due to Active BWP switching Requirement” may have impact on the X1/X2/Y1/Y2 values

- Discussion about DCI processing capabilities (FG 3-1 and FG 3-5b)

In Rel-15 NR, FG 3-1 is defined for per slot PDCCH monitoring limit and FG 3-5b is defined for per span PDCCH monitoring limit as shown in Figure 3 below. In Rel-17 DSS there are two scheduling cells (sSCell and P(S)Cell) for P(S)Cell and the SCS of the two scheduling cells (sSCell and P(S)Cell) can be different. Hence, introducing per span limit for Rel-17 DSS would be very complicated and we think following a similar form of FG 3-1 is adequate.

Figure 3

Proposal 4: For the scheduled P(S)Cell, UE can process 1D1U unicast DCI for FDD P(S)Cell and 1D2U unicast DCI for TDD PCell/PSCell per SCS=15K slot.

- Summary

In this contribution, we focus on the discussions for Rel-17 cross-carrier scheduling from SCell to P(S)Cell for NR and have the following observations and proposals:

Proposal 1: For Type A UE, since the blind decoding budget in 38.213 [3] is defined in the unit of slot, to minimize the spec impact on BD/CCE handling, and to enable potential backward compatibility of Type A UE to Rel-15/16 UE:

- the designated search space sets on P(S)Cell and search space sets on sSCell are not overlapping by slot

Observation 1: RAN1 agreed that PDCCH overbooking on sSCell USS set(s) is not allowed (RAN1 #104-bis-e), and α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure (RAN1 #106-bis-e), so only the P(S)Cell’s USS on P(S)Cell should be accounted for dropping in overbooking procedure using α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ.

Proposal 2: Adopt the following TP to 38.213 [3] 10.1 in Rel-17

- Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .  For cross-carrier scheduling from sSCell to P(S)Cell

  - considers only PDCCH candidates for monitoring for USS set on P(S)Cell for same cell scheduling

  - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ  is used for P(S)Cell overbooking procedure instead of minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ

  - PDCCH overbooking on sSCell USS set(s) is not allowed

Observation 2: FL proposal 9v3-1 in the FL summary [4] (RAN1 #107e) discusses the application delay of not applying the scaling factor α to P(S)Cell (under sSCell deactivation or sSCell switching to dormant BWP). There is no need to discuss the sSCell part since sSCell just stops monitoring PDCCH.

Observation 3: Based on FL proposal 9v3-1, the application delay of applying the scaling factor α to P(S)Cell due to sSCell activation or sSCell switching to non-dormant BWP, should also be defined. There is no need to discuss the sSCell part since the (1- α) scaling should be ready when the sSCell activation or sSCell switching to non-dormant BWP is finished.

Observation 4: According to 38.133 [5] 8.2.2.2.2 “Interruptions at SCell activation/deactivation”, and 8.2.2.2.5 “Interruptions due to Active BWP switching Requirement” (spec contents copied in the appendix), P(S)Cell would be interrupted during sSCell activation/deactivation and sSCell switching into/out dormant BWP. This interruption may have impact on the X1/X2 values defined in FL proposal 9v3-1. This would require RAN4’s input.

Proposal 3: Following is supported

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X1 of P(S)Cell when sSCell deactivation is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X2 when sSCell BWP switch to dormant BWP is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is applied for PDCCH monitoring on P(S)Cell from slot n+Y1 of P(S)Cell when sSCell activation is triggered in slot n of P(S)Cell

  - Scaling factor α (from RAN1#106bis-e agreement) is applied for PDCCH monitoring on P(S)Cell from slot n+Y2 when sSCell BWP switch to non-dormant BWP is triggered in slot n of P(S)Cell

Send an LS to RAN4 to let them determine the values of X1/X2/Y1/Y2 considering the interruption time defined in 38.133 [5] 8.2.2.2.2 “Interruptions at SCell activation/deactivation” and 8.2.2.2.5 “Interruptions due to Active BWP switching Requirement” may have impact on the X1/X2/Y1/Y2 values

- Appendix (38.133 spec on interruption of SCell activation/deactivation and BWP switch)

8.2.2.2.2	Interruptions at SCell activation/deactivation

When an intra-band SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed

-	an interruption on any active serving cell:

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

Where X2 is specified in Table 8.2.2.2.2-1.

or

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

8.2.2.2.5	Interruptions due to Active BWP switching Requirement

The requirements for DCI-based BWP switch, timer-based BWP switch or UL BWP switch triggered by consistent uplink CCA failures in this clause apply to the case that the BWP switch is performed on a single CC or multiple CCs.

When either of the DCI-based, timer-based or RRC-based downlink BWP switch and/or uplink BWP switch occur on multiple CCs simultaneously or over partially overlapping period, the interruption requirements described in this clause apply for each BWP switch.

When UE receives a DCI indicating UE to switch its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2 when BWP switch occurs on a single CC. The starting time of interruption caused by each BWP switch is only allowed within the BWP switch delay TMultipleBWPswitchDelay +Y as defined in clause 8.6.2A.1 when BWP switch occurs on multiple CCs. Interruptions are not allowed during BWP switch involving any other parameter change.

When a BWP timer bwp-InactivityTimer defined in TS 38.331 [2] expires, UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2 when BWP switch occurs on a single CC. The starting time of interruption caused by each BWP switch is only allowed within the BWP switch delay TMultipleBWPswitchDelay as defined in clause 8.6.2B.1 when BWP switch occurs on multiple CCs simultaneously or TMultipleBWPswitchDelayTotal as defined in clause 8.6.2B.2 when BWP switch occurs on multiple CCs over partially overlapping time period. Interruptions are not allowed during BWP switch involving any other parameter change.

When UE receives an RRC reconfiguration that only requests UE to switch its active BWP on one single CC, the UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1. The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC defined in clause 8.6.3 when BWP switch occurs on a single CC. The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC + DRRC*(N-1) as defined in clause 8.6.3A when BWP switch occurs on multiple CCs.

When UL BWP switch is triggered by consistent uplink CCA failures [7], UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active UL BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the UL BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing UL BWP switching. X is defined in Table 8.2.2.2.5-1. The starting time of interruption is only allowed within the UL BWP switching delay TBWPswitchDelay as defined in clause 8.6.2. Interruptions are not allowed during BWP switch involving other parameter change.

Table 8.2.2.2.5-1: Interruption length X

|  | NR Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Table 8.2.2.2.5-2: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| maxMIMO-Layers-r16 |  |

- Reference

Chairman’s Notes (Younsun’s Session), RAN1 #105-e (Rel-17 8.13 DSS)

3GPP TS 38.331 V16.7.0, ” NR; Radio Resource Control (RRC); Protocol specification”

3GPP TS 38.213 V16.6.0, ”NR; Physical layer procedures for control”

R1-2112733, “Summary#4 of Email discussion [107-e-NR-DSS-01]”, Moderator (Ericsson), RAN1 #107-e

3GPP TS 38.133 V16.10.0, ”NR; Requirements for support of radio resource management”