3GPP TSG RAN WG1 #108-e			R1-2202055

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	8.16.17

Source: 	MediaTek Inc.

Title: 	On UE features for others

Document for:	Discussion and decision

- Introduction

This paper provides our views on Rel-17 UE features in R1-2200781 (Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e) for others. Detailed proposals are described in the following sections.

- Views on UE features for others

In R1-2112858 “Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure”, RAN1 replied a LS to RAN4 about the feasibility of cross-PUCCH-group CSI report on PUCCH:

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

Answer: There is no restriction in the current RAN1 specification that would not allow UE to report CSI of a SCell belonging to secondary/primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary/secondary PUCCH group. But there is no RAN1 consensus on whether all UEs supporting NR-CA with dual PUCCH-groups for the BC support such CSI report in Rel-15 and Rel-16. Support of such CSI report is indicated in Rel-17 with a new UE capability. Potential CSI processing timeline relaxation for UEs reporting the new UE capability can be discussed.

It can be seen that RAN1 agrees to introduce a new UE capability in Rel-17 for cross-PUCCH-group CSI report on PUCCH.

Observation 1: In R1-2112858 “Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure”, RAN1 agrees to introduce a new UE capability in Rel-17 for cross-PUCCH-group CSI report on PUCCH.

We hence have the following proposal:

Proposal 1: Introduce a new UE capability in Rel-17 for cross-PUCCH-group CSI report on PUCCH in the RAN1 UE feature table.

# Proposed additional row to current RAN1 UE features table (additional contents in red text)

# 39. [NR_Cross_PUCCH_group_CSI_report]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Cross_PUCCH_group_CSI_report | 39-1 | Cross-PUCCH-group CSI report on PUCCH | Support cross-PUCCH-group CSI report on PUCCH | 6-7 | Y |  | UE does not support cross-PUCCH-group CSI report on PUCCH | Per UE | N | N | N |  | Optional with capability signaling |