3GPP TSG RAN WG1 #108-e                                                       R1-2202062

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.6.16

Source: 	MediaTek Inc.

Title: 	UE features for REDCAP

Document for:	Discussion and decision

# Introduction

This contribution provides our views on R17 RedCap UE features. In Section 2, we discuss the details of RedCap UE feature groups based on the baseline list in [1]. In Section 3, we provide answers to RAN2’s questions in [2] about whether any Rel-15/16 features are not applicable to RedCap UEs. Furthermore, our perspectives of the applicability of other Rel-17 UE features can be found in Section 4.

# Rel-17 RedCap UE Features

FG 28-1:

- Confirm RAN2’s working assumption that type of FG 28-1 is “Per UE.”

- Add that RedCap UEs are not allowed to support maximum UE bandwidth larger than 20MHz/100MHz in FR1/FR2 to the Note.

Proposal 1: For FG28-1, confirm the type is “Per UE.”

Proposal 2: Add that RedCap UEs are not allowed to support maximum UE bandwidth larger than 20MHz/100MHz in FR1/FR2, respectively, to the Note of FG 28-1.

As we describe in our companion contribution [3], the existing FG 1-4 is not applicable to RedCap UE because there is no RF retuning is assumed for non-RedCap UE to support the case when a configured CSI-RS resource is a different BWP than the associated SSB. Hence, we propose the following two new UE feature groups to explicitly distinguish the case that requires RF retuning from the case that doesn’t.

Proposal 3: Add two new UE feature groups, FG 28-4 and FG 28-5, for RedCap UE to indicate its support for CSI-RS based RRM measurement with associated SSB in a same and a different BWP, respectively.

- FG 28-4: CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth

- FG 28-5: CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth

| Index | Feature group | Component | Prerequisite FG | Type | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 28-4 | CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth | Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB is within UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement |  | [Per band] |  | Optional with capability signaling |
| 28-5 | CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth | Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB exceeds UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | FG 6-1a | [Per band] |  | Optional with capability signaling |

# Rel-15/16 UE Features Not Applicable to RedCap

Based on a RAN2 running CR for TS38.306 [4], UE features related to CA, MR-DC, DAPS, CPAC and IAB will be excluded by RedCap UEs. Hence, we don’t provide a list of features related to these identified aspects in this contribution.

### 4.2.xx RedCap Parameters RedCap UE is the UE with reduced capability: - The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2; - The maximum mandatory supported DRB number is 8; - The mandatory supported PDCP SN length is 12 bits while 18 bits being optional; - The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional; - 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; - CA, MR-DC, DAPS, CPAC and IAB ( i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise. Editor's Note: May be updated based on latest RAN1 and RAN4 agreements.

## Mandatory for non-RedCap but optional for RedCap

In our view, the following mandatory with capability signaling features for non-Red UEs should be optional for RedCap UEs to reduce RedCap UE’s complexity.

Table 1: Mandatory features for non-RedCap that are optional for RedCap

| Index | Feature group | Components | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| 1-7 | CSI-RS based RLM | CSI-RS based RLM | Applicable to UE supporting more than 4 layers | Mandatory with capability signalling |
| 2-4a | Additional active TCI state for PDCCH | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH | Only applicable if Component-1 of 2-4 is set to 1 | Mandatory with capability signalling |
| 2-16b | Support 1+2 DMRS (uplink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  | Mandatory with capability signalling |
| 2-55 | SRS Tx switch | 1) Support SRS Tx port switch, 2) Report whether the uplink TX switching impact to downlink receiving in a band, 3) Report whether the UL Tx is switched together with UL Tx in another band | Component-2 is per band pair per band combination Component-3 is per band pair per band combination The band pair in Component-2 and Component-3 can be an LTE band and an NR band 2T4R is 2 pairs of antennas "R" refers to a subset/set of receive antennas for PDSCH; "T" refers to the SRS antennas used for DL CSI acquisition | Mandatory with capability signalling Component-1 is a list of TRx pairs, candidates are {"Not supported", "1T2R", "1T4R", "2T4R", "1T4R/2T4R", "1T=1R", "2T=2R", "4T=4R"} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} |

Proposal 4: Mandatory FGs 1-7, 2-4a, 2-16b, 2-55, and 2-61 should be optional for RedCap.

## Mandatory for non-RedCap and RedCap but with different components and/or values

For FG 2-33, 2-35, and 2-51, components for “across all CCs” in the following FGs should not be applicable to RedCap UEs because they are related to CA aspects.

Table 2: Mandatory UE features for non-RedCap UEs that are mandatory for RedCap UEs but with different components and/or values

| Index | Feature group | Components | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC | All the candidate values are the range of capability signalling which doesn't determine whether UE is mandatory to support all the signalling values. | Mandatory with capability signalling Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } |
| 2-35 | CSI report framework | 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. | Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig. | Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
| 2-51 | TRS (CSI-RS for tracking) | 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs |  | Mandatory with capability signalling Component-1: Candidate values {1, "both 1 and 2"}. UE is mandated to report "both 1 and 2" Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. Optional with capability signalling |

Proposal 5: Components 4) and 6) of mandatory FG 2-33 are not applicable to RedCap.

Proposal 6: Component 9) of mandatory FG 2-35 are not applicable to RedCap.

Proposal 7: Component 4) of mandatory FG 2-51 are not applicable to RedCap.

## Optional for non-RedCap but not applicable for RedCap

According to the following description in WID [5], we should strive to reduce the cost of RedCap UEs.

As a baseline, the requirements for these three use cases are: Generic requirements: - Device complexity: Main motivation for the new device type is to lower the device cost and complexity as compared to high-end eMBB and URLLC devices of Rel-15/Rel-16. This is especially the case for industrial sensors. - Device size: Requirement for most use cases is that the standard enables a device design with compact form factor. - Deployment scenarios: System should support all FR1/FR2 bands for FDD and TDD.

Therefore, we propose UE features related to the following aspects are not applicable to RedCap UEs.

- FG 1-4 and 1-5 related to CSI-RS L3 measurement

- UE processing time capability 2

- Uplink CBG-based retransmission

- URLLC

- SCS of 60kHz in FR1

- BWP adaptation with different numerologies

In the following subsections, we provide lists of optional UE features related to the above aspects except for aspects in the first bullet.

### CSI-RS L3 measurement

The following optional FGs should not be applicable to RedCap UEs because they require more than two DL MIMO layers.

Table 3: Optional features FG 1-4 and FG 1-5 related to CSI-RS L3 measurement are not applicable to RedCap UE.

| Index | Feature group | Components | Prerequisite | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| 1-4 | CSI-RS based RRM measurement with associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | 1-1, CSI-RS | Optional with capability signaling |
| 1-5 | CSI-RS based RRM measurement without associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed | 1-1, CSI-RS | Optional with capability signaling |

Proposal 8: Optional FGs 1-4 and 1-5 related to CSI-RS L3 measurement are not applicable to RedCap.

### UE processing time capability 2

Proposal 9: The following optional feature groups related UE processing time capability 2 are not applicable to RedCap:

- FGs 5-5a, 5-5b, 5-5c, 5-13, 5-13a-f

- Uplink CBG with UE processing capability 2: FGs 22-3a, 22-3b, 22-3c, 22-3d, 22-3e, 22-3f, 22-g, 22-h

### Uplink CBG-based retransmission

Proposal 10: The following optional feature groups related to uplink CBG-based retransmission are not applicable to RedCap:

- FG 5-25: CBG-based re-transmission for UL using CBGTI

- FG 11-12: CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission

- Uplink CBG with UE processing capability 2: FGs 22-3a, 22-3b, 22-3c, 22-3d, 22-3e, 22-3f, 22-g, 22-h

- Uplink CBG with UE processing capability 1: FGs 22-4a, 22-4b, 22-4c, 22-4d

### URLLC

Proposal 11: The following optional feature groups related to URLLC are not applicable to RedCap:

- FG 11-2x: sub-slot based PDCCH monitoring

- FG 11-3x: support more than one PUCCH for HARQ-ACK in a slot

- FG 11-4x: support HARQ-ACK codebooks with different priorities at a UE

- FG 11-7x: UL cancellation

### SCS of 60kHz in FR1

| Index | Feature group | Components | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| RF/RRM 1-1 | 60kHz of subcarrier spacing for FR1 | 60kHz subcarrier spacing for data channel in FR1 |  | Optional with capability signaling |

Proposal 12: Optional RF/RRM FG1-1 (60kHz of subcarrier spacing for FR1) is not applicable for RedCap.

### BWP adaptation with different numerologies

| Index | Feature group | Components | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | Optional with capability signaling |

Proposal 13: Optional FG6-4 (BWP adaptation with different numerologies) is not applicable for RedCap.

## Optional for non-RedCap but mandatory for RedCap

We don’t see a need to mandate RedCap UEs to support optional features. Besides, we think RAN1 should prioritize on identifying features that are not applicable to RedCap to reduce RedCap UE implementation complexity.

Proposal 14: Optional UE features for non-RedCap UEs are not mandated for RedCap UEs.

# Other Rel-17 Features Not Applicable to RedCap

Among other Rel-17 work items, we prefer only features related power saving and coverage enhancements are considered as applicable to RedCap. The reason is not to complicate other Rel-17 work item’s advanced features by RedCap UE’s stringent design constraints and causing unnecessary optional components. We hence propose that feature groups in the following Rel-17 work items are not applicable to RedCap. For feature groups in other work items, we can discuss at a per-feature-group level.

Proposal 15: Rel-17 FGs 31-x in NR_IAB_enh, 34-x in NR_DSS, and 35-x in LTE_NR_DC_enh2 are not applicable to RedCap.

Proposal 16: Rel-17 FGs 26-x in NR_NTN_solutions are not applicable to RedCap.

Proposal 17: Rel-17 FGs 25-x in NR_IIOT_URLLC_enh are not applicable to RedCap.

Proposal 18: Rel-17 FGs 27-x in NR_pos_enh are not applicable to RedCap.

# Conclusion

Our proposals are summarized below.

Proposal 1: For FG28-1, confirm the type is “Per UE.”

Proposal 2: Add that RedCap UEs are not allowed to support maximum UE bandwidth larger than 20MHz/100MHz in FR1/FR2, respectively, to the Note of FG 28-1.

Proposal 3: Add two new UE feature groups, FG 28-4 and FG 28-5, for RedCap UE to indicate its support for CSI-RS based RRM measurement with associated SSB in a same and a different BWP, respectively.

- FG 28-4: CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth

- FG 28-5: CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth

Proposal 4: Mandatory FGs 1-7, 2-4a, 2-16b, 2-55, and 2-61 should be optional for RedCap.

Proposal 5: Components 4) and 6) of mandatory FG 2-33 are not applicable to RedCap.

Proposal 6: Component 9) of mandatory FG 2-35 are not applicable to RedCap.

Proposal 7: Component 4) of mandatory FG 2-51 are not applicable to RedCap.

Proposal 8: Optional FGs 1-4 and 1-5 related to CSI-RS L3 measurement are not applicable to RedCap.

Proposal 9: The following optional feature groups related UE processing time capability 2 are not applicable to RedCap:

Proposal 10: The following optional feature groups related to uplink CBG-based retransmission are not applicable to RedCap:

Proposal 11: The following optional feature groups related to URLLC are not applicable to RedCap:

Proposal 12: Optional RF/RRM FG1-1 (60kHz of subcarrier spacing for FR1) is not applicable for RedCap.

Proposal 13: Optional FG6-4 (BWP adaptation with different numerologies) is not applicable for RedCap.

Proposal 14: Optional UE features for non-RedCap UEs are not mandated for RedCap UEs.

Proposal 15: Rel-17 FGs 31-x in NR_IAB_enh, 34-x in NR_DSS, and 35-x in LTE_NR_DC_enh2 are not applicable to RedCap.

Proposal 16: Rel-17 FGs 26-x in NR_NTN_solutions are not applicable to RedCap.

Proposal 17: Rel-17 FGs 25-x in NR_IIOT_URLLC_enh are not applicable to RedCap.

Proposal 18: Rel-17 FGs 27-x in NR_pos_enh are not applicable to RedCap.

# References

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e,” AT&T and NTT DOCOMO, January 2022

- R2-2209218, RAN2, LS on capability related RAN2 agreements for RedCap, RAN2 #115-e, August 2021

- R1-2202061, MediaTek Inc., “On reduced bandwidth for NR RedCap UEs,” RAN1 #108-e, February 2022

- R2-2109668, Running 38.306 CR for the RedCap WI on capabilities, RAN2 #116-e, November 2021

- RP-211574, Ericsson, Revised WID on support of reduced capability NR devices, RAN #92e, June 2021