3GPP TSG RAN WG1 #108-e		R1-2202069

e-Meeting, February 21st – March 3rd, 2022

Agenda Item: 8.7.1.2

Source: MediaTek Inc.

Title: Maintenance on TRS/CSI-RS Information for Idle/Inactive UEs

Document for: Discussion and Decision

# Introduction

In this contribution, the remaining issue of TRS/CSI-RS information for idle/inactive UEs is identified and resolved.

# Application Delay of L1 Availability Indication

In RAN1#107bis-e, there agrees that availability information will be carried in PEI and paging PDCCH. When TRS availability is indicated by PEI before the reception of PO for current paging cycle, there should reserve a proper time gap for UE to warm up for TRS processing. Accordingly, proper application delay can be introduced, similar to the time gap between Rel-16 DCP and the start of the associated DRX on-duration.

According to Table 10.3-1 of TS 38.213, connected mode UE can report requirement of a 3-ms time gap between DCP and the start of the associated DRX on-duration. It is reasonable a longer time gap is reserved for idle/inactive mode UE to realize lower power consumption. By the above, [5]-ms application for PEI-based availability indication is suggested.

Observation 1: For TRS availability indication by PEI for upcoming PO in current paging cycle, a proper time gap should be accommodated for UE to warm up for TRS processing.

Observation 2: Connected mode UE can report requirement of a 3-ms time gap between DCP and the start of the associated DRX on-duration. It is reasonable a longer time gap is reserved for idle/inactive mode UE to realize lower power consumption.

Proposal 1: For PEI-based TRS availability indication, an application delay of [5]-ms is introduced.

Proposal 2: Adopt the following text proposal to Section 10.4B of TS 38.213:

<Unchanged parts are omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. The UE is not expected to receive the indicated TRS resources set within application delay of 5 ms after TRS availability indication provided by DCI format 2_7. <Unchanged parts are omitted>

# Reference

“Draft Report of 3GPP TSG RAN WG1 #107bis-e v0.2.0”, MCC support, RAN1#107bis-e

R1-2200816, “Physical layer procedures for control, 38.213 CR”, RAN1#107bis-e