3GPP TSG RAN WG1 #108-e                               	R1-2202071

e-Meeting, February 21st – March 3rd, 2022

Source: 	MediaTek Inc.

Title: 	Discussion on Rel-18 network energy savings

Agenda item:	8.7.3

Document for:	Discussion

# Introduction

The following objective has been agreed in [1] for the new study item on network energy savings in NR.

RP-213554, New SI-Study on network energy savings for NR, Huawei Definition of a base station energy consumption model [RAN1] - adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc.), sleep states and the associated transition times, and one or more reference parameters/configurations. Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

In this contribution, some initial views are provided for the following objectives.

- Suggested evaluation methodology, including base station (BS) energy consumption model, evaluation assumptions, and suggested metrics.

- Initial observations and potential techniques for network energy savings.

# Base Station Power Consumption Model

## BS power states and state transition modelling

For the gNB to achieve power saving during data inactivity, the concept of sleep mode or sleep state is introduced in [3], where gNB can gradually deactivate different sets of its components in each sleep mode that require similar deactivation time to achieve power saving during data inactivity. When gNB deactivates more of its components, the smaller power consumption it obtains at the cost of larger latency when the components are reactivated. When gNB has more than one power-saving mode, it has the flexibility of the hardware capability either to achieve more power saving or to respond quickly upon data transmission.

In a previous study of UE power saving in [2], there are three power-saving states for a UE, i.e., Micro Sleep, Light Sleep, and Deep Sleep. In addition to the Active state intended for transmission/reception, we propose to have Micro Sleep, Light Sleep and Deep Sleep power-saving state for gNB in consistent with the model for UE power states. For the Active state, we propose decomposing it into PDCCH + PDSCH transmission and RS transmission to distinguish between different factors that add up to the total power consumption. Since the proposed sleep model is similar to the power consumption model used in TR 38.840, the same framework can be reused to organize the study for gNB power consumption.

The same framework of the UE power consumption evaluation in 3GPP TR 38.840 can be leveraged to accommodate the study of network power saving.

The gNB power model should include gNB operations with more than one sleep mode to allow more flexible gNB power saving strategy. For example, ‘Micro Sleep’, ‘Light Sleep’ and ‘Deep Sleep’ state are considered in the power model for gNB.

Defining each state’s corresponding power consumption and transition time is necessary to evaluate these modes. The power consumption of each operation varies with different types of gNBs or gNBs with different hardware capabilities. For companies to report the power values, the configuration of the corresponding gNB should also be reported. For the 3GPP study, companies should agree on a table containing power consumption sets collected from different gNBs. Table 1 can serve as a template to collect the power states and the corresponding power consumptions with the values [3]. Moreover, scaling factors like TX/RX antenna numbers, resource utilization for the Active state could be considered in the model.

Include at least the following states in the gNB power model: PDCCH + PDSCH transmission, RS transmission, Micro Sleep, Light Sleep, and Deep Sleep.

Reference setting

- 4x4 MIMO Macro BS

- Number of carriers: 1CC

- System bandwidth: 100MHz

- Tx power: 49 dBm

- Full loaded at data transmission

Table 1: Template for the gNB power model

| Power State |  | Characteristics | Relative Power |
| --- | --- | --- | --- |
| Sleep | Micro Sleep | Immediate transition is assumed for power saving study purposes from or to a non-sleep state | [10] |
|  | Light Sleep | A sleep state with lower power consumption than Micro Sleep and can be entered if gNB stays in Micro sleep for more than a time threshold. It also requires additional transition time from/to Micro Sleep. | [2] |
|  | Deep Sleep | A sleep state with lower power consumption than Light Sleep and can be entered if gNB stays in Light sleep for more than a time threshold. It also requires additional transition time from/to Light Sleep. | [1] |
| Active | PDCCH + PDSCH transmission | DL transmission with active bandwidth of 100 MHz. | [250] |
|  | RS transmission | RS transmission, including SSB or CSI-RS/TRS. FFS: Different power consumption values for different RS characteristics in the time and frequency domain. | [100] |

gNB power state machine

For the implementation of the gNB power states in the system-level simulator, a state machine is proposed in [4]. to control the transition of the power states, as illustrated in Figure 1.

Figure 1: Base station power state machine

For the gNB to transit from a sleep mode j (SM j) to a deeper sleep state j+1, there is a timer with a value equal to TIDLE,j+1 starts when the gNB is considered to be idle. When the timer expires, the gNB starts to transit to a deeper sleep state with the deactivation time equal to  TT,j+1. The gNB wakes up from a sleep state if there is a need of transmission of data or reference signals. The activation time is assumed to be equal to the deactivation time and the wake-up process will traverse all previous sleep states until the gNB becomes fully activated. To determine the values of the idle threshold and the transition delay, we refer to [3] and the numbers are provided in Table 2. Note that we do not consider the transition delay from ACTIVE state to Micro Sleep state for simplicity because the delay is very short.

Table 2: The transition threshold and delay in the gNB power state machine

| Sleep Mode | Idle-time threshold, TIDLE,j [ms] | Transition delay, TT, j [ms] |
| --- | --- | --- |
| Micro Sleep (SM 1) | - | - |
| Light Sleep (SM 2) | 1 | 1 |
| Deep Sleep (SM 3) | 10 | 9 |

Despite the similarities between the gNB and UE sleep states, some difference still exists. The previous UE power saving study omitted the transition time from/to a sleep state and modeled by ramping energy otherwise. However, the transition time between states in the gNB state machine cannot be skipped since it may increase the data latency suggested to be one KPI in the evaluation. Note that the transition energy is modeled in the transition delay inherently.

Although the concept of sleep states is similar for the UE and gNB power saving, there is still some difference in the modeling: The transition delay between gNB power states cannot be omitted in the gNB state machine to include the latency impact on the network energy savings.

## BS Power consumption scaling for adaptation

In the gNB power model, the power consumption of each operation is based on the configuration of the reference gNB. Some scaling on the power values can be applied if a gNB with a different configuration or under different loading is considered. Various parameters can affect power consumption, such as resource utilization, number of carriers, and number of antennas. Table 3 captures the proposed parameters for adaptation with the potential equation for the scaling.

Table 3: gNB power consumption scaling

| Scaling for FR1 DL | Proposal | Comment |
| --- | --- | --- |
| Resource utilization (occupy X MHz) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. | Analogous to UE power scaling in TR 38.840 |
| CA | 2CC is 1.7x1CC 4CC is 3.4x1CC (i.e., 2x 2CC) | Analogous to UE power scaling in TR 38.840 |
| Antenna scaling | Scaling of Y Tx Antennae is 0.1 + 0.9 * Y/64 | Linear scaling for simplicity. The formula may be revised |

The power scaling factors regarding resource utilization, TX/RX antenna numbers, and number of CCs should be considered in the model for the evaluation.

On top of the BS model [3], some potential improvements for BS power consumption models can be further considered for Rel-18 network energy savings:

- RP-212422 [6] and R1-2200480 [7] proposed introducing improved models in 3GPP that better reflect the current gNB technology and the analysis structure anticipated in a prospective WI.

- RP-212156 [8] pointed out that this model can be viewed as an absolute power model, which depends on many implementation issues and is unsuitable for study in 3GPP.

Use the BS power consumption model in Tables 1, 2, and 3 as a starting point to consolidate the final model for network energy saving SI, further considering the digital front-end and beamforming impact for massive MIMO base stations.

# Simulation Assumptions

## General assumption and system metrics

This section lists the general assumption of the system-level evaluation for the NR network power saving study. The first and the most important is the scenario we are interested in. In the previous study for UE power saving, Dense Urban is prioritized as the deployment. As mentioned in our companion contribution [5] in the UE power saving SI, Dense Urban deployment can contain UEs with a variety of SINR conditions with low speed and is of interference-limited environment. Such deployment can also be foreseen to dominate the gNB power consumption characteristics and thus should be prioritized in the study.

Prioritize Dense Urban as the evaluated deployment for the network power saving study

Table 4 summarizes the simulation assumptions for Dense Urban in FR1.

Table 4: System-level simulation assumptions for Dense Urban in FR1

Besides the network power consumption that is the main target of the evaluation, other impacts to the network and UEs should be assessed. Other system metrics like UE power consumption, data latency, capacity, and resource utilization are suggested as KPIs to be considered. If the network power saving scheme involves any change of RS configuration, e.g., the periodicity of SSB, we should further check the evaluation methodology based on TR 36.839.

For the study of network power saving, KPIs such as gNB power consumption, UE power consumption, data latency, and resource utilization are considered. If the network power saving scheme involves any change of RS configuration, we should further investigate the evaluation methodology based on TR 36.839.

## Traffic models

Because the gNB’s activity is highly related to UE’s traffic, it is encouraged to evaluate the gNB’s power consumption under different traffic models. As a starting point, FTP, instant messaging (IM), and VoIP applications can be chosen to be the traffic models. The traffic assumptions from the previous UE power consumption evaluation [2] is captured in Table 5.

Table 5: Traffic models and the parameters

| Traffic type | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |
| DRX setting (if exists) | Period = 160 ms Inactivity timer = 100 ms | Period = 320 ms Inactivity timer = 80 ms | Period = 40 ms Inactivity timer = 10 ms |

Prioritize FTP, Instant messaging, and VoIP traffic with the traffic assumptions captured in TR 38.840 in the gNB power consumption evaluation.

## Assumption for periodic RS

The periodicity of RS may affect the level of sleep that gNB can achieve. During data inactivity, periodic RS like SSB may impede gNB from entering sleep mode and thus should be considered in the evaluation. There is no adequate time gap for gNB to enter deeper sleep mode when the RS is too frequent. Therefore, we propose to choose a period of 20ms since it is the periodicity that UE assumes during initial access.

Include the transmission of periodic RS in the baseline evaluation for the network power saving study and support 20ms to be the periodicity.

## Joint assumption with UE power saving

Since UE power saving gain is also a metric to be evaluated in the study, joint consideration with the UE power saving technique could be considered. For example, CDRX is the most common UE power saving technique and should be included in the baseline. The reference DRX setting is captured in the traffic model sections. Other power-saving schemes like wake-up signals can be optionally evaluated.

Include C-DRX in the baseline evaluation for the network power saving study.

# Initial Simulation Results

This section provides the preliminary simulation results of the gNB power consumption. The power model is based on Table 1, and the additional assumption is listed as below:

Additional assumptions

- Scaling: scaling for RU is applied

- RS periodicity: 20ms

- DRX configuration: according to Table 5

- Number of users per cell: 10 (FTP), 40 (IM, VoIP)

- The idle time (no transmission of any data) of the baseline is assumed to be Micro Sleep

In this evaluation, we have the following cases for comparison:

Cases

- Case 1 (baseline): Without BS power saving mode, C-DRX offset is multiples of 20ms

- Case 2: With BS power saving mode, C-DRX offset is multiples of 20ms

The simulation results of Case 1 and Case 2 are shown in Table 9, which demonstrates the benefits of the BS power-saving mode. Note that in Case1 and Case 2, the C-DRX offset is chosen to align with the RS periodicity to minimize sleep interruption. With no or little impact on the other system metrics, the application of BS power-saving mode can achieve a large power saving gain under all evaluated traffics.

Table 6: Simulation results for Case 1 and Case 2 in FR1 with Dense Urban deployment (with C-DRX)

| Traffic type | FTP, Case 1 | FTP, Case 2 | IM, Case 1 | IM, Case 2 | VoIP, Case 1 | VoIP, Case 2 |
| --- | --- | --- | --- | --- | --- | --- |
| BS power consumption (/ms) | 81.7450 | 69.7143 | 33.3854 | 15.7931 | 32.7790 | 22.2189 |
| BS power saving gain | n/a | 14.72% | n/a | 52.69% | n/a | 32.22% |
| UE power consumption (/ms) | 52.90 | 52.94 | 11.65 | 11.65 | 41.46 | 41.33 |
| UE power saving gain | n/a | -0.08% | n/a | 0.00% | n/a | 0.31% |
| Average packet latency (ms) | 50.99 | 51.59 | 141.97 | 142.54 | 11.91 | 12.22 |
| Average latency increment | n/a | 1.18% | n/a | 0.40% | n/a | 2.60% |
| RU | 31.64 | 31.46 | 2.29 | 2.29 | 0.21 | 0.21 |
| RU increment | n/a | -0.57% | n/a | 0.00% | n/a | 0.00% |

With BS power-saving modes utilized, 14.72%, 52.69%, 32.22% power saving gain can be achieved under FTP, IM, and VoIP traffic, respectively, with no or little impact to other system metrics.

Utilize BS power consumption with power-saving modes as a baseline for Rel-18 network energy-saving study to identify enhancements for further energy savings.

# Network Energy-Saving Technique

## Traffic awareness NW

One root cause of the high BS power consumption is high traffic load. However, according to our evaluation given in Table 6, VoIP has the lowest RU (0.21) but a higher power consumption value (22.23) than the IM’s value.

Given VoIP is an important use case, further enhancement for VoIP should be investigated. Additionally, traffic characteristics (bursty, sparse, period) and traffic QoS requirement (e.g., delay budget, data rate) could be useful information for the network to apply the optimized energy-saving techniques.

Enhance network energy saving for frequent but light-load traffic, e.g., VoIP.

Study the traffic awareness operation for Rel-18 NW energy savings.

## Reduction on SSB/SIB1

RP-212422 [6] observed that NR base station (gNB) consumes a large amount of energy even when the load is almost zero, e.g., only SSB and system information (SI) transmissions are ongoing. According to RP-212156 [8], 30% for FR1 and 15% for FR2 of symbols are active only for sending SSB and SIB1.

In Rel-17, at least for a primary cell, SSB signals are supposed to be “always-on.” A base station may broadcast up to 8 (for FR1) and 64 (for FR2) SSBs in a cell. It is a clear waste of energy if no user data is required (or no active UEs) in a particular beam direction.

For energy savings, a BS may reduce the number of SSBs in one SSB burst set. As a result, if to transmit X number of SSB needs X watts for a gNB, then if the gNB only transmits half SSB, there can be potential energy savings by half (X/2 watts). Also, if SSB transmission time can be less, gNB may stay in a sleep mode longer.

For example, considering the SSB configuration of Case A and B for FR1 larger than 3 GHz and less or equal to 6 GHz, up to 8 SSB beams are supported. TS 38.213: Case A - 15 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes of {2,8} + 14 ⋅ 𝑛. For carrier frequencies within FR1 larger than 3 GHz, 𝑛 = 0,1,2,3.

Figure 2: Case A: SSB candidates for 15 kHz SCS within FR1 larger than 3GHz

Depending on network requirement, it can selectively transmit only a few SSB and inform UE of which SSBs are transmitted and not transmitted. This transmission pattern is informed via an RRC IE ssb-PositionInBurst.

gNB can mute some SSB beams and inform UEs via an RRC IE ssb-PositionInBurst.

For example, when every SSBs are transmitted, the bitmap is set by all "1" bits, e.g., ssb-PositionsInBurst CHOICE {mediumBitmap = 11111111}. If only SSB 0, 1, 2, 3 are transmitted, the bitmap is set with half bits as “0”, e.g., ssb-PositionsInBurst CHOICE {mediumBitmap = 11110000}.

Figure 3: muting half SSBs in Case A by setting the bitmap as mediumBitmap = 11110000.

For a SS burst set periodicity of 20 ms, a 3GPP NR cell can stay in sleep mode from 16 ms to 18 ms, which leads to a sleeping ratio from 80% to 90% (improve 10%). Also, it saves energy from SSB transmission. With BSSM, for the SSB period of 20ms, reducing the SSB number from 8 to 4 may achieve a 40.7% power-saving gain (from 162 W to 96 W per 20ms) with the advanced sleep modes.

Figure: sleep mode operations for SSB duration of 4ms and 2ms when SSB period is 20ms.

Table 7: performance evaluation for reducing the number of SSB (no traffic load)

| Solution | Performance gain | Key assumptions |
| --- | --- | --- |
| Decrease the number of SSBs | 40.7 % | Baseline scheme: 8 SSBs Enhanced scheme: 4 SSBs SS burst set periodicity of 20 ms |

Decreasing the number of SSBs from 8 to 4 could save up to 40.7% energy consumption while there is no impact to legacy UE (no new UE behavior required).

Rel-18 network energy saving SI identifies network energy saving solutions with minimum impact to legacy UEs.

## Energy-saving state

In RP-213554, the study provides TS 28.310 as a reference for energy efficiency (EE) KPI, use cases, and requirements. In TS 28.310, an energy-saving state has been introduced, where a cell can move to an energy-saving state if some conditions are met. For example, when a cell is energy-saving, it may need candidate cells to pick up the load. All traffic on that cell is expected to be drained to other overlaid/umbrella candidate cells before the cell moves to the energy-saving state to prevent coverage holes or undue load on the surrounding cells.

Figure 4: scenarios in TS 28.310: overlaid by a macro cell and macro cell(s).

In TS 28.310, an energy-saving state has been specified to allow a cell to stay in a low-power operation; however, to ensure service quality, a cell can only move to the energy-saving state after its traffic load is picked up by candidate cells.

According to TS 28.310, a base station may decide by itself or be informed by NW to enter the energy-saving state when its traffic load is below a certain threshold, and its coverage can provide by its candidates.

One benefit of the energy-saving state is to ensure legacy UEs can continue accessing a network implementing Rel-18 network energy savings techniques. For example, if a base station initiates Rel-18 techniques that legacy UE cannot parse, the traffic load and QoS requirement shall be looked after by overlaid candidate cells.

Another benefit of the energy-saving state is to provide better flexibility to evaluate energy-saving techniques in the study. For example, since the candidate cells can provide coverage, a base station in the energy-saving state may use the SS block period higher than 20ms. It would allow more extended and deeper sleep periods up to 160ms, leading to more power-draining BS hardware can be progressively shut down.

Study the use case of the energy-saving state specified in TS 28.310, where the target gNB for energy saving is overlaid by one or more macro cells and the impact on legacy UEs can be controlled.

# Conclusion

In this contribution, we have the following observations

Observation 1	Although the concept of sleep states is similar for the UE and gNB power saving, there is still some difference in the modeling: The transition delay between gNB power states cannot be omitted in the gNB state machine to include the latency impact on the network energy savings.

Observation 2	With BS power-saving modes utilized, 14.72%, 52.69%, 32.22% power saving gain can be achieved under FTP, IM, and VoIP traffic, respectively, with no or little impact to other system metrics.

Observation 3	gNB can mute some SSB beams and inform UEs via an RRC IE ssb-PositionInBurst.

Observation 4	Decreasing the number of SSBs from 8 to 4 could save up to 40.7% energy consumption while there is no impact to legacy UE (no new UE behavior required).

Observation 5	In TS 28.310, an energy-saving state has been specified to allow a cell to stay in a low-power operation; however, to ensure service quality, a cell can only move to the energy-saving state after its traffic load is picked up by candidate cells.

Based on observations, the following proposals are made

Proposal 1	The same framework of the UE power consumption evaluation in 3GPP TR 38.840 can be leveraged to accommodate the study of network power saving.

Proposal 2	The gNB power model should include gNB operations with more than one sleep mode to allow more flexible gNB power saving strategy. For example, ‘Micro Sleep’, ‘Light Sleep’ and ‘Deep Sleep’ state are considered in the power model for gNB.

Proposal 3	Include at least the following states in the gNB power model: PDCCH + PDSCH transmission, RS transmission, Micro Sleep, Light Sleep, and Deep Sleep.

Proposal 4	The power scaling factors regarding resource utilization, TX/RX antenna numbers, and number of CCs should be considered in the model for the evaluation.

Proposal 5	Use the BS power consumption model in Tables 1, 2, and 3 as a starting point to consolidate the final model for network energy saving SI, further considering the digital front-end and beamforming impact for massive MIMO base stations.

Proposal 6	Prioritize Dense Urban as the evaluated deployment for the network power saving study

Proposal 7	For the study of network power saving, KPIs such as gNB power consumption, UE power consumption, data latency, and resource utilization are considered. If the network power saving scheme involves any change of RS configuration, we should further investigate the evaluation methodology based on TR 36.839.

Proposal 8	Prioritize FTP, Instant messaging, and VoIP traffic with the traffic assumptions captured in TR 38.840 in the gNB power consumption evaluation.

Proposal 9	Include the transmission of periodic RS in the baseline evaluation for the network power saving study and support 20ms to be the periodicity.

Proposal 10	Include C-DRX in the baseline evaluation for the network power saving study.

Proposal 11	Utilize BS power consumption with power-saving modes as a baseline for Rel-18 network energy-saving study to identify enhancements for further energy savings.

Proposal 12	Enhance network energy saving for frequent but light-load traffic, e.g., VoIP.

Proposal 13	Study the traffic awareness operation for Rel-18 NW energy savings.

Proposal 14	Rel-18 network energy saving SI identifies network energy saving solutions with minimum impact to legacy UEs.

Proposal 15	Study the use case of the energy-saving state specified in TS 28.310, where the target gNB for energy saving is overlaid by one or more macro cells and the impact on legacy UEs can be controlled.

# Reference

RP-213554, “New SI: Study on network energy savings for NR”, RAN#94e, Huawei.

3GPP TR 38.840 V16.0.0 (2019-06), Study on User Equipment (UE) power saving in NR, Release 16

B. Debaillie, C. Desset, and F. Louagie, “A flexible and future-proof power model for cellular base stations,” IEEE Vehicular Technology Conference, vol. 2015, 2015.

P. Lähdekorpi, M. Hronec, P. Jolma and J. Moilanen, “Energy efficiency of 5G mobile networks with base station sleep modes,” 2017 IEEE Conference on Standards for Communications and Networking (CSCN), 2017

R1-1810447, “Evaluation Methodology for NR UE Power Saving Study”, RAN1#94bis, Mediatek Inc.

RP-212422, “Motivation for Network Energy Saving in Rel-18”, Ericsson

R1-2200480, “Modeling of NR gNB power consumption”, Ericsson

RP-212156, “Updated views on Rel-18 network energy saving”, Huawei

3GPP TS 38.213 V17.0.0 (2021-12), “Physical layer procedures for control”, 2021

3GPP TS 28.310, “Energy efficiency of 5G”, V17.3.0, 2021