3GPP TSG RAN WG1 #108-e		    R1-2202082

e-Meeting, February 21st – March 3rd, 2022

Source: 	MediaTek Inc.

Title:	Views on UE features for NR MBS

Agenda item:	8.16.12

Document for:	Discussion & Decision

## Introduction

According to the guidance of agreed timeline and planning in RAN#92-e [1], the discussion for Rel-17 UE features in RAN1 has been started. The UE features have been discussed in several meetings, and the latest updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e can be found in [2]. In last RAN1 meeting [3], the MBS UE features were further discussed, and the corresponding conclusion can be found in [4]. In this contribution, we will further discuss the NR MBS UE features based on the agreements achieved in several last RAN1 meetings.

## Discussion

In the latest approved TS 38.212 spec, the three DCI formats were defined for MBS broadcast and multicast that DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. In order to align the MBS DCI format naming, we suggest the following proposal:

Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively.

Considering the concept of group-common PDCCH/PDSCH is used for broadcast and multicast and it only state “group-common PDCCH/PDSCH” in some FG, it may cause some reader confused, e.g., which means for broadcast or multicast? Therefore, we want to clarify the concept more clearly with some constrains, e.g., group-common PDCCH/PDSCH for multicast or broadcast, respectively.

Proposal 2: Clarify the wording of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast.

Regarding the discussion of which granularity type definition should be defined for MBS UE feature, from our view, the different carriers within the same band will have different characteristics or usage purpose, especially for considering the band n28 is a shared band by operators. What’s more, considering the IODT testing cost, we prefer the type definition for MBS UE features should be based on the granularity per FSPC.

Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC.

  - 33-1: Broadcast

Regarding the number of MCCH-RNTI and G-RNTI for broadcast, there has not been discussed for now and need to make it clearer. For the MCCH-RNTI, we think only one MCCH-RNTI is sufficient for UE obtaining the MBS control information as similar with the legacy LTE SC-PTM mechanism. For the G-RNTI issue, RAN2 has achieved an agreement that “one-to-many mapping between G-RNTI and MBS sessions is supported” in previous meeting, which means the multiple services if existed can use only for G-RNTI for services reception. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multiple multicast G-RNTIs/G-CS-RNTIs, broadcast MCCH-RNTI) however, a total number RNTI supported by UE is limited. Considering the above reason and broadcast is a best-effort reception, we suggest only one G-RNTI is supported for broadcast reception.

Proposal 4: For FG 33-1, adding a note that “For component 1, only one MCCH-RNTI is supported for broadcast”.

Proposal 5: For FG 33-1, adding a note that “For component 2, only one G-RNTI is supported for broadcast”.

Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception since we also have agreed that the CFR frequency for MCCH and MTCH can be configured by SIBx.

Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

Proposal 6: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”.

Regarding slot-level repetition for MTCH, the following agreement was achieved in last meeting:

Agreement: A new component for support of higher layer configured slot-level repetition up to 8 for MTCH is added into FG 33-1 - FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG

Since the semi-static slot-level repetition added into FG33-1 has been agreed, the reliability concern for MTCH has been resolved. Besides, considering the dynamic slot-level indication was introduced in Rel-16 as a separate FG and it also has been agreed for multicast as a separate FG, we suggest the following mechanism can be reused for broadcast.

Proposal 7: Adding a new separate FG 33-1-1 to capture the dynamic slot-level repetition for broadcast MTCH.

| 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG |  | Yes |  |  | Per UE Per FSPC | No | No |  | For component 1, only one MCCH-RNTI is supported for broadcast For component 2, only one G-RNTI is supported for broadcast For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-1-1 | Dynamic slot-level repetition for broadcast MTCH | - Support up to X times dynamic slot-level repetition for broadcast MTCH. | 33-1 |  |  |  | Per FSPC |  |  |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

  - 33-2: Dynamic scheduling for multicast

Regarding the FG 33-2, the following agreements were achieved in last meeting:

Agreement - Slot level repetition (FG 33-3-1) is merged with FG 33-2 - FFS whether to include semi-static repetition only or both semi-static and dynamic repetitions - FFS max value of dynamic repetition - FFS max value of semi-static repetition - Components 6/7/8 are separated from FG 33-2, FFS how to separate Agreement - Only the capability for semi-static repetition is included in FG 33-2 - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions - For dynamic repetition, a separate FG is introduced; UE reports one of the maximum values from {8, 16} Agreement - Add following FGs - FG 33-2a: Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - [FG 33-2c: PTM retransmission for multicast] - [FG 33-2d: PTP retransmission for multicast]

There is a remaining issue that “At least 33-2c or 33-2d is merged, FFS which one or both”. From our perspective, considering PTM transmission is a basic feature for MBS services and can enhance the spectrum resource utilization, we prefer the FG 33-2c can be merged into FG 33-2a, and PTP retransmission can be split as a separate FG.

Proposal 8: The FG 33-2c (i.e., PTM retransmission for multicast) can be merged into FG 33-2a.

Proposal 9: The FG 33-2d (i.e., PTP retransmission for multicast) is split as a separate FG.

Regarding the CFR number for multicast reception, the following agreement was achieved in previous RAN1 meeting:

Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.

Thus, we prefer to update the 2nd component based on the latest agreement.

Proposal 10: For FG 33-2, adding a note that “for component 2, only one CFR is supported for multicast reception”.

Regarding whether to separate the capability for support of DCI format 4_2 for multicast, we think the DCI format 4_1 for multicast is sufficient and the DCI format 4_2 can be defined as the similar with DCI format 1_2 that as a separate UE capability.

Proposal 11: For FG 33-2, monitoring DCI format 4_2 can be as a separate FG.

| 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1/4_2 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast |  | Yes |  |  | Per UE Per FSPC | No | No |  | For component 2, only one CFR is supported for multicast reception | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling At least 33-2c or 33-2d is merged, FFS which one or both - Support of PTM retransmission for multicast | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2-1 | Monitoring DCI format 4_2 | Support monitoring DCI format 4_2 for multicast DL scheduling | 33-2 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |

  - 33-2-X: Multiple G-RNTIs for group-common PDSCHs

In the RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following.

one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.

Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear, and supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multicast G-RNTI/G-CS-RNTI, broadcast MCCH-RNTI/G-RNTI), however, a total number RNTI supported by UE is limited. Considering the reason above, We think one G-NRTI is sufficient for Rel-17 UE supporting multicast services.

Proposal 12: Only one multicast G-RNTI is sufficient for Rel-17 UE supporting multicast services.

Proposal 13: For FG 33-2-X, RAN1 needs to re-consider the motivation for UE supporting multiple G-RNTIs.

Proposal 14: For FG 33-2-X, adding a note that “For component 1, the candidate value for M is: {0, 1}”.

| 33-2-x | Multiple G-RNTIs for group-common PDSCHs | - Capability on number of Support up to M G-RNTI for multicast groupcast FFS details. | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  | For component 1, the candidate value for M is: {0, 1} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH

Regarding the FG 33-3-2, we need to further discuss whether/how to separate the capability for HARQ-ACK codebook. From our understanding, supporting FDM between unicast and GC-PDSCH does not mean FDM-ed HARQ-ACK codebook. Thus, we suggest to separate the FDM-ed HARQ-ACK from the FG 33-3-2.

Proposal 15: For FG 33-3-2, the FDM-ed HARQ-ACK codebook capability should be as a separate FG.

Regarding the prerequisite feature groups 33-1, we think the broadcast reception is “best-effort” reception and the UE in RRC IDLE/INACTIVE cannot report capability. So, we think FDMed reception for MBS is supported for multicast and not broadcast.

Proposal 16: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted.

| 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-3-2-1 | HARQ-ACK codebook for FDM-ed unicast and multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast | 33-3-2 |  |  |  | Per UE Per FSPC |  |  |  |  | Optional with capability signalling |

  - 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

Regarding the FG 33-3-3 we need to further discuss whether/how to separate the capability for HARQ-ACK codebook. We share the similar view as that of FG 33-3-2 and have the following proposals:

Proposal 17: For FG 33-3-3, the TDM-ed HARQ-ACK codebook capability should be as a separate FG.

Proposal 18: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-3-2-1 | HARQ-ACK codebook for FDM-ed unicast and multicast | - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast |  |  |  |  | Per UE Per FSPC |  |  |  |  | Optional with capability signalling |

  - 33-5-1: SPS group-common PDSCH for multicast

Regarding the SPS group-common PDSCH for multicast, we need to further discuss whether/how to separate the capabilities from FG 33-5-1. Actually, we think the capability for SPS should be align with the dynamic scheduling and have the following proposal:

Proposal 19: For FG 33-5-1, the similar separate capability for dynamic scheduling can be reused for SPS group-common PDSCH for multicast.

| 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-5-1a | ACK/NACK based HARQ-ACK feedback for SPS multicast | - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1b | NACK only based HARQ-ACK feedback for SPS multicas | - Support NACK only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1c | PTM retransmission for SPS multicast | - Support PTM retransmission of SPS group common PDSCH for multicast | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1d | PTP retransmission for SPS multicast | - Support PTP retransmission of SPS group common PDSCH for multicast | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1f | Dynamic Slot-level repetition for SPS multicast | - Support up to X times dynamic slot-level repetition for group-common PDSCH for SPS multicast. | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

## Conclusion

In this contribution, it discusses the NR MBS UE features, and the following proposal are suggested:

Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively.

Proposal 2: Clarify the wording of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast.

Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC.

### 33-1: Broadcast

Proposal 4: For FG 33-1, adding a note that “For component 1, only one MCCH-RNTI is supported for broadcast”.

Proposal 5: For FG 33-1, adding a note that “For component 2, only one G-RNTI is supported for broadcast”.

Proposal 6: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”.

Proposal 7: Adding a new separate FG 33-1-1 to capture the dynamic slot-level repetition for broadcast MTCH.

| 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG |  | Yes |  |  | Per UE Per FSPC | No | No |  | For component 1, only one MCCH-RNTI is supported for broadcast For component 2, only one G-RNTI is supported for broadcast For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resouece is configured by SIBx | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-1-1 | Dynamic slot-level repetition for broadcast MTCH | - Support up to X times dynamic slot-level repetition for broadcast MTCH. | 33-1 |  |  |  | Per FSPC |  |  |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

### 33-2: Dynamic scheduling for multicast

Proposal 8: The FG 33-2c (i.e., PTM retransmission for multicast) can be merged into FG 33-2a.

Proposal 9: The FG 33-2d (i.e., PTP retransmission for multicast) is split as a separate FG.

Proposal 10: For FG 33-2, adding a note that “for component 2, only one CFR is supported for multicast reception”.

Proposal 11: For FG 33-2, monitoring DCI format 4_2 can be as a separate FG.

| 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1/4_2 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast |  | Yes |  |  | Per UE Per FSPC | No | No |  | For component 2, only one CFR is supported for multicast reception | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling At least 33-2c or 33-2d is merged, FFS which one or both - Support of PTM retransmission for multicast | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| 33-2-1 | Monitoring DCI format 4_2 | Support monitoring DCI format 4_2 for multicast DL scheduling | 33-2 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |

### 33-2-X: Multiple G-RNTIs for group-common PDSCHs

Proposal 12: Only one multicast G-RNTI is sufficient for Rel-17 UE supporting multicast services

Proposal 13: For FG 33-2-X, RAN1 needs to re-consider the motivation for UE supporting multiple G-RNTIs.

Proposal 14: For FG 33-2-X, adding a note that “For component 1, the candidate value for M is: {0, 1}”.

| 33-2-x | Multiple G-RNTIs for group-common PDSCHs | - Capability on number of Support up to M G-RNTI for multicast groupcast FFS details. | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  | For component 1, the candidate value for M is: {0, 1} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH

Proposal 15: For FG 33-3-2, the FDM-ed HARQ-ACK codebook capability should be as a separate FG.

Proposal 16: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted.

| 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-3-2-1 | HARQ-ACK codebook for FDMed unicast and multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast | 33-3-2 |  |  |  | Per UE Per FSPC |  |  |  |  | Optional with capability signalling |

### 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

Proposal 17: For FG 33-3-3, the TDM-ed HARQ-ACK codebook capability should be as a separate FG.

Proposal 18: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-3-2-1 | HARQ-ACK codebook for FDMed unicast and multicast | - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast |  |  |  |  | Per UE Per FSPC |  |  |  |  | Optional with capability signalling |

### 33-5-1: SPS group-common PDSCH for multicast

Proposal 19: For FG 33-5-1, the similar separate capability for dynamic scheduling can be reused for SPS group-common PDSCH for multicast.

| 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-5-1a | ACK/NACK based HARQ-ACK feedback for SPS multicast | - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1b | NACK only based HARQ-ACK feedback for SPS multicas | - Support NACK only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1c | PTM retransmission for SPS multicast | - Support PTM retransmission of SPS group common PDSCH for multicast | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1d | PTP retransmission for SPS multicast | - Support PTP retransmission of SPS group common PDSCH for multicast | 33-5-1 |  |  |  | Per FSPC |  |  |  |  | Optional with capability signalling |
| 33-5-1f | Dynamic Slot-level repetition for SPS multicast | - Support up to X times dynamic slot-level repetition for group-common PDSCH for SPS multicast. | 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

## Reference

- RP-211582, “RAN Timeline and Meeting Planning”, RAN#92-e, RAN Chair, RAN1 Chair, RAN2 Chair, RAN3 Chair, RAN4 Chair, RAN5 Chair, June 14th -18th, 2021.

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 107bis-e”, RAN1#107bis-e, Moderators (AT&T, NTT DOCOMO, INC.), January 17th - 25th, 2022

- RAN1 Chairman’s Notes, RAN1 #107bis-e, January 2022.

- R1-2200258, “Summary on UE features for NR MBS”, RAN1#107bis-e, NTT DOCOMO, INC, January 17th - 25th, 2022