3GPP TSG RAN WG1 #108-e	R1-2202089

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Source:	Lenovo

Title:	CSI enhancements for multi-TRP and FDD reciprocity

Agenda Item:	8.1.4

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e, the discussion on the main aspects of CSI enhancements for multi-TRP deployments, as well as CSI enhancements under FDD channel reciprocity was concluded. In this contribution we provide our views on a few different aspects of the CSI enhancements under multi-TRP/panel transmission, as well as on CSI enhancements under FR1 FDD reciprocity based on the class of Type-II Port Selection codebooks.

# CSI Reporting for DL multi-TRP/Panel Transmission

In RAN1#107-e [1], the following agreements were made for CSI enhancements under multi-TRP transmission:

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting: - Two CBSRs can be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP. Agreement For the ordering of UCI payload construction for reported CSIs - Alt 4: modify mapping order of CSI fields of one CSI report, e.g. Table 6.3.2.1.2-3/4/5 in 38.212 - i.e. introducing mapping order of CSI fields in the order of MTRP CSI, the first TRP CSI, and the second TRP CSI. - It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which corresponds multiple single-TRP and/or NCJT measurement hypotheses. Note: There is no further optimization for CSI part 2 omission rules in Rel-17 Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE can be configured with pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI only for Mode 1 with X=0 Agreement Regarding to the restriction applying to parameter combination for Rel-17 PS codebook: - Alt 3: {M, alpha, beta ={2,1,3/4} and {2, 1, 1/2} are only applicable to P <= 24 ports Agreement for the ordering of UCI payload construction with PMI/CQI configured for subband reporting, mapping order of CSI fields are in the order of NCJT CSI, the first TRP CSI, and the second TRP CSI applied separately, for CSI part 1, CSI part 2 WB, even subbands of CSI part 2 SB, and odd subbands of CSI part 2 SB.

## CBSR configuration for multi-TRP Transmission

In RAN1#107-e, it was agreed to support two CBSRs for Rel. 17 multi-TRP CSI framework, one corresponding to each CSI-RS resource group. No further details of the CBSR were provided. First, it was not agreed whether CBSR would be supported for 2Tx  and more than 2Tx. In our opinion, CBSR should be supported for both 2Tx and >2Tx. Since the codebook type is restricted to Type-I single panel codebook for multi-TRP framework, the CBSR corresponding to 2Tx should be twoTX-CodebookSubsetRestriction as defined in Clause 5.2.2.2.1 of TS 38.214 [2], whereas the CBSR corresponding to >2Tx should be typeISinglePanel-codebookSubsetRestriction-i2 as defined in Clause 5.2.2.2.1 of TS 38.214.

CBSR is supported for 2Tx based on twoTX-CodebookSubsetRestriction configuration, and for >2Tx based on typeISinglePanel-codebookSubsetRestriction-i2 configuration

When the UE is configured with two CSI-RS resource groups corresponding to multi-TRP, the two CBSRs can be applied via two codebook configurations corresponding to the two CSI-RS resource groups.

Two codebook configurations are defined corresponding to the two CSI-RS resource groups, where each codebook configuration comprises a CBSR configuration

# Type-II Port Selection Codebook Enhancement

In RAN1#107-e, the following agreements were made for port-selection codebook enhancements under FDD channel reciprocity in FR1:

Agreement The window size is min(N,N3). - Note: the UCI payload of i1,6 is ceiling(log2(N-1)) bits regardless of values of N3 Agreement Support to report Port indicator, SCI, and FD indicator in Group 0 - FFS (to be concluded in RAN1 107): Report bitmap in Group 0 or Group 1 without bitmap partition - FFS (to be concluded in RAN1 107): whether/how to deal with coefficients partition issue when ceil(KNZ/2-v) < 0 or report coefficients together without partitioning - Note: It is RAN1 common understanding that when UCI omission occurs for Rel-17 PS codebook, the associated CQI does not have to be recalculated conditioned on the PMI after omission. Agreement In Rel-17, the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l whereas for ψ(i), - Alt 2: Support non-interleaving between polarization, ψi=i Agreement Regarding to codebook parameters for Rel-17 PS codebook, (Alt-1) alpha =3/4 is not applicable to 4 and 12 CSI-RS ports. Agreement For Rel-17 PS codebook: - Alt 2(updated #2): UCI Group 1 includes the max(0, KNZ2-υ) highest priority elements of i2,4,l and the max(0, KNZ2-υ) highest priority elements of i2,5,l (l=1,…,υ). UCI Group 2 includes the min(KNZ-v, KNZ2) lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) lowest priority elements of i2,5,l (l=1,…,υ) Agreement Regarding to the restriction applying to parameter combination for Rel-17 PS codebook: - Alt 3: {M, alpha, beta ={2,1,3/4} and {2, 1, 1/2} are only applicable to P <= 24 ports Conclusion Excepting for reporting port indicator, SCI, and FD indicator in Group 0, the remaining indicators are mapped to reporting Groups 1 and 2 same as Rel-16 eType-II PS codebook with updated the priority value of Phi(i).

## Parameter combination configuration for FeType-II codebook

In RAN1#106bis-e [3], it was agreed to support two value R=1,2, where R is the number of PMI sub-bands per CQI sub-band with R=2 being optionally supported based on a UE feature. We prefer to follow the legacy values in Rel. 16 Type-II codebook with R=1, 2, with the value “2” being optional. Since a configuration with R=2 is only valid if M >1, we prefer including the parameter R to set of parameters of the parameter combination, as shown in Table 3, as follows

| M | Alpha | Beta | R |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 1 | 1 | 3/4 | 1 |
| 1 | 1 | 1/2 | 1 |
| 1 | 3/4 | 1/2 | 1 |
| 2 | 1 | 3/4 | 1 |
| 2 | 1 | 1/2 | 1 |
| 2 | 3/4 | 1/2 | 2 |
| 2 | 1/2 | 1/2 | 2 |

Table 3: Proposed parameter combination table for FeType-II codebook

In the table above, only configurations associated with parameter combinations including both M=2, Alpha<1 are associated with R=2, in order to avoid further increasing the complexity for configurations with M=2, Alpha=1.

The parameter paramCombination-r17 configures the supported parameter combination values of the parameters (α, Mv, β, R)

# Conclusion

This contribution addressed CSI enhancements for NR Rel. 17, including enhancements for NCJT as well as CSI enhancements under FDD channel reciprocity in FR1. We have the following proposals

- CBSR is supported for 2Tx based on twoTX-CodebookSubsetRestriction configuration, and for >2Tx based on typeISinglePanel-codebookSubsetRestriction-i2 configuration

- Two codebook configurations are defined corresponding to the two CSI-RS resource groups, where each codebook configuration comprises a CBSR configuration

- The parameter paramCombination-r17 configures the supported parameter combination values of the parameters (α, Mv, β, R)

# References

| [1] | 3GPP TSG RAN WG1 #107-e, "Draft Meeting Report," Online, Nov. 11-19, 2021. |
| --- | --- |
| [2] | 3GPP TS 38.214, "Physical layer procedures for data," Mar. 2020. |
| [3] | 3GPP TSG RAN WG1 #106bis-e, "Draft Meeting Report," Online, Oct. 11-19, 2021. |