3GPP TSG RAN WG1 #108-e									R1-2202105

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft Reply to RAN4 LS on reporting and definition of DL PRS path RSRP

Response to:       R4-2202780

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	Qualcomm Incorporated [RAN1]

To:	RAN4

CC:

# Contact Person:

# Name:	Alexandros Manolakos

E-mail Address:	amanolak@qti.qualcomm.com

# 1. Overall Description:

RAN1 thanks RAN4 for their LS on reporting and definition of DL PRS path RSRP and would like to provide a reply to the following topic:

The PRS-RSRPP can be reported in ways below : - RAN4 understands that PRS-RSRPP should be defined as the path RSRP per RE, in the same way PRS-RSRP is defined as power per RE. RAN4 respectfully asks RAN1 to confirm if this understanding is correct. - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting. RAN4 concludes that these ways are possible to support PRS-RSRPP in Rel-17. RAN4 kindly requests RAN1/2 to take the above information into consideration in the further specification work, and respectfully asks to inform RAN4 if RAN1/2 finds any issue or update for RAN4 understanding.

# RAN1 discussed the above aspects and would like to provide the following observations:

- RAN1 confirms that PRS-RSRPP should be defined as the path RSRP per RE.

- It is preferrable, from RAN1 perspective, to support reporting the first-path PRS-RSRPP by reusing the absolute PRS-RSRP measurement report mapping for the first measurement (similar to PRS-RSRP in NR Rel-16).

- It is preferrable, from RAN1 perspective, to support reporting the first-path PRS-RSRPP for additional measurements by reusing differential PRS-RSRP measurement report mapping with respect to the first-path PRS-RSRPP of the first measurement (similar to the reporting of PRS-RSRP for the additional measurements in NR Rel-16).

- It is preferrable, from RAN1 perspective, to support reporting additional path PRS-RSRP for a measurement (first or additional) by using differential reporting with respect to the PRS-RSRPP of the first-path of the measurement.

# 2. Actions to RAN4:

RAN1 respectfully requests RAN4 to take the abovementioned information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e	May 2022 		      		e-meeting

TSG RAN WG1 Meeting #110	August 2022 			     Toulouse, FR