3GPP TSG RAN WG1 Meeting #108-e    	            R1-2202117

E-meeting, February 21st – March 3rd, 2022

Source:	Qualcomm Incorporated

Title:	Clarification on TPC Command Handling

Agenda Item:	7.1

Document for: 	Discussion and Decision

# Background

In the last RAN1 meeting, based on discussions on DMRS bundling and TPC command deferral there appears to be insufficient clarity on how the timeline pertaining to TPC command accumulation is to be applied across different repetitions of a DG-PUSCH. In particular it is not clear how the time reference KPUSCHi is to be computed for the second and subsequent repetitions of a DG-PUSCH.

Further, there is no clarity on how this time reference is to be computed for the case where the UE is configured for absolute TPC commands. The specification makes no mention of this currently.

In this document we aim to address this issue and propose a text proposal targetting Release 16 specification.

# Issues and Proposals

There are two issues to address with regard to how TPC commands are received and applied by a UE. A UE may either be configured to accumulate TPC commands or treat them individually and interpret them as indicating an absolute TPC update.

When a UE is configured to accumulate TPC commands, the duration over which the UE accumulates and applies the TPC update seems unclear for certain cases with PUSCH repetition. In particular, its not clear clear how the time reference KPUSCHi is to be computed for the second and subsequent repetitions of a DG-PUSCH.

To fix this issue, the time reference KPUSCHi for second and subsequent repetitions is clarified to follow the procedure used for CG-PUSCH --- a virtual DCI using K2min is used to derive KPUSCHi.

The second issue pertains to the case when a UE is configured for absolute TPC commands. In this case, the specification does not provide a clear timeline for application of TPC updates.

To fix this issue, for  the case when UE is configured for absolute TPC commands, the procedure used to compute the time reference KPUSCHi  for the case of accumulated TPC commads is reused.

If these issues are not clarified, all repetitions of a DG-PUSCH may be transmitted at the same power level. TPC commands received between repetitions of a DG-PUSCH may not be applied in a timely manner. UE behavior for absolute TPC commands received via group common DCI Format 2_2 remains undefined.

In order to address the above issues, we propose to adopt the following text proposal TPs on 38.213:

### 7.1.1 UE behaviour < Unchanged part is omitted > - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where - The values are given in Table 7.1.1-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUSCH transmission occasion and symbols before PUSCH transmission occasion on active UL BWP of carrier of serving cell for PUSCH power control adjustment state , where is the smallest integer for which symbols before PUSCH transmission occasion is earlier than symbols before PUSCH transmission occasion - If a PUSCH transmission is scheduled by a DCI format and is the first repetition of PUSCH Repetition Type A or PUSCH Repetition Type B, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i. - If a PUSCH transmission is scheduled by a DCI format and is the second or subsequent repetition of PUSCH Repetition Type A or PUSCH Repetition Type B, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If a PUSCH transmission is configured by ConfiguredGrantConfig, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP of carrier of serving cell - If the UE has reached maximum power for active UL BWP of carrier of serving cell at PUSCH transmission occasion and , then - If UE has reached minimum power for active UL BWP of carrier of serving cell at PUSCH transmission occasion and , then - A UE resets accumulation of a PUSCH power control adjustment state for active UL BWP of carrier of serving cell to - If a configuration for a corresponding value is provided by higher layers - If a configuration for a corresponding value is provided by higher layers where is determined from the value of as - If and the UE is provided higher SRI-PUSCH-PowerControl, is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to - If and the UE is not provided SRI-PUSCH-PowerControl or j=0, - If , is provided by the value of powerControlLoopToUse - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1 - δPUSCH,b,f,ci,l is a TPC command value that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format and is the first repetition of PUSCH Repetition Type A or PUSCH Repetition Type B, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i. - If a PUSCH transmission is scheduled by a DCI format and is the second or subsequent repetition of PUSCH Repetition Type A or PUSCH Repetition Type B, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c < Unchanged part is omitted >

# Conclusion

In this contribution, we discussed the issue of TPC command handling when a UE transmits multiple PUSCH repetitions and also on the timeline for TPC updates when the UE is configured to receive absolute TPC updates. We also propose to update Rel-16 specs using the suggested text proposal in Section 2.

# Reference

- 3GPP TS 38.213