3GPP TSG-RAN1 Meeting # 108-e	R1- 2202119

# E-Meeting,  February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft] CR on Receiving TDD-pattern bits in a dedicated or FDD band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current specification text states that the transmitter UE sets the bit sequence a0, a1, a2, a3, …, a11 in sl-TDD-Config in sidelink MIB to all 1’s in paired spectrum or PC5-only spectrum. However, there is no mention of receiver behavior, leaving possible the interpretation that the receiver can assume the values are all 1’s and not decode them, which would lead to loss of forward compatibility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update text to state that the receiver UE assumes bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload can have any value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Loss of forward compatiblity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------begin text proposal for 38.213-----------------------------------------------------

## 16.1	Synchronization procedures

<<<<<unchanged text omitted>>>>>

For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1],

-	a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1';

- The receiver UE assumes that a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 can have any value.

------------------------------------------------------end text proposal for 38.213------------------------------------------------------