3GPP TSG RAN WG1 Meeting #108-e	R1- 2202122

February 21st – March 03rd, 2022

Agenda item:	8.1.1

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Multi-beam Operation

Document for:	Discussion/Decision

- Introduction

The following agreements have been achieved for multi-beam operation in RAN1 #107-e.

Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, Rel-15 L1-RSRP reporting format is reused for all SSBRI-RSRP pairs in one L1-RSRP reporting instance, i.e. for K>1, (K-1) 4-bit differential L1-RSRP(s) calculated relative to the reference (absolute) 7-bit L1-RSRP Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI, when a UE is configured with separate DL/UL TCI - The number of configured TCI states a UE can support is a UE capability including the following candidate values per BWP per CC: - DL TCI: 64, 128 - UL TCI: 32, 64 - Note: This doesn’t imply that UL TCI shares the same TCI state pool as or uses a different TCI state pool from joint DL/UL TCI. Agreement On Rel-17 unified TCI framework, for intra-cell beam management, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for all PDSCH /PDCCH receptions in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PDSCH /PDCCH reception. - The above applies to Rel-15 SpCell BFR , Rel-16 CBRA based SpCell BFR , and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams Agreement On Rel-17 unified TCI framework, after X symbols from the UE receives the BFRR from NW, the UE uses the same UL spatial filter as the one associated with the index q new or the last PRACH transmission for all PUSCH transmissions and all of PUCCH resources in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PUSCH and all of PUCCH resources. - The above applies to Rel-15/16 SpCell BFR , Rel-16 CBRA based SpCell BFR, and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams - FFS : UL PC control including q u , q d , and closed loop index Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, in Rel-17, there is no consensus on supporting event-driven beam reporting Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the UE can assume that one beam application time (BAT) for a given SCS is configured for all the CCs configured with the common TCI state ID update, - Note: It was agreed that the BAT associated with the carrier(s) (hence BWP(s)/CC(s)) on which the beam indication applies is determined based on the carrier with the smallest SCS among the carrier(s) (hence BWP(s)/CC(s)) applying the beam indication - TBD (maintenance): whether a second configured BAT is also supported, e.g. for MPUE or inter-cell BM - The detailed signaling of the BAT is up to RAN2 - FFS: For CC(s) not configured with a common TCI state ID update Conclusion On Rel.17 enhancements to facilitate MPE mitigation, there is no consensus on a specification-based criterion for selecting N from a candidate SSB/CSI-RS resource pool Agreement With regards to the below question in RAN2 LS, provide the following response. If UE is receiving DL data from TRP with different PCI on dedicated channels, is the UE still able to receive short message (e.g. paging) and system information from serving cell TRP at the same time?. Answer: No, it is not. LS is endorsed in R1-2112707. Working Assumption For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception, for intra-cell and inter-cell BM, support per CORESET determination as follows: - For any PDCCH reception on a CORESET [other than CORESET#0] that is associated with [at least or only] [USS and/or CSS type 3] set(s) and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP , in Rel-17, there is no consensus that the agreed L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP , in Rel-17, on the UE behavior when there is overlap for L1-RSRP measurement for SSB associated with serving cell PCI and PCIs different from the serving cell PCI - Prepare an LS to RAN4 informing and recommending RAN4 to investigate the issue - Note: Discussion in UE feature agenda on this issue is not ruled out LS to RAN4 is endorsed in R1-2112762. Agreement On Rel-17 unified TCI framework, any SRS resource or resource set that is a valid target signal of a Rel-15/16 spatial relation based on the Rel-15/16 spatial relation rules (on source-target relations) can be configured as a target signal of a Rel-17 UL or, if applicable, joint TCI (hence the Rel-17 UL or, if applicable, joint TCI state pool). - This feature does not require a new type of source RS on top of the ones supported for UL TCI and joint DL /UL TCI - Note: This doesn’t preclude the content of pending proposal 1.E (on CSI -RS for CSI as QCL Type-A/D source RS) to be agreed in the future - Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL /UL TCI (this issue is up to RAN2) - Note: A Rel-17 UE is not required to support both this feature and optional Rel-16 features of SRS spatial relation info within a same band Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI Agreement For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: - For discussion purposes, define as follows: - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [USS and/or CSS Type 3] - ‘CORESET B’: A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [CSS or CSS other than Type 3] - ‘CORESET C’: A CORESET other than CORESET#0 associated with both UE-dedicated and non-UE-dedicated reception on PDCCH in a CC - CORESET#0 - For Rel-17 TCI state indication, support per CORESET determination as follows: - For any PDCCH reception on a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC - FFS: For intra-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: For inter-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: The TCI state of CORESET 0 Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Agreement On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance. - The Rel-15/16 beam reporting framework is used, i.e. the index of corresponding UE capability value set is reported along with the pair of SSBRI/CRI and L1-RSRP/SINR (up to 4 pairs, with 7-bit absolute and 4-bit differential) in the beam reporting UCI and down select (maintenance) between the following two options: - Option 1: UE can report one index for all the reported CRIs/SSBRIs in one beam reporting - Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting. - FFS: whether/how to take DL-only panel into account in the report - FFS: Time-domain behaviour, e.g. the support periodic, semi-persistent, and aperiodic reporting - FFS: Semi-persistent and/or aperiodic reporting is triggered only when periodic reporting is configured - (Working assumptions): Support acknowledgement mechanism of the reported correspondence from NW to UE, which doesn't preclude reusing/reinterpreting existing signaling/procedure - FFS (maintenance): the application time for the reported correspondence (if any), the exact acknowledgement mechanism and whether spec impact is needed, e.g. based on TCI state update, BFR response like mechanism, including the application time for the reported correspondence, if any - No new DCI format and no new RNTI are introduced for this function. Conclusion On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, other than the max supported number of SRS ports (note: currently pending endorsement in proposal 4.A), there is no consensus on supporting another UE capability type

- Issue 1 – Unified TCI Framework

In RAN1 #107e, it has been agreed that after the BFR response is received, the UL beam for channels/RSs sharing the same indicated TCI as PUCCH/PUSCH is reset to the PRACH beam in case of RACH based BFR or the UL beam indicated by q_new in case of SCell BFR. For the PC parameters associated with the new UL beam, the PL RS can simply reuse the q_new, while other PC parameters can reuse the default P0, alpha, and closed loop index for each of the PUCCH, PUSCH, and SRS, respectively.

Proposal 1: 28 symbols after the UE receives the SpCell or SCell BFR response, for all UL channels/RSs sharing the same indicated UL or joint TCI state as PUSCH and PUCCH, the corresponding PC parameters are determined as below

- The PL RS is set as q_new for all above UL channels/RSs

- For each of PUCCH, PUSCH, SRS, other PC parameters are set as the corresponding default P0, alpha, and closed loop index values.

Another issue is whether the indicated TCI for dedicated PDCCH/PDSCH should also be applied to CORESET Type C, which is associated with both USS and CSS. To align with previous agreements, the determination can be per SS type. Specifically, for search space classified as USS, the corresponding PDCCH/PDSCH reception shares the same indicated TCI as UE-dedicated PDCCH/PDSCH, while for search space classified as CSS, whether to apply the same indicated TCI can be determined by an RRC flag associated with this CORESET.

Proposal 2: For Type-C CORESET, whether to share the indicated TCI state depends on the search space type

- For search space classified as USS, the corresponding PDCCH/PDSCH reception shares the same indicated TCI as UE-dedicated PDCCH/PDSCH

- For search space classified as CSS, whether to apply the same indicated TCI is determined by an RRC flag associated with this CORESET.

As for the default PDSCH beam, the R15/16 rule is currently reused for unified TCI. In case of different CORESET beams, the default beam follows the beam of the lowest CORESET ID in latest monitored slot and hence can vary across slots. This may not be desirable if the followed CORESET beam is a wide beam not optimized for the SNR. Similar issue exists in case of cross-carrier scheduling, where the default beam of the scheduled CC follows the active TCI state for PDSCH with lowest ID on that CC, which may not be the indicated TCI state for that CC. Given that the indicated TCI state for dedicated PDCCH/PDSCH is most likely a refined beam, it would be beneficial to reuse the indicated TCI state as the default beam, at least when instructed by gNB.

Proposal 3: For both self scheduling and cross-carrier scheduling, support default beam for PDSCH/AP CSI-RS can be indicated to share the same indicated TCI as UE-dedicated PDCCH/PDSCH for the scheduled CC.

In addition, for the DL RSs potentially transmitted with same beam as data/control, they can also share the same indicated DL or joint TCI as UE dedicated PDSCH/PDCCH, e.g. BFD/RLM/PL RS.

Proposal 4: BFD/RLM/PL RS can share the indicated DL or joint TCI state as UE-dedicated PDSCH/PDCCH.

In RAN1 #105e, it has been agreed to introduce UE capability on support of beam misalignment between UL beam indicated by the UL or joint TCI state and Rx beam of the associated PL RS. In RAN1 #106e, the beam alignment is defined at least as the event when the PL RS is identical to the spatial RS in the UL or joint TCI state. However, there are other conditions under which the Rx beam of PL RS is identical to the indicated Tx beam. Those conditions may include: (1) the PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS; (2) the QCL Type-D RS of PL-RS is identical to the UL TCI spatial relation RS; (3) the QCL Type-D RS of PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS. It would be beneficial to define the UE capability accordingly for aligned understanding at both gNB and UE, and gNB should ensure one of the “beam alignment” conditions is satisfied if UE indicates not supporting the “beam misalignment”.

Proposal 5: The “beam alignment” in the UE capability of “beam misalignment” is defined as the following additional events:

- The PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS.

- The QCL Type-D RS of PL-RS is identical to the UL TCI spatial relation RS.

- The QCL Type-D RS of PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS.

In R15/16, UE will reset the PUSCH PC adjustment state whenever P0 or alpha is reconfigured. In RAN1 #105e, it is agreed that PC parameters can be associated with an UL or joint TCI state. A similar issue could be whether UE should reset the PC state if the associated PC parameters are updated for an UL or joint TCI state used for UL transmission. It would be beneficial to clarify the corresponding UE behaviour.

38.213:

Proposal 6: Clarify whether the corresponding PC adjustment state should be reset when the associated PC parameters are updated for an UL or joint TCI state used for UL transmission.

Another remaining issue for the common TCI state pool is the determination of PC parameters used by the selected common TCI state ID on individual target CC. For the PC parameters except for the PL RS, they can reuse those associated with the selected common TCI state ID on the reference CC. For the PL RS, it can be the same PL RS for the common TCI state ID on the reference CC or the PL RS configured on the target CC with the same RS ID as the PL RS on the reference CC. Similar to R15, which option to be used can be configured by gNB.

Proposal 7: For a common TCI state pool shared by multiple CCs, the PC parameters for a selected common TCI state ID on each target BWP/CC can be determined as below

- For the PC parameters except for the PL RS, they can reuse those for the selected common TCI state ID on the reference BWP/CC.

- The used PL RS can have the following two alternatives, which can be selected by gNB

  - Alt1: Use the same PL RS for the selected common TCI state ID on the reference BWP/CC.

  - Alt2: Use the PL RS configured on the target BWP/CC with the same RS ID as the PL RS for the selected common TCI state ID on the reference BWP/CC.

In R15, the TCI state of periodic CSI-RS and the spatial relation of periodic SRS have to be updated via RRC. To save RRC reconfiguration latency, the joint DL/UL TCI state of periodic CSI-RS and SRS can be dynamically updated via

Proposal 8: Joint DL/UL TCI state of periodic CSI-RS and SRS can be updated by MAC-CE or DCI.

- Issue 2 – L1/L2 Centric Inter-Cell Mobility

In addition, the baseline assumption for L1 measurement over SSBs from different PCIs should be TDM based measurement, i.e. SSBs configured for L1-RSRP measurement should be at non-overlapping occasions to facilitate UE to use corresponding refined Rx beam at each time. However, the TDM based measurement may require staggering SSB bursts from different PCIs in time and, hence, the L1 measurement latency will scale up as the number of measured PCIs increases, e.g. 40 ms period per PCI for two measured PCIs, 60 ms period per PCI for three measured PCIs, and so on. This may defeat the purpose of fast cell switching based on timely L1 report. On the other hand, the measurement latency may not scale up if the UE is capable of measuring overlapped SSBs, which can be feasible at least for some UEs. Therefore, it would be beneficial to introduce UE capability for supporting measurement of SSBs overlapped in time to reduce the measurement latency. It should be emphasized that the there is no newly introduced constraint on the SSB burst transmission in inter-cell BM.

Proposal 9: For L1-RSRP measurement in inter-cell BM, introduce UE capability for supporting measurement of SSB burst sets from different PCIs that overlapped in time.

- The maximum number of overlapped SSB burst sets configured for L1-RSRP measurement is up to UE capability.

- For UEs not supporting such capability, the SSB burst sets configured for L1-RSRP measurement should not overlap in time.

- There is no newly introduced constraint on the SSB burst set transmission in inter-cell BM.

In RAN1 #106e, it has been clarified that for UE only supporting 1 active TCI state, MAC-CE based beam indication (activation of one TCI state) can be used to switch between common and dedicated channels on serving and non-serving cells, respectively. However, a large number of MAC-CEs may have to be sent if a large number of UEs are expected to receive the common channel, e.g. for system info update. One way to mitigate the MAC-CE overhead and latency is to introduce a pre-determined beam switch pattern for UE to periodically switch to the common PDCCH channel for potential control info and then switch back to the dedicated channel.

Proposal 10: At least for UE only supporting 1 active TCI state, the UE can be configured to preform pre-determined periodic beam switch between the common channel on serving cell and the dedicated channel on the non-serving cell.

- Issue 3 – Dynamic TCI Update Signalling

In RAN1 #104bis-e, it is agreed that RV field of all ‘1’s is used to differentiate DCI format 1_1/1_2 for TCI update without DL assignment from other use cases. However, the RV field may be configured with 0 bit by higher layer parameter numberOfBitsForRV-DCI-1-2. In this case, the rvid is assumed to be 0, which is different from the all ‘1’s used to identify the DCI format for TCI update. Therefore, the RV field should always present for DCI format 1_2 for TCI update and without DL assignment.

Proposal 11: For DCI format 1_2 for TCI update and without DL assignment, the RV field should always present.

In case of Type-1 HARQ-ACK codebook, one issue for the TCI updating DCI without DL assignment is that the indicated virtual PDSCH occasion may not fall into the candidate K1 window of the indicated PUCCH resource carrying the codebook. For example, suppose the candidate K1 set includes two candidate values of K1={2, 4}, TCI updating DCI without DL assignment is sent in slot #n, the indicated virtual PDSCH occasion is in slot #n+1, and the indicated PUCCH for A/N is in slot #n+2. In this case, the Type-1 codebook will only consider PDSCH occasions in slot #n and #n-2, and will not consider the indicated virtual PDSCH occasion in slot #n+1. Therefore, there is no valid A/N bit position for the TCI updating DCI, which is based on the virtual PDSCH occasion. A simple solution is to clarify that the indicated virtual PDSCH occasion must be in the candidate K1 window, i.e. the value of K1-K0 shall be among the “set of slot timing values K1” as defined in Section 9.1.2.1 of 38.213, where the K1 in the “K1-K0” refers to the HARQ-ACK timing indicated in the DCI and determines the time offset between the DCI and the PUCCH, while the indicated K0 in the “K1-K0” determines the time offset between the DCI and the virtual PDSCH occasion.

Proposal 12: For Type-1 HARQ-ACK codebook, if TDRA of DCI indicates K0 for the virtual PDSCH and the DCI indicates a value K1 for HARQ-ACK timing, then the value of K1-K0 shall be among the “set of slot timing values K1” as defined in Section 9.1.2.1 of 38.213.

In R16, the application time for a PL RS activated by the MAC-CE is defined as 3 ms after the ACK for the MAC-CE if the PL RS has been maintained by the UE. In RAN1 #105e, it is agreed that the application time for the DCI indicated beam indication is X ms after the ACK for the DCI. However, the application time for the PL RS associated with the beam indication still needs to be clarified. In addition, it is agreed that UE shall maintain the PL RS associated with each activated UL or joint TCI state. Therefore, a simple clarification could be that the application time for the associated PL RS can be the same as that for the beam indication.

Proposal 13: The application time for the PL RS associated with an updated beam indication by the DCI can be the same as that for the updated beam indication.

Furthermore, additional application time should be added when the SCS of the TCI updating DCI is smaller than that of applied channel(s) to account for extra DCI decoding latency. The additional application time may reuse the additional beam switching delay defined for cross-carrier AP CSI-RS triggering with different numerologies.

Proposal 14: If the TCI updating DCI has smaller SCS than the applied channel(s), the time gap between DCI and the application time should be no less than the corresponding UE capability plus an additional value to account for extra DCI decoding latency.

- Value may reuse the additional beam switching timing delay d defined in 38.214 Table 5.2.1.5.1a-1.

As agreed in RAN1 #107e, in case of common TCI ID applied to multiple BWPs/CCs, the SCS of the beam application time is determined based on the smallest SCS of all applied BWPs/CCs. A remaining issue is the determination for the SCS of the beam application time in case of per-CC based TCI update. We believe same rule for multiple CCs can be extended to single CC, i.e. the SCS of the beam application time is the smallest SCS of the applied DL BWP and/or UL BWP of that CC.

Proposal 15: For a CC not configured with a common TCI state ID update, the beam application time is based on the smallest SCS of the applied DL BWP and/or UL BWP of that CC.

- Issue 4 – Fast UL Panel Selection

As a remaining issue of the SRS port number carried in the beam report, there are two options to be down selected in this meeting. Option 1 is to report a single common SRS port number for all reported DL RSs in the beam report, while Option 2 is to report an SRS port number per reported DL RS in the beam report. We prefer Option 2, which essentially allows UE to associate different panels with different DL RSs. In case of Option 1, the remaining DL RSs except for the best one may have worse RSRP if they are enforced to map to the same panel as the best DL RS. To learn the best RSRP per DL RS, gNB may have to schedule multiple reports, which may cause more overhead than Option 2. In addition, even if only a single report is sent, the overhead saving of Option 1 is marginal, since at most 2-bit SRS port number is needed per DL RS. The corresponding overhead increase should not be significant compared with existing bits for the DL RS ID and corresponding RSRP.

Proposal 16: For the report of SRS port number index, support UE to report one index per reported CRI/SSBRI in one beam report.

In RAN1 #107e, WA has been made to consider acknowledgement for the beam report carrying the SRS port number. In our view, this UCI reliability issue should be fundamentally addressed by introducing UCI retransmission mechanism, which can also serve as implicit acknowledgement for the UCI report. For example, DCI can request retransmission of a beam report for a particular reportConfigID. If no retransmission is requested within X ms after the beam report, UE may assume that the beam report has been successfully received.

Proposal 17: For the acknowledgement of the beam report, introduce UCI retransmission mechanism, e.g. DCI can request retransmission of a beam report for a particular reportConfigID

- If no retransmission is requested within X ms after the beam report, the beam report is implicitly acknowledged.

- Issue 5 – UL Beam Selection Considering MPE

For event triggered enhanced P-MPR report, the failed UL beam can be quickly recovered by resetting the UL beam to the reported UL candidate beam after UE receives gNB’s response to the report. For example, UE can send the P-MPR report via MAC-CE on the PCell in FR1 and, after receiving the gNB’s response on PCell, both sides will reset the UL beam on the SCell suffering from the MPE issue in FR2.

Proposal 18: Support UL beam resetting based on the reported UL beam in the enhanced P-MPR report, after UE receives gNB’s response to the report.

- Conclusion

In this contribution we discussed enhancements for Rel-17 NR FeMIMO beam management. Below are the observations and proposals.

Proposal 1: 28 symbols after the UE receives the SpCell or SCell BFR response, for all UL channels/RSs sharing the same indicated UL or joint TCI state as PUSCH and PUCCH, the corresponding PC parameters are determined as below

- The PL RS is set as q_new for all above UL channels/RSs

- For each of PUCCH, PUSCH, SRS, other PC parameters are set as the corresponding default P0, alpha, and closed loop index values

Proposal 2: For Type-C CORESET, whether to share the indicated TCI state depends on the search space type

- For search space classified as USS, the corresponding PDCCH/PDSCH reception shares the same indicated TCI as UE-dedicated PDCCH/PDSCH

- For search space classified as CSS, whether to apply the same indicated TCI is determined by an RRC flag associated with this CORESET.

Proposal 3: For both self scheduling and cross-carrier scheduling, support default beam for PDSCH/AP CSI-RS can be indicated to share the same indicated TCI as UE-dedicated PDCCH/PDSCH for the scheduled CC.

Proposal 4: BFD/RLM/PL RS can share the indicated DL or joint TCI state as UE-dedicated PDSCH/PDCCH.

Proposal 5: The “beam alignment” in the UE capability of “beam misalignment” is defined as the following additional events:

- The PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS.

- The QCL Type-D RS of PL-RS is identical to the UL TCI spatial relation RS.

- The QCL Type-D RS of PL-RS is identical to the QCL Type-D RS of UL TCI spatial relation RS.

Proposal 6: Clarify whether the corresponding PC adjustment state should be reset when the associated PC parameters are updated for an UL or joint TCI state used for UL transmission.

Proposal 7: For a common TCI state pool shared by multiple CCs, the PC parameters for a selected common TCI state ID on each target BWP/CC can be determined as below

- For the PC parameters except for the PL RS, they can reuse those for the selected common TCI state ID on the reference BWP/CC.

- The used PL RS can have the following two alternatives, which can be selected by gNB

  - Alt1: Use the same PL RS for the selected common TCI state ID on the reference BWP/CC.

  - Alt2: Use the PL RS configured on the target BWP/CC with the same RS ID as the PL RS for the selected common TCI state ID on the reference BWP/CC.

Proposal 8: Joint DL/UL TCI state of periodic CSI-RS and SRS can be updated by MAC-CE or DCI.

Proposal 9: For L1-RSRP measurement in inter-cell BM, introduce UE capability for supporting measurement of SSB burst sets from different PCIs that overlapped in time.

- The maximum number of overlapped SSB burst sets configured for L1-RSRP measurement is up to UE capability.

- For UEs not supporting such capability, the SSB burst sets configured for L1-RSRP measurement should not overlap in time.

- There is no newly introduced constraint on the SSB burst set transmission in inter-cell BM.

Proposal 10: At least for UE only supporting 1 active TCI state, the UE can be configured to preform pre-determined periodic beam switch between the common channel on serving cell and the dedicated channel on the non-serving cell.

Proposal 11: For DCI format 1_2 for TCI update and without DL assignment, clarify that the RV field should always present.

Proposal 12: For Type-1 HARQ-ACK codebook, if TDRA of DCI indicates K0 for the virtual PDSCH and the DCI indicates a value K1 for HARQ-ACK timing, then the value of K1-K0 shall be among the “set of slot timing values K1” as defined in Section 9.1.2.1 of 38.213.

Proposal 13: The application time for the PL RS associated with an updated beam indication by the DCI can be the same as that for the updated beam indication.

Proposal 14: If the TCI updating DCI has smaller SCS than the applied channel(s), the time gap between DCI and the application time should be no less than the corresponding UE capability plus an additional value to account for extra DCI decoding latency.

- Value may reuse the additional beam switching timing delay d defined in 38.214 Table 5.2.1.5.1a-1.

Proposal 15: For a CC not configured with a common TCI state ID update, the beam application time is based on the smallest SCS of the applied DL BWP and/or UL BWP of that CC.

Proposal 16: For the report of SRS port number index, support UE to report one index per reported CRI/SSBRI in one beam report.

Proposal 17: For the acknowledgement of the beam report, introduce UCI retransmission mechanism, e.g. DCI can request retransmission of a beam report for a particular reportConfigID

- If no retransmission is requested within X ms after the beam report, the beam report is implicitly acknowledged.

Proposal 18: Support UL beam resetting based on the best reported UL candidate beam in UE triggered P-MPR report, after UE receives gNB’s response to the report.

- References

- R1-2007151 R17 ReMIMO EVM final