3GPP TSG RAN WG1 #108-e	R1-2202129

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.2.1

Source: 	Qualcomm Incorporated

Title: 	Initial access aspects for NR to support operation between 52.6 GHz and 71 GHz

Document for:	Discussion/Decision

# Introduction

In RAN #90e meeting, a Rel-17 work item for NR operation in a frequency regime between 52.6 GHz and 71 GHz was approved, and the WID was further revised in RAN #94e [1]. The work item phase in RAN1 was completed in RAN1 #107e meeting and the status report (SR) was captured in RAN #94e [2].

At the end of RAN1 #107e, most of the RAN1 related design aspects were completed with few pending working assumptions that depended on other RAN WGs feedback/input for their completion, namely:

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used Final LS endorsed in R1-2112805.

Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET,NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration. Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET,NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design.

In RAN1 #107b-e, further maintenance work was done and the final email discussion document in [3].

In this contribution, we present our views on the remaining topics for initial access aspects for NR operation in the frequency regime between 52. 6GHz and 71 GHz.

# Discussion

## Q (NSSBQCL) Values

The following agreement was made in RAN1 #107b-e:

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

However, in RAN2 #116bis-e, the following agreement was made:

A1: RAN2 does not agree to using the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. Respond to RAN1 LS accordingly. A2: The legacy MIB is used for FR2-2 (i.e. we do not define new MIB for FR2-2). A3: Up to RAN1 how to resolve QCL configuration (no suggestions from RAN2). This need not be included in LS to RAN1.

Hence, per the RAN1 agreement “The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used”, a revisit is needed.

For FR2, due to the highly directive nature of the beams, we believe that the probability of LBT failures will be very low and the hence there may not be a need to support a large number of candidate SSB positions per QCL in a DBTW. With this, we think that 2 candidate SSB positions per QCL in a DBTW is sufficient. Hence, only 1 bit is needed to signal the NSSBQCL.

In addition to this argument, a 2nd bit may not be readily available from the MIB. A possibility is reusing one bit from ‘controlResourceSetZero’ leading to 8 possible rows/configurations in the table that can be used for CORESET#0. Per the agreements from RAN1 #107e, the priorities for this table are: multiplexing pattern 1 (24/48 RBs)  multiplexing pattern 3  multiplexing pattern 1 (96 RBs). Given that multiple configurations are prioritized, it is most likely that 8 rows will not be sufficient and an extra bit from ‘controlResourceSetZero’ will not be available.

From RAN1 #106b-e, the following agreement was made:

Agreement: For 120kHz SCS, for NSSBQCL values: - If 2 bits are available in MIB for NSSBQCL, at least support {16, 32, 64} - If 1 bit is available in MIB for NSSBQCL, support {32, 64} - …

And the following in RAN1 #107-e:

Agreement - Same values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - …

Given the above, we propose the following:

Proposal 1: for 120/480/960 kHz SCS, consider using only 1 bit in MIB to indicate the NSSBQCL, where:

- NSSBQCL  {32, 64}

- ‘subCarrierSpacingCommon’ is used to signal the 1 bit for NSSBQCL

The UE regardless of the network being licensed or unlicensed needs to know how to interpret the Q value in either case since the UE does not know if it is licensed or unlicensed until later in RMSI. Hence, we propose the following TP:

Proposal 2: consider adopting the following TP for TS 38.213:

Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved.

## DBTW and Q Value for Licensed Operation

In RAN1 #107b-e, some concerns were risen about the UE assumptions for DBTW and the Q value for a licensed operation network and how to capture this in the specifications. After few rounds of discussions in [3], the following was the proposed TP (not agreed in light of the RAN2 spare bit agreement):

4.1 Cell search *** unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. TP# 1-9a for TS38.213: For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ and spare = ‘0’ from a MIB provided by a SS/PBCH block. TP# 1-9b for TS38.213: For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ and spare = ‘0’. from a MIB provided by a SS/PBCH block. *** unchanged part omitted ***

In RAN1 #107e, the following agreement was made:

Agreement - … - UE is expected to be configured with NSSBQCL= 64 in licensed operations

Given the RAN2 agreement above about not using the spare bit in the MIB signalling the Q value, and considering Proposal 2, we propose the following TP:

Proposal 3: consider adopting the following TP for TS 38.213:

4.1 Cell search *** unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block. *** unchanged part omitted ***

## CORESET#0 Configuration

In RAN4 # 101b-e, an LS was sent to RAN1 [4] indicating the following:

Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz - FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands In order to facilitate discussions in RAN1, RAN4 would like to also note for licensed operations the minimum granularity, RAN4 is considering 3 x 17.28 MHz for 120 kHz, 12 x 17.28 MHz for 480 kHz and 6 x 17.28 MHz for 960 kHz. RAN1 may use this information for further discussions.

Clearly, RAN4 still did not conclude on the sync raster. Also, RAN4 concluded to use fixed sync channel raster for the unlicensed spectrum with an FFS for trying to use a sub-set of sync raster for licensed bands.

Hence, we still cannot have a final design for the CORESET0 RB offsets unless RAN4 concludes on the channelization design. However, in this contribution, we present our views for general guidelines that can be used in the future once RAN4 concludes the design.

From previous agreements, the following {RB, symbol} combinations are assumed:

| SCS = 120 kHz​ |  |  |
| --- | --- | --- |
| Multiplexing pattern ​ | Number of RBs ​ | Number of Symbols |
| 1 ​ | 24​ | 2​ |
| 1 ​ | 48​ | 1​ |
| 1 ​ | 48​ | 2​ |
| 3​ | 24​ | 2​ |
| 3​ | 48​ | 2​ |
| 1 [WA]​ | 96​ | 1​ |
| 1 [WA]​ | 96​ | 2​ |

| SCS = 480kHz and 960 kHz​ |  |  |
| --- | --- | --- |
| Multiplexing pattern ​ | Number of RBs ​ | Number of Symbols |
| 1 [Priority 1]​ | 24​ | 2​ |
| 1 [Priority 1]​ | 48​ | 1​ |
| 1 [Priority 1]​ | 48​ | 2​ |
| 3 [Priority 2]​ | 24​ | 2​ |
| 3 [Priority 2]​ | 48​ | 2​ |
| 1 [Priority 3]​ | 96​ | 2​ |

In Rel-15, for 120/120 kHz, the RB offset design principles were such that (blue components in Figure 1):

- For multiplexing pattern 1:

  - For 24 RBs: 2 configs aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB

  - For 48 RBs: 1 config aligning the center of the CORESET 0 with the center of the 20 RB SSB

- For multiplexing pattern 3:

  - For 24 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

  - For 48 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

For 480/960 kHz, we can reuse the same design principles used for Rel-15 120 kHz.

For multiplexing pattern 1 and 96 RB CORESET0, we can reuse the Rel-15 120 kHz principle for 48 RBs as shown in the orange component of Figure 1 (i.e., aligning the center of the CORESET 0 with the center of the 20 RB SSB).

For multiplexing pattern 1 and 48 RB CORESET0, since the BW is now increased, we can also consider using the Rel-15 120 kHz principle for 24 RBs as shown in the orange component of Figure 1 (i.e., aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB).

These design principles may be made applicable to both licensed and unlicensed cases with floating and fixed raster, respectively.

Figure 1: SSB/CORESET0 Multiplexing Options

Proposal 4: consider adopting the following design principles (as much as possible) for CORESET0 RB offsets for both licensed and unlicensed operation for SCS = 120 kHz, 480 kHz, and 960 kHz (per # of symbols):

- For multiplexing pattern 1:

  - 24 RBs: 2 configs aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB

  - 48 RBs: 1 config aligning the center of the CORESET 0 with the center of the 20 RB SSB

  - 96 RBs: 1 config aligning the center of the CORESET 0 with the center of the 20 RB SSB

  - 48 RBs (open for discussion): 2 configs aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB

- For multiplexing pattern 3:

  - 24 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

  - 48 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

# Conclusions

In this contribution, we presented our views on some of the remaining issues for initial access aspects for NR operation in the frequency regime between 52.6 GHz and 71 GHz.

The following proposals were made:

Proposal 1: for 120/480/960 kHz SCS, consider using only 1 bit in MIB to indicate the NSSBQCL, where:

- NSSBQCL  {32, 64}

- ‘subCarrierSpacingCommon’ is used to signal the 1 bit for NSSBQCL

Proposal 2: consider adopting the following TP for TS 38.213:

Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved.

Proposal 3: consider adopting the following TP for TS 38.213:

4.1 Cell search *** unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block. *** unchanged part omitted ***

Proposal 4: consider adopting the following design principles (as much as possible) for CORESET0 RB offsets for both licensed and unlicensed operation for SCS = 120 kHz, 480 kHz, and 960 kHz (per # of symbols):

- For multiplexing pattern 1:

  - 24 RBs: 2 configs aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB

  - 48 RBs: 1 config aligning the center of the CORESET 0 with the center of the 20 RB SSB

  - 96 RBs: 1 config aligning the center of the CORESET 0 with the center of the 20 RB SSB

  - 48 RBs (open for discussion): 2 configs aligning CORESET 0 edges with the upper and lower edges of the 20 RB SSB

- For multiplexing pattern 3:

  - 24 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

  - 48 RBs: 2 configs placing the CORESET 0 at either side of the 20 RB SSB

# References

[1] RP-213540, “Revised WID: Extending current NR operation to 71GHz”, RAN #94e, Qualcomm

[2] RP-212990, “Status Report to TSG”, RAN1 #94e, Qualcomm; Intel Corporation

[3] R1-2200690, “Summary #2 of email discussion on initial access aspect of NR extension up to 71 GHz”, RAN1 #107bis-e, Moderator (Intel Corporation)

[4] R4-2200081, “LS for the channelization for up to 71 GHz”, RAN4 #101bis-e