3GPP TSG RAN WG1 #108-e			R1-2202138

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.4.2

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on UL Time and Frequency Synchronization for NTN

Document for:	Discussion/Decision

# Introduction

In RAN1#107-e, a working assumption was made on TA range for Msg2/MsgB [1]:

- Working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

Where, TA is the TAC field in msg2/msgB

In this contribution, we provide further discussion on the issue.

# Discussions

During initial access, a PRACH transmission may arrive earlier than the start of a PRACH occasion. Without the support of TA commands of negative values (i.e., delays), subsequent UE UL transmissions can all have negative timing until a TA command with a negative timing advance is received during connected mode. According to RAN4 LS R1-2200869 [2], the transmit timing error requirements for FR1 are established for both PRACH transmission and the first transmission in a DRX cycle.  The required timing accuracy is well within half of the CP. However, for FR2 or FR3, same accuracy requirement may lead to a timing error close to or larger than the CP. Further tightening of the requirement for PRACH transmission would mean unnecessarily tight requirement on GNSS accuracy.

Hence, without further delay of the specification and to avoid different specification on the subject between FR1 and FR2, negative NTA values in Msg2/MsgB should be supported.

Proposal 1: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as follows:

NTA=-256. 16.642μ+TA. 16.642μ  ,

where, TA is the TAC field in msg2/msgB.

# Conclusions

In this contribution, we have discussed the working assumption on TAC in Msg2/Msg4

Proposal 1: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as follows:

NTA=-128. 16.642μ+TA. 16.642μ  ,

where, TA is the TAC field in msg2/msgB.

# References

- Chairman’s notes, 3GPP RAN1#107-e.

- R1-2200869/R4-2120311, “Reply LS on NTN UL time and frequency synchronization requirements”, 3GPP RAN1#108-e.