3GPP TSG RAN WG1 Meeting #108-e       	                R1-2202146

E-meeting, February 21st – March 3rd, 2022

Source:	Qualcomm Incorporated

Title:	Remaining Issues on UE Complexity Reduction

Agenda Item:	8.6.1

Document for:	Discussion/Decision

# Introduction

In RAN1#107e meeting, RAN1 agreed on the transmission of NCD-SSB in the RRC-configured DL BWP of RedCap UE without CD-SSB. An LS on the use of NCD-SSB and CSI-RS in the DL BWP of RedCap UE was sent to RAN2 and RAN4 [1]. In RAN2#116bis-e and RAN4#101bis-e meetings, RAN2 and RAN4 discussed the use of NCD-SSB and CSI-RS in the DL BWP of RedCap UE,  and provided replies to the RAN1 LS in [2] and [3], respectively.

In this contribution, we provide our views on the remaining issues of UE complexity reduction for RedCap UE, and addressed the feedback of RAN2 and RAN4  [2-5].

# Discussion

In RAN1#107e meeting, it was agreed that on a cell that allows a RedCap UE to access, network can configure a separate initial DL or UL BWP for RedCap UEs, and the bandwidths for the separate initial BWP of RedCap UE can have any values up to the maximum BW of RedCap UE. If a DL BWP without CD-SSB/CORESET#0 is separately configured for random access of an RedCap UE in idle/inactive state, and the DL BWP is not configured with CSS sets for paging, RAN1 also agreed that the RedCap UE does not need to monitor paging when performing random access procedure in the RedCap-specific initial DL BWP.

On the other hand, if an initial DL BWP is not separately configured for RedCap UE in SIB, the RedCap UE in idle state will use MIB-configured CORESET#0 as the default initial DL BWP. If an initial DL BWP is not separately configured for RedCap UE in SIB or RRC,  a RedCap UE in inactive state will use MIB-configured CORESET#0 as the default initial DL BWP.  Therefore, we have the following proposal for RedCap UE:

Proposal 1: If a separate initial DL BWP is not configured for idle/inactive RedCap UE on  a cell that allows RedCap UE to access, the RedCap UE will use MIB-configured CORESET#0 as the default initial DL BWP in idle/inactive state.

For a NR R15/16 UE in connected state, NW has two options to configure the initial BWP (BWP#0) as specified in TS 38.331. For both options, CD-SSB and CORESET#0 are included in DL BWP#0 of the NR R15/16 UE. Based on the agreements of RAN1#107e, if the DL BWP#0 of a RedCap UE is configured by BWP-DownlinkDedicated, either CD-SSB or NCD-SSB will be transmitted in the DL BWP#0 of the RedCap UE. If the DL BWP#0 of a RedCap UE is configured by Option 1,  the presence of SSB (CD-SSB or NCD-SSB) depends at least on the configurations of CORESET#0 as well as the RedCap-specific initial DL BWP indicated by BWP-DownlinkCommon in DownlinkConfigCommonRedCapSIB, as shown in Figure 1 and Figure 2.

Figure 1: DL BWP#0 of RedCap UE Contains CD-SSB and CORESET#0

Figure 2: DL BWP#0 of RedCap UE Contains CD-SSB but Not the Entire CORESET#0

Proposal 2:  For a RedCap UE in connected state, its DL BWP#0 is configured by either Option 1 (BWP-DownlinkCommon only) or Option 2 (both BWP-DownlinkCommon and BWP-DownlinkDedicated).

- If the DL BWP#0 of RedCap UE is configured by Option 1, the presence of SSB transmission depends at least on the configurations of DownlinkConfigCommonRedCapSIB.

- If the DL BWP#0 of RedCap UE is configured by Option 2, SSB is transmitted in the DL BWP#0 of RedCap UE.

In RAN1#107-e, RAN1 has reached the following agreements for PUCCH transmission of RedCap UE:

- When the frequency hopping for the RedCap PUCCH resources (for HARQ feedback for Msg4/MsgB) is deactivated,

  - Each PUCCH resource is mapped to a single PRB.

  - What side[(s)] of the RedCap UL BWP center frequency to which PUCCH resources are mapped is[/are] configurable by the network, including SIB-configurable [additional] offset (with no more than [4] candidate values) using the existing equations for determining the PRB index of the PUCCH transmission as a starting point.

- RedCap and non-RedCap can be configured with the same or different PUCCH resource set indices (see TS 38.213 Table 9.2.1-1).

Considering the frequency hopping for PUCCH can be enabled/disabled by SI in the initial UL BWP of RedCap UE and coverage enhancement for PUCCH has been specified in NR R17, we support the following proposal for PUCCH transmission of RedCap UE:

Proposal 3:  When the frequency hopping for PUCCH is disabled by SI in the initial UL BWP of idle/inactive RedCap UE,

- PUCCH resources used by a RedCap UE is mapped to a single PRB at the edge of the initial UL BWP of the RedCap UE;

- the equations for PUCCH resource mapping in NR R15/16 can be re-used;

- the frequency offset between the PUCCH resources and the edge of the carrier BW is configured in UplinkConfigCommonRedCapSIB;

- the time-domain repetitions for the PUCCH transmission is configured in UplinkConfigCommonRedCapSIB.

Based on [4], RSRP measurement before Msg1 or MsgA retransmission of RedCap UE can be left to UE implementation. When an idle/inactive RedCap UE is performing random access in a separate initial DL BWP without SSB, the RedCap UE is not required to monitor paging or measure CD-SSB outside its initial DL BWP. Therefore, we have the following proposals for the RedCap UE:

Proposal 4:  Confirming the RSRP measurement before msg1/msgA retransmission of RedCap UE is left to UE implementation.

Proposal 5:  For a RedCap UE operating in Type-A HD-FDD mode, if there is not a sufficient DL-to-UL switching gap between SSB and msg3 or PUCCH (HARQ feedback to msg4/msgB), whether to prioritize DL reception or UL transmission can be left to UE implementation.

Proposal 6:  When an idle/inactive RedCap UE is performing random access in a separate  initial DL BWP without SSB, or a RedCap UE is operating in Type-A HD-FDD mode,  the RedCap UE is not required to retransmit msg1 or msgA within NT,1+0.75 msec after the last symbol of the RAR window of msg2 or msgB, or the last symbol of the PDSCH reception, wherein NT,1 + 0.75 msec is a timeline for msg1/msgA retransmission specified for non-RedCap UE in Clause 8.2 and Clause 8.2A of TS 38.213.

# Conclusion

In this contribution, our have the following proposals on the remaining issues of RedCap UE complexity reduction:

Proposal 1: If a separate initial DL BWP is not configured for idle/inactive RedCap UE on  a cell that allows RedCap UE to access, the RedCap UE will use MIB-configured CORESET#0 as the default initial DL BWP in idle/inactive state.

Proposal 2:  For a RedCap UE in connected state, its DL BWP#0 is configured by either Option 1 (BWP-DownlinkCommon only) or Option 2 (both BWP-DownlinkCommon and BWP-DownlinkDedicated).

- If the DL BWP#0 of RedCap UE is configured by Option 1, the presence of SSB transmission depends at least on the configurations of DownlinkConfigCommonRedCapSIB.

- If the DL BWP#0 of RedCap UE is configured by Option 2, SSB is transmitted in the DL BWP#0 of RedCap UE.

Proposal 3:  When the frequency hopping for PUCCH is disabled by SI in the initial UL BWP of idle/inactive RedCap UE,

- PUCCH resources used by a RedCap UE is mapped to a single PRB at the edge of the initial UL BWP of the RedCap UE;

- the equations for PUCCH resource mapping in NR R15/16 can be re-used;

- the frequency offset between the PUCCH resources and the edge of the carrier BW is configured in UplinkConfigCommonRedCapSIB;

- the time-domain repetitions for the PUCCH transmission is configured in UplinkConfigCommonRedCapSIB.

Proposal 4:  Confirming the RSRP measurement before msg1/msgA retransmission of RedCap UE is left to UE implementation.

Proposal 5:  For a RedCap UE operating in Type-A HD-FDD mode, if there is not a sufficient DL-to-UL switching gap between SSB and msg3 or PUCCH (HARQ feedback to msg4/msgB), whether to prioritize DL reception or UL transmission can be left to UE implementation.

Proposal 6:  When an idle/inactive RedCap UE is performing random access in a separate  initial DL BWP without SSB, or a RedCap UE is operating in Type-A HD-FDD mode,  the RedCap UE is not required to retransmit msg1 or msgA within NT,1+0.75 msec after the last symbol of the RAR window of msg2 or msgB, or the last symbol of the PDSCH reception, wherein NT,1 + 0.75 msec is a timeline for msg1/msgA retransmission specified for non-RedCap UE in Clause 8.2 and Clause 8.2A of TS 38.213.

# Reference

- R1-2112802, “LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE,” RAN1#107e, November 11th-19th , 2021.

- R2-2201759, “Reply LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs,” RAN2#116bis-e, January 17th -25th , 2022.

- R4-2202773, “Reply LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE,”  RAN4#101bis-e, January 17th -25th , 2022.

- R2- 2201760, “LS on RSRP measurement before Msg1 or MsgA retransmission,” RAN2#116bis-e, January 17th -25th , 2022.

- R4-2202674, “Reply LS on use of NCD-SSB for RedCap UE,”  RAN4#101bis-e, January 17th -25th , 2022.