3GPP TSG RAN WG1 #108-e	R1-2202198

e-Meeting, February 21th – March 3rd, 2022

Source:	Sharp

Title:	Joint channel estimation for PUSCH

Agenda Item:	8.8.1.3

Document for:	Discussion and Decision

Background

RAN1 specification for Rel-17 CovEnh has been published. We discuss issues for finalization of the work.

TPC command

At RAN1#106b-e meeting, the following Working assumption was made in AI 8.8.1.3 [1].

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

For Rel-15, KPUSCH(i) for DG-PUSCH is defined as the number of symbols between the starting symbol of the PDCCH with the DCI scheduling a PUSCH and the start of “the PUSCH transmission”. Depending on whether “the PUSCH transmission” is each PUSCH repetition or the first PUSCH transmission among PUSCH repetitions, value of KPUSCH(i) is different from the one for another PUSCH transmission occasion among the PUSCH repetitions or is the same among the PUSCH repetitions, respectively. Using figures as written in [2], we show these interpretations as Interpretation 1 and Interpretation 2 below.

- Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the PUSCH transmission occasion i. This interpretation follows that “the PUSCH transmission” in the spec. as the PUSCH transmission in the PUSCH transmission occasion i. With this interpretation, value of KPUSCH(i) for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB.

Figure 1: Interpretation 1 [2]

- Interpretation 2: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the first PUSCH repetition for a TB. This interpretation follows that “the PUSCH transmission” in the spec. as the whole repetitions of the PUSCH for the TB. With this interpretation, value of KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

Figure 2: Interpretation 2 [2]

Observation 1: There are two interpretations of Rel-15 power control for DG-PUSCH as below:

- Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the PUSCH transmission occasion i. This interpretation follows that “the PUSCH transmission” in the spec. as the PUSCH transmission in the PUSCH transmission occasion i. With this interpretation, value of KPUSCH(i) for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB.

- Interpretation 2: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the first PUSCH repetition for a TB. This interpretation follows that “the PUSCH transmission” in the spec. as the whole repetitions of the PUSCH for the TB. With this interpretation, value of KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

However, TPC accumulation is mandatorily supported and repetition is important for not only CovEnh, but also other AIs. Particularly, it is not possible to specify operation of DMRS bundling unless Interpretation is determined. For this reason, RAN1 should adopt either Interpretation 1 or 2 for Rel-17.

Proposal 1: For Rel-17, RAN1 should adopt either Interpretation 1 or 2.

FFS: For Rel-16

If Interpretation 1 is adopted, TPC commands in DCI format 2_2 are not applied from the PDCCH with scheduling DCI to the last PUSCH repetition among all the PUSCH repetitions. This is because the set Di includes TPC commands that UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i where i0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i. Namely, for Interpretation 1, i-i0 is a PUSCH transmission occasion before the PDCCH.

Observation 2: If Interpretation 1 is adopted, TPC commands in DCI format 2_2 are not applied from the PDCCH with scheduling DCI to the last PUSCH repetition among all the PUSCH repetitions.

Furthermore, for CG-PUSCH, KPUSCH(i) is the same among all the PUSCH repetitions and used to consider minimum preparation time. Likewise, for DG-PUSCH, KPUSCH(i) can be the same among all the PUSCH repetitions and the large value of KPUSCH(i) is unnecessary. For this reason, Interpretation 2 should be adopted for Rel-17.

Proposal 2: Interpretation 2 should be adopted for Rel-17.

When DMRS bundling is enabled, the UE needs to accumulate TPC commands without taking effect during the configured TDW. We propose that fb,f,ci,l=fb,f,c(i-i0',l) is applied to the PUSCH transmission if the PUSCH transmission occasion i is within the configured TDW and not the first PUSCH transmission occasion within the configured TDW. i0' is determined such that i-i0' is the first PUSCH transmission occasion within the configured TDW. This condition is not applied to calculation of fb,f,c(i-i0',l) because i-i0' is the first PUSCH transmission occasion within the configured TDW. Therefore, fb,f,c(i-i0',l) can be substituted into the legacy equation (i.e., fb,f,ci-i0',l=fb,f,ci-i0'-i0,l+m=0CDi-i0'  -1δPUSCH,b,f,c(m,l)).

Proposal 3: Adopt TP1 in Appendix to capture Working assumption.

Conclusion

In this contribution, we have the following proposal:

Observation 1: There are two interpretations of Rel-15 power control for DG-PUSCH as below:

- Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the PUSCH transmission occasion i. This interpretation follows that “the PUSCH transmission” in the spec. as the PUSCH transmission in the PUSCH transmission occasion i. With this interpretation, value of KPUSCH(i) for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB.

- Interpretation 2: KPUSCH(i) is defined as the number of OFDM symbols between the starting OFDM symbol of the scheduling DCI and the start of the first PUSCH repetition for a TB. This interpretation follows that “the PUSCH transmission” in the spec. as the whole repetitions of the PUSCH for the TB. With this interpretation, value of KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

Observation 2: If Interpretation 1 is adopted, TPC commands in DCI format 2_2 are not applied from the PDCCH with scheduling DCI to the last PUSCH repetition among all the PUSCH repetitions.

Proposal 1: For Rel-17, RAN1 should adopt either Interpretation 1 or 2.

FFS: For Rel-16

Proposal 2: Interpretation 2 should be adopted for Rel-17.

Proposal 3: Adopt TP1 in Appendix to capture Working assumption.

References

- [1]	3GPP, RAN1#107-e, RAN1 Chairman’s Notes, Oct. 2021.

- [2]	R1-2200790, Summary of email discussion on joint channel estimation for PUSCH, Jan., 2022.

Appendix

TP1

### 7.1.1 UE behaviour <Unchanged parts are omitted> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,cm,l is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCHi symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCHi-i0 symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCHi symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, fb,f,ci,l=fb,f,c(i-i0',l), where i0'≥0 is determined such that PUSCH transmission occasion i-i0' is the first PUSCH transmission occasion within the nominal TDW if PUSCH transmission occasion i is within the nominal TDW and not the first PUSCH transmission occasion within the nominal TDW. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission <Unchanged parts are omitted>