3GPP TSG-RAN WG1 #108-e		R1-2202223

eMeeting, Feb 21- Mar 03, 2022

Source:	Ericsson

Title:	Evaluation of PDCCH enhancement for DSS

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction

In this document, we discuss PDCCH reception enhancement to improve DSS performance.

# Discussion

## PDCCH reception in symbols overlapping with LTE CRS

Currently, UEs are not expected to decode NR PDCCH that overlaps with LTE CRS. This limits PDCCH capacity in DSS deployments, as symbol 0 and 1 cannot be used for PDCCH if the CRS is transmitted on four ports. Removing the restriction that UEs are not expected to decode PDCCH in symbols that overlap with LTE CRS can increase PDCCH capacity. Below we evaluate the impact on PDCCH capacity.

For the evaluation, it is assumed that NR PDCCH is transmitted in symbols overlapping with LTE CRS, with the CRS puncturing the PDCCH and DMRS. Impact on decoding performance of the PDCCH is shown in Figure 2. ‘1 symbol’ (solid blue) corresponds to PDCCH transmitted only in symbol 2, while ‘2 symbol’ (dashed red) corresponds to PDCCH transmitted in symbol 1 and 2, where the PDCCH is punctured by CRS in symbol 1. The simulations are for a DCI payload of 55 bits excluding CRC.

Assuming the SINR distribution in Figure 3 (a low-band UMA scenario), the decrease in PDCCH decoding performance corresponds to an increase in average CCEs per DCI of 28% (5.8 CCEs for PDCCH in symbol 2 and 7.4 CCEs for PDCCH in symbol 1 and 2). However, allowing PDCCH in two symbols results in 2x more resources for PDCCH The net expected gain in PDCCH capacity is then approximately 2/7.4 1/5.8=1.57, i.e., 57%.

Since no change is needed for PDCCH transmission or reception (ratematching around CRS not required), the implementation impact by removing the restriction is expected to be limited.

Figure 2. NR PDCCH decoding performance

Figure 3. Example SINR distribution

# Annex A: Simulation assumptions

## Link level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 100 ns |
| UE speed | 3 km/h |
| Correlation | Medium |
| Symbols reserved for PDCCH | For evaluation in section 2.1. -- Symbol 0 and 1 reserved for LTE PDCCH, symbol 2 reserved for NR PDCCH (NR PDCCH also in symbol 1 for LTE CRS puncturing NR PDCCH) |
| Number of BS antennas | 4 Tx (Two cross-polarized antenna pairs) |
| Number of UE antennas | 2 Rx (One cross-polarized antenna pair) |
| Channel estimation | Practical, DMRS-based channel estimation with separate time and frequency domain filtering. No special handling for punctured REs |
| Receiver type | MMSE |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 55 bits |
| Interleaving | Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

## System level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |