3GPP TSG RAN WG1 Meeting #108-e  	          	      		            R1-2202251

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:   5

Source:   LG Electronics

Title: Discussion on LS on PEMAX for NR-V2X

Document for:   Discussion and decision

- Introduction

In this contribution, we will provide our views on the questions of PEMAX used in the calculation of the configured transmitted power for NR V2X in RAN4 LS [1].

- Discussion

For 1st question below on [1], our understanding of TS 38.331 is that the parameter of sl-maxTxPower is used for indicating the maximum power of PSSCH/PSCCH transmissions as one of the list of transmission parameters to (pre)configure the congestion control (see the yellow marked part below). Also the parameter of SL-TxPower represents candidate values that can be (pre)configured by sl-maxTxPower (see the cyan marked part below).

- Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

#### – SL-CBR-CommonTxConfigList The IE SL-CBR-CommonTxConfigList indicates the list of PSSCH transmission parameters (such as MCS, sub-channel number, retransmission number, CR limit) in sl-CBR-PSSCH-TxConfigList, and the list of CBR ranges in sl-CBR-RangeConfigList, to configure congestion control to the UE for sidelink communication. SL-CBR-CommonTxConfigList information element -- ASN1START -- TAG-SL-CBR-COMMONTXCONFIGLIST-START SL-CBR-CommonTxConfigList-r16 ::= SEQUENCE { sl-CBR-RangeConfigList-r16 SEQUENCE (SIZE (1..maxCBR-Config-r16)) OF SL-CBR-LevelsConfig-r16 OPTIONAL, -- Need M sl-CBR-PSSCH-TxConfigList-r16 SEQUENCE (SIZE (1.. maxTxConfig-r16)) OF SL-CBR-PSSCH-TxConfig-r16 OPTIONAL -- Need M } SL-CBR-LevelsConfig-r16 ::= SEQUENCE (SIZE (1..maxCBR-Level-r16)) OF SL-CBR-r16 SL-CBR-PSSCH-TxConfig-r16 ::= SEQUENCE { sl-CR-Limit-r16 INTEGER(0..10000) OPTIONAL, -- Need M sl-TxParameters-r16 SL-PSSCH-TxParameters-r16 OPTIONAL -- Need M } SL-CBR-r16 ::= INTEGER (0..100) -- TAG-SL-CBR-COMMONTXCONFIGLIST-STOP -- ASN1STOP SL-CBR-CommonTxConfigList field descriptions. sl-CBR-RangeConfigList Indicates the list of CBR ranges. Each entry of the list indicates in SL-CBR-LevelsConfig the upper bound of the CBR range for the respective entry. The upper bounds of the CBR ranges are configured in ascending order for consecutive entries of sl-CBR-RangeConfigList. For the first entry of sl-CBR-RangeConfigList the lower bound of the CBR range is 0. Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on.. sl-CR-Limit Indicates the maximum limit on the occupancy ratio. Value 0 corresponds to 0, value 1 to 0.0001, value 2 to 0.0002, and so on (i.e. in steps of 0.0001) until value 10000, which corresponds to 1.. sl-CBR-PSSCH-TxConfigList Indicates the list of available PSSCH transmission parameters (such as MCS, sub-channel number, retransmission number and CR limit) configurations.. sl-TxParameters Indicates PSSCH transmission parameters.. #### – SL-PSSCH-TxConfigList The IE SL-PSSCH-TxConfigList indicates PSSCH transmission parameters. When lower layers select parameters from the range indicated in IE SL-PSSCH-TxConfigList, the UE considers both configurations in IE SL-PSSCH-TxConfigList and the CBR-dependent configurations represented in IE SL-CBR-PriorityTxConfigList. Only one IE SL-PSSCH-TxConfig is provided per SL-TypeTxSync. SL-PSSCH-TxConfigList information element -- ASN1START -- TAG-SL-PSSCH-TXCONFIGLIST-START SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 SL-PSSCH-TxConfig-r16 ::= SEQUENCE { sl-TypeTxSync-r16 SL-TypeTxSync-r16 OPTIONAL, -- Need R sl-ThresUE-Speed-r16 ENUMERATED {kmph60, kmph80, kmph100, kmph120, kmph140, kmph160, kmph180, kmph200}, sl-ParametersAboveThres-r16 SL-PSSCH-TxParameters-r16, sl-ParametersBelowThres-r16 SL-PSSCH-TxParameters-r16, ..., [[ sl-ParametersAboveThres-v1650 SL-MinMaxMCS-List-r16 OPTIONAL, -- Need R sl-ParametersBelowThres-v1650 SL-MinMaxMCS-List-r16 OPTIONAL -- Need R ]] } SL-PSSCH-TxParameters-r16 ::= SEQUENCE { sl-MinMCS-PSSCH-r16 INTEGER (0..27), sl-MaxMCS-PSSCH-r16 INTEGER (0..31), sl-MinSubChannelNumPSSCH-r16 INTEGER (1..27), sl-MaxSubchannelNumPSSCH-r16 INTEGER (1..27), sl-MaxTxTransNumPSSCH-r16 INTEGER (1..32), sl-MaxTxPower-r16 SL-TxPower-r16 OPTIONAL -- Cond CBR } -- TAG-SL-PSSCH-TXCONFIGLIST-STOP -- ASN1STOP SL-PSSCH-TxConfigList field descriptions. sl-MaxTxTransNumPSSCH Indicates the maximum transmission number (including new transmission and retransmission) for PSSCH.. sl-MaxTxPower This field indicates the maximum transmission power for transmission on PSSCH and PSCCH.. sl-MinMCS-PSSCH, sl-MaxMCS-PSSCH This field indicates the minimum and maximum MCS values used for transmissions on PSSCH. The UE shall ignore the minimum and maximum MCS values used for the associated MCS table(s) in sl-ParametersAboveThres-r16 and sl-ParametersBelowThres-r16 if sl-ParametersAboveThres-v1650 and sl-ParametersBelowThres-v1650 are present, respectively.. sl-MinSubChannelNumPSSCH, sl-MaxSubChannelNumPSSCH This field indicates the minimum and maximum number of sub-channels which may be used for transmissions on PSSCH.. sl-TypeTxSync This field indicates the synchronization reference type. For configurations by the eNB/gNB, only gnbEnb can be configured; and for pre-configuration or when this field is absent, the configuration is applicable for all synchronization reference types.. sl-ThresUE-Speed This field indicates a UE absolute speed threshold.. Conditional Presence; Explanation. CBR; The field is optionally present, Need R, when the IE SL-PSSCH-TxParameters is present in SL-CBR-CommonTxConfigList, SL-UE-SelectedConfig, SIB12 or SidelinkPreconfigNR; otherwise the field is not present, need R.. #### – SL-TxPower The IE SL-TxPower is used to limit the UE's sidelink transmission power on a carrier frequency. The unit is dBm. Value minusinfinity corresponds to –infinity. SL-TxPower information element -- ASN1START -- TAG-SL-TXPOWER-START SL-TxPower-r16 ::= CHOICE{ minusinfinity-r16 NULL, txPower-r16 INTEGER (-30..33) } -- TAG-SL-TXPOWER-STOP -- ASN1STOP

Considering the part of TS 38.331 marked in green below, we think that the parameter of sl-MaxTransPower is the correct one used for indicating a value of PEMAX in the calculation of the configured transmitted power for NR V2X.

#### – SL-ResourcePool The IE SL-ResourcePool specifies the configuration information for NR sidelink communication resource pool. SL-ResourcePool information element -- ASN1START -- TAG-SL-RESOURCEPOOL-START SL-ResourcePool-r16 ::= SEQUENCE { sl-PSCCH-Config-r16 SetupRelease { SL-PSCCH-Config-r16 } OPTIONAL, -- Need M sl-PSSCH-Config-r16 SetupRelease { SL-PSSCH-Config-r16 } OPTIONAL, -- Need M sl-PSFCH-Config-r16 SetupRelease { SL-PSFCH-Config-r16 } OPTIONAL, -- Need M sl-SyncAllowed-r16 SL-SyncAllowed-r16 OPTIONAL, -- Need M sl-SubchannelSize-r16 ENUMERATED {n10, n12, n15, n20, n25, n50, n75, n100} OPTIONAL, -- Need M dummy INTEGER (10..160) OPTIONAL, -- Need M sl-StartRB-Subchannel-r16 INTEGER (0..265) OPTIONAL, -- Need M sl-NumSubchannel-r16 INTEGER (1..27) OPTIONAL, -- Need M sl-Additional-MCS-Table-r16 ENUMERATED {qam256, qam64LowSE, qam256-qam64LowSE } OPTIONAL, -- Need M sl-ThreshS-RSSI-CBR-r16 INTEGER (0..45) OPTIONAL, -- Need M sl-TimeWindowSizeCBR-r16 ENUMERATED {ms100, slot100} OPTIONAL, -- Need M sl-TimeWindowSizeCR-r16 ENUMERATED {ms1000, slot1000} OPTIONAL, -- Need M sl-PTRS-Config-r16 SL-PTRS-Config-r16 OPTIONAL, -- Need M sl-UE-SelectedConfigRP-r16 SL-UE-SelectedConfigRP-r16 OPTIONAL, -- Need M sl-RxParametersNcell-r16 SEQUENCE { sl-TDD-Configuration-r16 TDD-UL-DL-ConfigCommon OPTIONAL, -- Need M sl-SyncConfigIndex-r16 INTEGER (0..15) } OPTIONAL, -- Need M sl-ZoneConfigMCR-List-r16 SEQUENCE (SIZE (16)) OF SL-ZoneConfigMCR-r16 OPTIONAL, -- Need M sl-FilterCoefficient-r16 FilterCoefficient OPTIONAL, -- Need M sl-RB-Number-r16 INTEGER (10..275) OPTIONAL, -- Need M sl-PreemptionEnable-r16 ENUMERATED {enabled, pl1, pl2, pl3, pl4, pl5, pl6, pl7, pl8} OPTIONAL, -- Need R sl-PriorityThreshold-UL-URLLC-r16 INTEGER (1..9) OPTIONAL, -- Need M sl-PriorityThreshold-r16 INTEGER (1..9) OPTIONAL, -- Need M sl-X-Overhead-r16 ENUMERATED {n0,n3, n6, n9} OPTIONAL, -- Need S sl-PowerControl-r16 SL-PowerControl-r16 OPTIONAL, -- Need M sl-TxPercentageList-r16 SL-TxPercentageList-r16 OPTIONAL, -- Need M sl-MinMaxMCS-List-r16 SL-MinMaxMCS-List-r16 OPTIONAL, -- Need M ..., [[ sl-TimeResource-r16 BIT STRING (SIZE (10..160)) OPTIONAL -- Need M ]] } [...] SL-PowerControl-r16 ::= SEQUENCE { sl-MaxTransPower-r16 INTEGER (-30..33), sl-Alpha-PSSCH-PSCCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M dl-Alpha-PSSCH-PSCCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need S sl-P0-PSSCH-PSCCH-r16 INTEGER (-16..15) OPTIONAL, -- Need S dl-P0-PSSCH-PSCCH-r16 INTEGER (-16..15) OPTIONAL, -- Need M dl-Alpha-PSFCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need S dl-P0-PSFCH-r16 INTEGER (-16..15) OPTIONAL, -- Need M ... } -- TAG-SL-RESOURCEPOOL-STOP -- ASN1STOP SL-PowerControl field descriptions. sl-MaxTransPower Indicates the maximum value of the UE's sidelink transmission power on this resource pool. The unit is dBm.. sl-Alpha-PSSCH-PSCCH Indicates alpha value for sidelink pathloss based power control for PSCCH/PSSCH when sl-P0-PSSCH is configured. When the field is absent the UE applies the value 1.. sl-P0-PSSCH-PSCCH Indicates P0 value for sidelink pathloss based power control for PSCCH/PSSCH. If not configured, sidelink pathloss based power control is disabled for PSCCH/PSSCH.. dl-Alpha-PSSCH-PSCCH Indicates alpha value for downlink pathloss based power control for PSCCH/PSSCH when dl-P0-PSSCH is configured. When the field is absent the UE applies the value 1.. dl-P0-PSSCH-PSCCH Indicates P0 value for downlink pathloss based power control for PSCCH/PSSCH. If not configured, downlink pathloss based power control is disabled for PSCCH/PSSCH.. dl-Alpha-PSFCH Indicates alpha value for downlink pathloss based power control for PSFCH when dl-P0-PSFCH is configured. When the field is absent the UE applies the value 1.. dl-P0-PSFCH Indicates P0 value for downlink pathloss based power control for PSFCH. If not configured, downlink pathloss based power control is disabled for PSFCH..

Observation 1: According to the current specification, the parameter of sl-maxTxPower is used for indicating the maximum power of PSSCH/PSCCH transmissions as one of the list of transmission parameters to (pre)configure the congestion control. Also the parameter of SL-TxPower represents candidate values that can be (pre)configured by sl-maxTxPower.

Proposal 1: For Issue 1 in [1], RAN1’s understanding is that the parameter of sl-MaxTransPower is used for indicating a value of PEMAX in the calculation of the configured transmitted power for NR V2X.

For 2st question below on [1], according to the current specification, we think that the parameter of sl-MaxTransPower for PEMAX can be configured in case of in-coverage and can be pre-configured in case of out-of-coverage. Also sl-MaxTransPower for PEMAX and p-max for Uu are separately configured in case of in-coverage and the relationship or selection between these two parameters is not explicitly specified.

- Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed:

  - Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu

  - Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell.

  - Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used.

Observation 2: The relationship or selection between sl-MaxTransPower for PEMAX and p-max for Uu is not explicitly described in the current specification.

Proposal 2: For Issue 2 in [1], RAN1’s understanding is that the parameter of sl-MaxTransPower for PEMAX can be associated either with a serving cell (i.e., in-coverage) or without a serving cell (i.e., out-of-coverage) on the V2X carrier, and it can be configured separately with p-max for Uu in case of in-coverage.

- Conclusion

In this contribution, we discussed how RAN1 provides feedbacks on RAN4’s questions in LS [1]. The following observations and proposals are given.

Observation 1: According to the current specification, the parameter of sl-maxTxPower is used for indicating the maximum power of PSSCH/PSCCH transmissions as one of the list of transmission parameters to (pre)configure the congestion control. Also the parameter of SL-TxPower represents candidate values that can be (pre)configured by sl-maxTxPower.

Observation 2: The relationship or selection between sl-MaxTransPower for PEMAX and p-max for Uu is not explicitly described in the current specification.

Proposal 1: For Issue 1 in [1], RAN1’s understanding is that the parameter of sl-MaxTransPower is used for indicating a value of PEMAX in the calculation of the configured transmitted power for NR V2X.

Proposal 2: For Issue 2 in [1], RAN1’s understanding is that the parameter of sl-MaxTransPower for PEMAX can be associated either with a serving cell (i.e., in-coverage) or without a serving cell (i.e., out-of-coverage) on the V2X carrier, and it can be configured separately with p-max for Uu in case of in-coverage.

- Reference

[1] R1-2200866, “LS on PEMAX for NR-V2X,” RAN4, Huawei, CATT.