3GPP TSG-RAN WG1 #108-e 	R1-2202273

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.2.2

Source:	Panasonic

Title:	PDCCH monitoring for NR operation from 52.6 to 71 GHz

Document for:	Discussion, Decision

# Introduction

In RAN1#107-e meeting, RAN1 has finally reached consensus of a workable solution on how to define multi-slot monitoring capability. It is based on the fixed pattern of slot groups where each slot group contains Xs slots. For Group (1) search spaces (i.e. Type 1 CSS with dedicated RRC configuration, Type 3 CSS, and UE specific SS), UE only needs to monitor Ys consecutive slots out of Xs slots.

As also agreed, the Ys consecutive slots can be located anywhere within the slot group of Xs slots, but the location of the Ys consecutive slots within the slot group of Xs slots should be maintained across different slot groups.

For convenience, the related part of agreement is recited as below:

Agreement: - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots … (the rest of agreement is irrelevant for the current discussion and hence omitted.)

The following is the corresponding text from 38.213 v17.0.0 that reflects the above.

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. …

In this paper, we discuss following two corrections for the above spec description.

- For the agreement that “BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots”, the spec only describes that what UE “can” do within the same Y consecutive slots, but does not describe any UE behavior for slots outside of the Y consecutive slots.

- We have identified that “the location of Y consecutive slots within the slot group of X slots is maintained across different slot groups” cannot be always true.

Therefore, we propose some amendment to the above agreement and corresponding description to the 38.213.

# Discussion

Capture the restriction the BD attempts of Group(1) SSs within Y slots

As mentioned in section 1, the current 38.213 describe the following behavior regarding Group (1) SSs monitoring:

“If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, …”

Our view is the above description is not sufficient to reflect the earlier agreement that BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots. The reason is that the spec only describes that what UE “can” do for Ys slots, but does not describe any UE behavior for slots outside Ys slots. Without such restriction, it implies UE still needs to monitor slots outside outside Ys slots for Group (1) SSs if configured by gNB via search space configuration. Then the whole multi-slot monitoring mechanism of Xs,Ys becomes meaningless. Therefore, we propose the following changes to 38.213:

Proposal 1: To capture the earlier agreement that BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots in 38.213, adopt the following TP (with modifications highlighted in yellow).

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots and the UE is not expected to monitor any other slot of the Xs slots than Ys slots for PDCCH of the above-mentioned types, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. ...

Adaptation of location of Y within X

The earlier agreement that “the location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups” cannot be true when new monitoring occasion is configured or indicated to the UE. If this case is not allowed, the system cannot operate correctly.

Considering the TDMed-beam transmission of CSS (of Group (1)), UE may need to monitor a different slot for CSS when UE’s serving beam is changed. However, for USS, gNB can simply change the serving beam without changing the MO location since USS MO is unicast to the UE. In this case, it can happen that CSS MO becomes far away from the USS MO of the new serving beam such that Ys cannot include both CSS and USS MOs anymore.

The following Fig.1 illustrates above case more concretely. As shown, in the first slot group (UE is served by yellow beam), the location of Ys is the first slot, covering USS MO and CSS MO of the yellow beam. When UE moves from the coverage of yellow beam to green beam in the second slot group, the corresponding CSS MO location for the UE has to be changed as well because the previous CSS MO would still be used for other UEs covered by the yellow beam. Consequently, the new CSS MO for the UE becomes outside the Ys. According to the current spec, the UE may not receive such CSS because the location of Ys cannot be changed.

Fig.1 Issue of fixed location of Ys due to beam switching

To fix the above issue, the location of Ys should be allowed to change if new MO is configured or indicated to the UE. Considering the fact that Group (1) SSs include both CSS and USS, CSS should be prioritized over USS in order not to miss important control information. In other words, location of Ys should follow the new configured/indicated MOs of CSS (of Group (1)). Then USS MOs can be further included in Ys from USS with lower to higher indices. Certain USS MO would be dropped if it cannot be included in Ys .

To reflect the above, the following TP is suggested:

Proposal 2: In case that MO of Group (1) CSS is changed, the location of Ys within Xs can be adapted accordingly to include all CSS MOs (of Group(1) SS) that are monitored by UE.

Adopt the following TP to 38.213 (with modifications highlighted in yellow).

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots until new monitoring occasion of Type1-PDCCH CSS set provided by dedicated higher layer signalling, or of Type3-PDCCH CSS sets is configured or indicated to the UE. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

# Conclusion

In this paper, we suggest some amendment to earlier agreement on multi-slot PDCCH monitoring framework and corresponding description to 38.213. Specifically, the proposals are as follows.

Proposal 1: To capture the earlier agreement that BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots in 38.213, adopt the following TP (with modifications highlighted in yellow).

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots and the UE is not expected to monitor any other slot of the Xs slots than Ys slots for PDCCH of the above-mentioned types, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. ...

Proposal 2: In case that MO of Group (1) CSS is changed, the location of Ys within Xs can be adapted accordingly to include all CSS MOs (of Group(1) SS) that are monitored by UE.

Adopt the following TP to 38.213 (with modifications highlighted in yellow).

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots until new monitoring occasion of Type1-PDCCH CSS set provided by dedicated higher layer signalling, or of Type3-PDCCH CSS sets is configured or indicated to the UE. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. …