3GPP TSG-RAN WG1 Meeting #108-e	Tdoc R1-2202276

e-Meeting, February 21st– March 3rd 2022

Agenda Item:	8.1.4

Source:	Ericsson

Title:	Remaining issues on multi-TRP CSI

Document for:	Discussion

# 1	Introduction

In this paper, we provide Ericsson’s views on the remaining issues related to multi-TRP CSI considering the progress made on this topic until RAN1#107-e.

# 2	Discussion

In the multi-TRP CSI enhancement, two CMR groups were agreed.  The two CMR groups are used when forming a CMR pair for NC-JT CSI wherein the CMR pair constitutes one CMR from each CMR group.  In the current running CR of 38.331, the two CMR groups are captured as part of the NZP-CSI-RS-ResourceSet information element below.  In the below modified NZP-CSI-RS-ResourceSet IE, the CMR grouping information as well as the CMR pairs are given by the higher layer parameter cmrGroupingAndPairing-r17.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                       OPTIONAL,   -- Need R

...,

[[

aperiodicTriggeringOffset-r16       INTEGER(0..31)                                                          OPTIONAL   -- Need S

]] ,

[[

cmrGroupingAndPairing-r17                CMRGroupingAndPairing-r17                                           OPTIONAL      Need R

]]

}

CMRGroupingAndPairing ::=              SEQUENCE {

nrofResourcesGroup1-r17              INTEGER {1..7},

nrofResourcesGroup2-r17              INTEGER {1..7},

pair1OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing                                                      OPTIONAL,  -- Need R

pair2OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing                                                      OPTIONAL   -- Need R

}

NZP-CSI-RS-Pairing-r17  ::=        SEQUENCE {

nzp-CSI-RS-ResourceId,

nzp-CSI-RS-ResourceId

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

In the multi-beam for multi-TRP agenda, for enhanced group based beam reporting, two NZP CSI-RS resource sets are agreed in each CSI-AssociatedReportConfigInfo for aperiodic beam reporting.  In the current running CR of 38.331, the 2nd NZP CSI-RS resource set is given by resourcesForChannel2-r17 as part of the CSI-AperiodicTriggerStateList IE below.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

... ,

[[

followUnifiedTCIstate-r17             ENUMERATED {enabled}                                         OPTIONAL,  -- Need R

-- Editor’s note: OPTION 2: at CSI hypothesis level, which means each CSI hypothesis can separately be configuredd

-- Editor’s note: this applies only to CMR

-- Editor’s note: FFS on further input if this is the correct placement

ap-CSI-MultiplexingMode-r17             ENUMERATED {enabled}                                      OPTIONAL,  -- Need R

resourcesForChannel2-r17                 CHOICE {

nzp-CSI-RS-r17                          SEQUENCE {

resourceSet-r17                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info-r17                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet-r17                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

}                                                                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

Note that the above two features are independent and should not be configured together.  A similar issue exists when a periodic or semi-persistence CSI Resource Configuration is configured with two NZP CSI-RS resource sets.  Hence, we make the following two proposals:

The UE is not expected to be configured with higher layer parameter cmrGroupingAndPairing-r17 in an NZP CSI-RS resource set that is indicated as the second NZP CSI-RS resource set via higher layer parameter resourcesForChannel2 in CSI-AssociatedReportConfigInfo.

For a higher layer parameter resourcesForChannelMeasurement configured with two Periodic or semiPersistent NZP CSI-RS resource sets, the UE is not expected to be configured with higher layer parameter cmrGroupingAndPairing-r17 in any of the two NZP CSI-RS resource sets.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE is not expected to be configured with higher layer parameter cmrGroupingAndPairing-r17 in an NZP CSI-RS resource set that is indicated as the second NZP CSI-RS resource set via higher layer parameter resourcesForChannel2 in CSI-AssociatedReportConfigInfo.

Proposal 2	For a higher layer parameter resourcesForChannelMeasurement configured with two Periodic or semiPersistent NZP CSI-RS resource sets, the UE is not expected to be configured with higher layer parameter cmrGroupingAndPairing-r17 in any of the two NZP CSI-RS resource sets.