3GPP TSG RAN WG1 Meeting # 108-e 	R1-2202281

# E-meeting, February 21st - March 3rd, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1415 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the support of 16-QAM for NB-IoT in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 16.2.1.1.1, it has been missed to capture that ΔTF introduced for the support of 16-QAM in UL also applies for PUR. - In clause 16.4.1.5, there is no statement to reflect the support of 16-QAM in DL during the PUR procedure. - In clause 16.5.1.2, it is not captured from where the indices used to determine a TBS are obtained in the case of NPUSCH transmission using preconfigutred uplink resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 16.2.1.1.1, it has been clarified that ΔTF introduced for the support of 16-QAM in UL also applies for PUR. - In clause 16.4.1.5, a statement has been added to reflect the support of 16-QAM in DL during the PUR procedure. - In clause 16.5.1.2, a complementary statement citing the corresponding information element has been added to clarify from where the indices used to determine a TBS are obtained in the case of NPUSCH transmission using preconfigutred uplink resources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - In clause 16.2.1.1.1, ΔTF won’t be applied for 16-QAM in UL for PUR. - In clause 16.4.1.5, the support of 16-QAM in DL during the PUR procedure would be missing. - In clause 16.5.1.2, the stament for PUR would remain incomplete and it would be unclear from where the indices used to determine a TBS are obtained in the case of NPUSCH transmission using preconfigutred uplink resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.1.1, 16.4.1.5, and 16.5.1.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- Text Start -----------------------------------------------------------------

##### 16.2.1.1.1	UE behaviour

The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD,  enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level.

------------------------------------------------------------ Text Omitted --------------------------------------------------------------

-	If NPUSCH (re)transmissions with 16QAM or NPUSCH (re)transmission corresponding to preconfigured uplink resource with 16QAM,

-	ΔTF,ci=10log102BPRE⋅Ks-1 for and ∆TF,c(i)=0 for where  is given by the parameter deltaMCS-Enabled provided by higher layers for serving cell , and

-	BPRE=K/NRE where K is the code block size and NRE is the number of resource elements determined as NRE=(NsymbUL-1)NslotsULNscRUNRU where NsymbUL, NslotsUL, NscRU are defined in [3], and NRU is defined in section 16.5.1.1

-	otherwise ∆TF,c(i)=0.

------------------------------------------------------------ Text Omitted --------------------------------------------------------------

#### 16.4.1.5	Modulation order and transport block size determination

To determine the modulation order in the NPDSCH, the UE shall

-	if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI or if the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI, and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’,

-	use modulation order, = 4

-	otherwise

-	use modulation order, = 2.

------------------------------------------------------------ Text Omitted --------------------------------------------------------------

#### 16.5.1.2	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the "modulation and coding scheme" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and

-	read the "redundancy version" field () in the DCI or initiate with rvDCI=0 for NPUSCH transmission using preconfigured uplink resource, and

-	read the "resource assignment" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Clause 16.5.1.1.

------------------------------------------------------------ Text Omitted --------------------------------------------------------------

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if , or ITBS=IMCS'+14 if NPUSCH with 16QAM except for NPUSCH transmission using preconfigured uplink resource in which case the corresponding indices are provided in PUR-Config-NB,  otherwise. IMCS' is the value of the "modulation and coding scheme for 16QAM" in the DCI.

---------------------------------------------------------------- Text End ----------------------------------------------------------------