3GPP TSG-RAN WG1 Meeting #108-e	R1-2202309

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft LS reply on feMIMO RRC parameters

Response to:	R2-2202002/ R1-2200887

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN2

Cc:	RAN3, RAN4

Contact Person:

#### Name:	Mihai Enescu

E-mail Address:	mihai.enescu@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-2200887 (R2-2202002) on feMIMO RRC parameters.

RAN1 discussed the questions posed by RAN2 and would like to provide the following answers:

1. MultiBeam

CORESET to follow Unified TCI state

RAN2 has discussed the per CORESET RRC based indication based on RAN1 agreements.

  - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC

RAN2 understands that the 1 bit RRC indication “followUnifiedTCI-State” would be needed for CORESET type “B”. RAN2 understanding is that it seems to indicate how the CORESET behaves with respect to the TCI state of PDSCH depending on the type (i.e. CSS or USS) of the SearchSpace that is linked to that CORESET. However, as in RRC there is no types of CORESETs RAN2 would like to ask RAN1 to clarify the intention of the indication in more details.

Question 1.1: What is the intent behind this indication and why was it put to CORESET but not per SearchSpace?

Answer 1.1 The reason why Rel17 TCI state put to CORESET and not associated directly to SearchSpace set was that the TCI state is configured per CORESET in existing specification. The reason for having the 1-bit RRC indication for the CORESET that is associated with CSS is to allow network to use also different TCI state than indicated TCI state for the common control signalling.

Question 1.2: Are there any limitation or conditions needs to specified for the "followUnifiedTCI-State" parameter?

Answer 1.2 For the CORESET for which only USS is associated the “followUnifiedTCI-State” is always set “true” per RAN1 agreement. For CORESET B, configured with followUnifiedTCI-State, should not follow the unified TCI state beam indication when the TCI state indicates RS associated with PCI different from that of serving cell. RAN1 specification 38.213 will capture the needed limitations and RAN1 will communicate any needed RRC limitations via the L1 parameter updates.

Question 1.3: How are the “DM-RS for non-UE dedicated PDCCH” in parameter "applyTCI-State-DL-List-r17"  and the CORESET B “followUnifiedTCI-State” related?

Answer 1.3 The intent is the same: whether the channel/RS follows the indicated TCI state or not.

Parameter applyTCI-StateDL-List-r17

RAN2 notes there is discrepancy with the description and comment related to applyTCI-State-DL-List-r17. RAN2 has baseline implementation for this functionality where 1 bit “followUnifiedTCI-State" indication is added to “AssociatedReportConfigInfo” IE where QCL per an aperiodic resource set is currently configured i.e. all resource within NZP-CSI-RS resource set follow unified TCI state in DCI.

Question 1.4: Is this RRC parameter implementation is according to intended functionality or should the indication be placed per NZP-CSI-RS resource set or resource. Note that these NZP-CSI-RS resource sets and resource configurations are not specific to AP?

Answer 1.4 The indication may be placed per NZP CSI-RS resource set.

Note that it will be RAN2 signalling design whether supporting this functionality is 1 bit indication per field X, or by maintaining lists of field X.

Parameter ApplyTCI-State-r17forSRS

RAN2 intends to add the parameter “followUnifiedTCI-State-r17” (ApplyTCI-State-r17forSRS in RAN1 RRC parameter list) to SRS-ResourceSet IE according to RAN1 guidance.

Question 1.5: Are the stated restrictions indicated in the L1 parameter excel (i.e. “This applies to the following: 1) Aperiodic SRS for BM, 2) SRS (of any time-domain behavior) for codebook, non-codebook, and antenna switching “)  should be placed in TS 38.331 or these will be specified by RAN1? If they should be specified in RAN2, are there any additional restrictions that have not yet been communicated?

Answer 1.5 We believe it is simpler if the restrictions are captured in RAN2 specification.

Question 1.6: RAN2 would also like to confirm whether also semi-persistent SRS (as RAN1 mentioned “of any time-domain behaviour) will follow unified TCI state in DCI or some coordination between RRC signalling, MAC CE and DCI is needed?

Answer 1.6 Semi-persistent SRS would follow unified TCI state in DCI.

MPE

In RAN2#116, RAN2 agreed the following

  - 4: Rel-17 MPE configuration can be included in PHR-Config. Will ask R1 whether MPE information can apply to both ICBM and mTRP

This will impact at least the corresponding MAC CE design but potentially also configuration. Further, the parameter excel has TBD on the range for configuring the MPE resource pool. RAN2 understanding is that the MPE-ResourcePool may be a list of SSB or CSI-RS resources, which will be configured by RRC but for which RAN1 has not yet indicated maximum number. RAN2 would need to know this to derive the number of bits needed for the resource IDs in the MPE resource pool.

Question 1.7: Please clarify  the structure of the mpe-ResourcePool: Is it a list of SSB or CSI-RS resources (i.e. SSBRI or CRI), and what is the maximum number of resources configured in the pool?

Answer 1.7 The structure of the mpe-ResourcePool is a list of SSB or CSI-RS resources (i.e. SSBRI or CRI). The maximum number is yet to be agreed by RAN1 and could be the maximum number of RS I.e. 64, in order to maximize the likelihood of reporting an MPE-free candidate beam.

RAN2 was also not clear on whether the MPE reporting would apply for the mTRP PHR and whether configuration mpe-Reporting-FR2 can apply to both BM case and mTRP case to activate the reporting, so RAN2 would like RAN1 to clarify this.

Question 1.8: Does the enhanced MPE reporting applies also to mTRP operation, and, if it does, will this be configured by mpe-Reporting-FR2 or is another RRC configuration needed?

Answer 1.8 The mTRP framework application of MPE reporting is discussed in the separate LS R1-2202313. The support of two PHR reports was agreed under Rel-17 M-TRP URLLC, which can serve as a base for MPE reporting based on the Rel-16 MAC CE (if mpe-Reporting-FR2 is configured). The RAN1 impact of applying MPE reporting changes to mTRP framework is viewed as manageable since any 		configuration can be part of the resource pool. For mTRP inter- and intra- cell (including single and multi –DCI), 	the extension of MPE reporting to mTRP framework is feasible. RAN1 does not see road-blockers for applying MPE reporting changes to the mTRP framework and recommends implementing such extension in Rel-17 in RAN2. RAN1 could agree whether additional UE capabilities are needed, e.g. differentiating between ICBM and mTRP.

Question 1.9: RAN1 to confirm whether the RAN2 should keep the MPE-Config-FR2-r17 in the PHR-Config IE, which is per cell group, or move it to (per-cell) per BWP level as indicated in L1 parameter excel?

Answer 1.9 the MPE-Config-FR2-r17 should be moved per BWP since the UE power settings change per BWP. The MPE P-MPR value depends on UE EIRP.

Question 1.10: Is reporting of PCMax,f,c needed for MPE information and if it is, should it be included per indicated SSBRI/CRI value or is it cell-specific?

Answer 1.10 Reporting of PCMax,f,c is needed for MPE information and should be included per indicated SSBRI/CRI value as different beams can have different PcMax. Furthermore, PCMax offers a finer granularity than MPE P-MPR bits:  MPE granularity is 3 dB, while PCMax granularity is 1 dB.

BeamAppTime value range

RAN2 has been discussing on what level the BAT parameter should be configured. RAN2 has found guidance for per CCs per CSC  “with the common TCI state ID update”. However, it is not clear what “common TCI state ID update” means or exactly what is the correct level for configuring the parameter.

Question 1.11: RAN2 would like to further confirm whether this parameter is per-UE (i.e. applicable to all cell groups per SCS), per cell group (i.e. within the same cell group, all cells use the same values per SCS), per cell (i.e. different cells may use different value per SCS), or something else?

Answer 1.11 Per cell group: BAT indicates the earliest slot when TCI state is applied according to smallest SCS within the cell group.

Question 1.12: Is it correct understanding that the common TCI state ID update is when the same TCI state list is configured for multiple CCs with reference BWP/CC?

Answer 1.12 There are two configuration alternatives supported for the multiple CC case:

1. The same TCI state list is configured for multiple BWPs/CCs in the set of configured CCs

2. The TCI state list is configured only in one BWP/CC (reference BWP/CC) in the set of configured CCs

Question 1.13: Please indicate what should be the value range for parameter beamAppTime-r17?

Answer 1.13 A clear RAN1 decision is still pending. Could reuse Rel-16 parameter (=PDSCH beam indication) as baseline.

CSI-SSB-ResourceSet

Question 1.13: Should it be possible for different SSB indexes in the same CSI-SSB-ResourceSet to be associated with different additionalPCI?

Answer 1.14 Yes, different SSB indexes can be associated with different PCIs (the number of additionalPCIs are subjected to reported UE capability). Also, in a single reporting instance both serving and additional PCI measurements can be provided.

Agreement

On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP.

- The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM

- Detailed signaling design is up to RAN2

Agreement (RAN1#104b-e)

On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP,

- In one reporting instance, depending on NW configuration, beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell

Simultaneous usage of different operation for different serving cells

RAN2 understanding is that all channels and RS in one serving cell have to follow one TCI state framework, either Rel-17 or Rel 15/16.

Question 1.14a: Please confirm whether above RAN2 understanding is correct.

Answer 1.14a Yes, RAN2 understanding is correct with the following clarification: any DL RS that does not follow indicated Rel17 TCI state can be a target RS of the Rel17 TCI state, and Rel17 mechanism which reuse Rel15/16 TCI state update signalling/configuration design.

Agreement

On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), Rel-17 mechanism(s) which reuse the Rel-15/16 TCI state update signaling/configuration design(s) are used to update/configure such DL RS(s) with Rel-17 TCI state(s).

- Applies for both intra-cell and inter-cell beam indication

Question 1.15: can different serving cells in a cell group use different TCI framework (Rel-16 or Rel-17)?

Answer 1.15 Yes

Question 1.16: can different serving cells in a cell group use different TCI mode (joint or separate) if Rel-17 unified TCI framework is configured?

Answer 1.16 Our understanding is that it should be possible.

BM power control configuration

In current running RRC CR the PO set(P0, alpha, closed loop index) is encoded in both UL TCI state as well in BWP-UL-Dedicated (that is outside of UL TCI state) and different values are enabled for each UL channel PUSCH, PUCCH, SRS. UE receives the UL pc configuration in either UL TCI states or in BWP UL-dedicated.

Question 1.15a: Is it correct understanding that network may provide UE the UL pc configuration in either UL TCI states or in BWP-UL-dedicated or should RAN2 choose one? If UL PC configuration is signalled in BWP-UL-dedicated only, how can the specific PC configuration (actually applied) be decided in PHY layer?

Answer 1.15a Based on RAN1 agreement, the UL PC parameters can be associated to a UL (or joint when applicable) TCI state.

2. mTRP (PUCCH, PDCCH)

For mTRP PUCCH, RAN2 has agreed to add a new IE for power control for mTRP FR1 operation. However, RAN2 would need information on the number of power control sets to be configured with respect to the each TRP and then in relation to the corresponding MAC CE.

Question 2.1: How many power control sets needs to be configured with respect to the each TRP and then in relation to the corresponding MAC CE per UE/cell/BWP?

Answer 2.1 RAN1 does not have an agreed number of power control sets yet. Maximum of 8 power control parameter sets per BWP seem a reasonable assumption to have some degree of freedom.

For mTRP PDCCH, RAN1 indicates that parameter searchSpaceLinking is suppposed to link two SearchSpace sets by RRC configuration with various limitations. However, it was not clarified whether the linking should be applied to all SearchSpaces set under Rel-15 and Rel-16 configurations.

Question 2.2: Should the searchSpaceLinking be applied to all or selected set of SearchSpaces under Rel-15 and Rel-16 configurations?

Answer 2.2 Applied on both Rel-15 and Rel-16 configurations for selected Search Spaces (not applied for searchSpaceSIB1, ra-SearchSpace, recoverySearchSpaceId, etc.).

RAN2 agreed to have separate MAC CEs for PUSCH pathloss reference RS update:

[060] To revise the legacy PUSCH Pathloss Reference RS Update MAC CE with additional field(s) to differentiate the TRP for mTRP PUSCH repetition, replace the Reserve bit (‘R’) to a TRP index field (‘T’) so that the MAC CE can indicate which TRP the PUSCH pathloss reference RS update can apply for.

Question 2.3: How is the "TRP identity" defined for this MAC CE or other potential per TRP MAC CEs?- is it based on SRS-ResourceSet ID, BFD RS SET ID or something else? Note that current ASN1 does not have yet BFD RS SETs implemented.

Answer 2.3 For M-TRP URLLC enhancements framework, TRP identity for PUSCH is mainly coming from SRS resource set. SRS resource set indicator also indicated in the DCI such that the UE knows which SRS resource set(s) is/are valid for PUSCH transmission/repetition operation. TRP index field “T” can be linked to the SRS-ResourceSet IDs of the two configured SRS resource sets, e.g. T=‘0’ indicates the SRS resource set with the lower SRS-ResourceSet ID (i.e. first TRP) and T=‘1’ indicates the SRS resource set with the higher SRS-ResourceSet ID (i.e. second TRP).

BFD-RS set ID is one to one mapped to coresetPoolIndex value (TPR identity used only for discussion purposes).

Agreement (RAN1#106e)

Support the following BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET:

- Implicit configuration:

  - M-DCI:

    - BFD-RS set k (k = 0, 1) is derived based on X TCI of CORESETs with CORESETPoolIndex = k

The L1 parameter excel does not have input on how to implement beam failure detection RS sets for mTRP. There is also not information on what is the maximum number of detection resources to be configured per UE per cell or per TRP. There is also not information on what is the maximum number of recovery resources to be configured per UE per cell or per TRP.

Question 2.4: Please inform how to implement beam failure detection RS sets for mTRP. Also what is the maximum number of detection resources to be configured per UE per cell or per TRP? What is the maximum number of recovery resources to be configured per UE per cell or per TRP?

Answer 2.4 RAN1 has agreed to support 2 BFD-RS sets per BWP, each set is associated with a TRP. The maximum number of BFD-RS resource per set is UE capability (under discussion within FG 23-5-2), and candidate values will be decided. The maximum number of recovery resources is also a UE capability (under discussion within FG 23-5-2a), where the maximum candidate value is 2.

Agreement

For M-TRP BFR

- Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set

  - FFS: value of N (e.g. fixed in specification, or UE capability)

- FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

Agreement

The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17.

3. CSI mTRP

For mTRP CSI, RAN2 was instructed to configure two codebook subset restrictions (CBSRs) per CodebookConfig, and two RI restrictions per CodebookConfig. However, it is not clear which CBSRs are intended to be used and whether there are specific restrictions to be applied for the RRC configuration.

Question 3.1: Which CBSRs are intended to be used and whether there are specific restrictions to be applied for the RRC configuration? Also whether is it introduced for both typeI-SinglePanel1 and typeI-SinglePanel2 and also for both 2Tx and more than 2Tx?

Answer 3.1 In Rel-15 Type I SP codebook there are three CBSR RRC parameters. The first two indicate restrictions of the DFT beams used to form the PMI to reduce interference in certain transmission directions. These beam restrictions are configured in the RRC parameter twoTX-CodebookSubsetRestriction, for two antenna ports, and in n1-n2, for each of the 13 supported configurations with more than two antenna ports listed in Table 5.2.2.2.1-2 of 38.214. The third CBSR RRC parameter is typeISinglePanel-codebookSubsetRestriction-i2, which restricts the choice of random subband precoder used for CQI calculation when the reportQuantity is set to 'cri-RI-i1-CQI'.

For mTRP CSI, CodebookConfig should contain two CBSRs, one per Resource Group in the resource set linked to the CSI-ReportConfig. These CBSR should have the same functionality as twoTX-CodebookSubsetRestriction, for 2Tx, and n1-n2, for more than 2Tx. The CBSR related to ‘i2’ does not need supporting because 'cri-RI-i1-CQI' is not supported as reportQuantity for mTRP CSI reporting.

The only configuration restriction applicable to the two CBSR parameters for mTRP CSI is that they should be configured with the same number of ports.

In the understanding that by ‘typeI-SinglePanel1’ and ‘typeI-SinglePanel2’ RAN2 refer to ‘mode 1’ and ‘mode 2’ of Type I single panel codebook, the CBSR parameters should be supported for both modes.

The CBSR parameters should be supported for both 2Tx and more than 2Tx.

4. SRS

RAN2 also noted that the parameter startPosition was not included in the indicated Rel-17 resourceMapping for SRS, but it was not clear if this was intentionally or accidentally omitted from the Rel-17 SRS configuration.

Question 4.1:  Should the parameter startPosition should be included in resourceMapping also for Rel-17 (similarly as it was there in Rel15 and Rel 16 configurations)?

Answer 4.1 Yes, the startPosition should be included in resourceMapping also for Rel-17.

2. Actions:

To RAN2 group:

ACTION: 	RAN1 would kindly ask RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

RAN1#109-e	May 16th – May 27th, 2022		E-meeting

RAN1#110	August 22nd – August 26th, 2022		Toulouse, FR