3GPP TSG-RAN WG1 Meeting #108-e	R1-2202311

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft LS reply on UE capability for supporting single DCI transmission schemes for multi-TRP

Response to:	R4-2120652/ R1-2200872

Release:	Rel-16

Work Item:	NR_eMIMO-Perf

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

Name:	Keeth Jayasinghe

E-mail Address:	keeth.jayasinghe@nokia.com

# 1. Overall Description

RAN1 would like to thank RAN4 for their LS in R1-2200872 (R4-2120652) on UE capability for supporting single DCI transmission schemes for multi-TRP.

RAN1 discussed the question posed by RAN4 and would like to clarify the following:

To support the single DCI FDM Scheme A (UE capability supportFDM-SchemeA-r16), the UE shall support more than one active TCI state for the BWP. Single DCI FDM Scheme A is configured via RRC “'fdmSchemeA”, and the scheme is only applied when the UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' (the UE must receive an MAC-CE activation command as in clause 6.1.3.24 of TS 38.321).

TS 38.214 Section 5.1 captures the following (RRC parameter  'fdmSchemeA' is used configuring FDM scheme A)

When a UE is configured by higher layer parameter repetitionScheme set to one of 'fdmSchemeA', 'fdmSchemeB', 'tdmSchemeA', if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)'.

-	When two TCI states are indicated in a DCI and the UE is set to 'fdmSchemeA', the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3.

Similarly, to support single DCI SDM (UE capability singleDCI-SDM-scheme-r16), the UE shall support more than one active TCI state for the BWP and receive an MAC-CE command activating a TCI codepoint with two TCI states (same MAC-CE used for FDM scheme A).

TS 38.214 Section 5.1 captures the following (no RRC parameter used for SDM operation)

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field 'Antenna Port(s)', the UE may expect to receive a single PDSCH where the association between the DM-RS ports and the TCI states are as defined in Clause 5.1.6.2.

Also, it should be noted that the single DCI inter-slot TDM scheme (supportInter-slotTDM-r16) is having a component of maxNumberTCI-states-r16 only to indicate the applicability of the inter-slot TDM scheme for single TRP mode (single TCI state) or multi-TRP mode (two TCI states), and it may not directly relate to maxNumberActiveTCI-PerBWP. Moreover, both single TRP and multi-TRP modes for single DCI inter-slot TDM are supported by the RAN1 specifications. As highlighted before, SDM and FDM schemes do not have a single TRP mode, and such a component (maxNumberTCI-states-r16) is not mentioned within the UE FG.

# 2. Actions:

To: RAN4

ACTION: RAN1 respectfully asks RAN4 to take into account the above information.

# 3. Date of Next TSG-RAN WG1 Meetings:

RAN1#109-e	May 16th – May 27th, 2022		E-meeting

RAN1#110	August 22nd – August 26th, 2022		Toulouse, FR