3GPP TSG RAN WG1 #108-e		                         R1- 2202346

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.7.1.1

Source: 	LG Electronics

Title: 	Discussion on potential paging enhancements

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss and provide our view on remaining issues of potential paging enhancement for idle/inactive mode UE power saving.

- Discussion

In a previous RAN1 meeting, following agreement was made to clarify the PEI-O determining method.

Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

According to the first bullet in this agreement, UE can assume two consecutive PFs are configured via SIB only. However, it should be noted that UE can be configured with ‘ran-PagingCycle’ in RRCRelease message. Since the determining the PFs for a UE is dependent on the paging cycle, the condition for the ‘consecutive PFs’ should be updated.

Proposal 1: The two PF are consecutive PFs configured in SIB or RRCRelease message.

- Conclusion

In this contribution, we discuss and provide our view on potential paging enhancement for idle/inactive mode UE power saving. Proposals in this contribution are summarized as follows.

Proposal 1: The two PF are consecutive PFs configured in SIB or RRCRelease message.