3GPP TSG RAN WG1 #108-e			R1-2202356

e-Meeting, February 21st – March 3rd, 2022

Source:	Mitsubishi Electric

Title:	Inter-UE coordination for enhanced resource allocation

Agenda Item:	8.11.1.2 - Feasibility and benefits for mode 2 enhancements

Document for: 	Discussion/Decision

# Introduction

In the latest plenary meeting RAN#94e it was concluded that RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022 and that all RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e. RAN1 is given guidance to use the following list of open issues provided RP-212880 (status report of WI: NR sidelink enhancement) as a starting point for technical discussions in RAN1:

- Physical layer aspects on solution(s) on enhancement(s) in mode 2 for enhanced reliability and reduced latency including

  - Scheme 1

    - Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

    - Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

    - Finalization of when UE-B generates and/or transmits an explicit request

    - Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of prioritization of inter-UE coordination information and explicit request

    - Combination of preferred/non-preferred resources with explicit request/condition triggers

  - Scheme 2

    - Finalization of determination of PSFCH resource/index for conflict indication

    - Finalization of behaviour of UE-B receiving a conflict indication from UE-A

    - Finalization of prioritization of conflict indication

    - Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

In the past RAN1#107be meeting decisions were made on scheme 1 (position of selection window, determining preferred resource set) and Scheme 2 (how to derive PSFCH occasions, candidate resource reporting in case of conflict indication, PSFCH prioritization).

In this contribution, we give our view on some open issues related to inter-UE coordination.

# Discussion

## On scheme 1 specific issues

### On scheme 1 with inter-UE coordination information transmission triggered by a condition other than explicit request

Both explicit request and condition-triggered schemes are beneficial. While scheme 1 with explicit request can control the signaling overhead through the explicit request mechanism itself, the condition-triggered scheme benefits from a lower latency since assistance information is available more rapidly to UE-B.

The following was agreed in the past RAN1#107b-e:

Agreement

For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

  - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

- Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

Working Assumption

For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

- Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

  - FFS: Under which conditions groupcast/broadcast can be supported

- Unicast

- FFS: Under which conditions unicast can be supported

The non-preferred resource set determined with condition 1-B-1/option 1 or condition 1-B-2 have the purpose of avoiding the reception of a TB onto a resource already occupied/unavailable for reception. This info is meaningless for UEs not intending to transmit a TB to UE-A. Preferred resource sets are also meaningless for UEs not intending to transmit a TB to UE-A. Moreover, sending preferred resource sets to groups of UE-s may increase the probability of collision To limit the useless transfer of inter-UE coordination information, we propose the following:

Proposal 1: For inter-UE coordination triggered by a condition other than request reception Scheme 1 
-	Broadcast of non-preferred resource set is supported only if the non-preferred resource set has been determined by condition 1-B-2 (protect reception of UE-A)	
-	Groupcast of non-preferred resource set is supported only in resource pools configured with Alt 2 and transmitted together with data	
-	Unicast of non-preferred resource set is supported only when UE-A is a destination UE of a TB transmitted by UE-B

Proposal 2: For inter-UE coordination triggered by a condition other than request reception Scheme 1,
-	 Broadcast/groupcast of preferred resources is not supported	
-	Unicast of preferred resource set is supported only when UE-A is a destination UE of a TB transmitted by UE-B

Under the current agreements, and in order to limit the overhead created by IUC transmissions:

Proposal 3: For scheme 1 inter-UE coordination information transmission triggered by a condition other than explicit request, UE-A sends preferred/non-preferred resources via unicast to UE-B only if the distance between UE-A and UE-B is larger than and/or smaller than a threshold.

### On scheme 1 with inter-UE coordination information transmission triggered by an explicit request

Allowing UEs other than destination UEs of a TB transmitted by UE-B to provide preferred/non-preferred resources for a transmission they are not destined to receive puts too much constraint onto the resource selection process at UE-B and may lead to insufficient number of candidate resources, which automatically leads to re-inclusion of some excluded resources and thus to overriding part of the inter-UE assistance information. UE-B should prioritize its own sensing and assistance from intended receivers when selecting the best fit resource for transmission.

Proposal 4: Confirm the working assumption on UE-A being a destination UE of a UE transmitted by UE-B for scheme 1 with explicit request. The trigger is sent with the same L1 source/destination ID as a TB to be transmitted by UE-B.

Under the current agreements, and in order to limit the overhead created by IUC transmissions:

Proposal 5: For scheme 1 inter-UE coordination information transmission triggered by explicit request, UE-A drops sending the coordination message when the distance between UE-A and UE-B is larger than and/or smaller than a threshold.

### On cast types

Assistance should not be restricted to a cast type, since all cast types can achieve benefits with inter-UE coordination methods.

Proposal 6: Support that UE-B uses inter-UE coordination for the resource selection for transmission of a TB regardless of the TB’s cast type.

## On the need of selection of UEs acting as UE-A for schemes 1 and 2

Systematically sending assistance info from all users in a multicast/broadcast scenario on a regular basis is counterproductive for scheme 2 (especially in dense traffic situations), and potentially overhead consuming for scheme 1 (although the explicit triggering may somehow limit this effect).

Figure 1– Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges

Reasonable criteria of selecting which UEs are fit to act as UE-A and of the events which trigger the sending of the assistance information are needed. It is intuitive to deduce in Figure 1 that, with a full-duplex assumption, orange ”UE-A”-type UEs close to the blue UE-B Tx would provide redundant assistance with respect to what UE-B is already sensing. Groups of UEs (e.g. groups of green UEs) in the outer communication detect similar hidden nodes and need not simultaneously report assistance information. A selection criterion is needed.

Reasonable selection criteria can include, for example, distance-based criteria, since UEs close to each other will produce similar sensing results, at least for the full duplex case. For example, only users in zones situated at a distance between configurable dmin and dmax distances from UE-B provide assistance, possibly with further down-selection to one user per zone. This is particularly beneficial to eliminate hidden node problems all in keeping the associated overhead low.

Time interval restrictions may be applied on top of that to address half duplex issues. For example, a first UE selection is performed based on distance as here-above. Among the users in zones closer than dmin from UE-B, only UEs able to sense during blind intervals of UE-B are allowed to send assistance information. Such a strategy mitigates both half duplex issue at the transmitter and hidden nodes problem.

Observation 1: In groupcast/broadcast inter-UE coordination, a selection mechanism needs to be introduced to select few assisting UEs (UE-As).

## Performance evaluations

### Performance for unicast transmission

Let us take the example of the hidden node problem. It is intuitive to say that the number of nodes hidden to the TX that a certain RX experiences collision from is dependent on the TX-RX distance. Indeed, when TX and RX are close together, they have similar sensing information. When the RX is in the outer part of the communication range, the sensing information at the receiver side can be significantly different from the one at the transmitter side, and in this case it is expected that receiver assistance information is useful to the transmitter in order to reduce the collision probability. When the TX-RX distance approaches or surpasses the communication range, the interference becomes a secondary issue since the link budget vanishes anyhow, so trying to solve the hidden node problem would probably not bring too much benefit at system level.

Our unicast evaluations in Figure 2 show the potential PRR gains that can be achieved by using RX assisted scheduling with scheme 1 instead of TX scheduling for unicast transmissions with periodic traffic and when the TX has full knowledge of the RX sensing.

Figure 2 – PRR in function of TX-RX distance with and without assistance information for unicast with periodic traffic, scheme 1, non-preferred resource set, full duplex

The simulation assumptions are summarized in the appendix. The corresponding communication range is around 420m. UE-A sends to UE-B the ordered list of non-preferred resources based on RSRP criterion. UE-B excludes them from its candidate list. In the case of blocking situation (not enough remaining resources), RSRP-based thresholding at UE-B may re-integrate some of the excluded resources in the inverse order from the ordered list provided by UE-A. A gain of up to ≈10% can be achieved in the simulated scenario, different results can be obtained with other parameters. It is clear that the PRR gain of the RX assisted scheme increases with the communication distance, until a certain limit where the communication becomes noise limited rather than interference limited; when the distance approaches the communication range, both schemes suffer mainly from the pathloss effect and exhibit similarly bad PRR. The gain of RX assisted scheduling over TX scheduling mainly comes from overcoming the hidden node effect loses in the interference-limited scenarios, where the advantage lies in predicting and eliminating hidden nodes.

Observation 2: Receiver-assisted TX scheduling can achieve 10% PRR gain with respect to TX autonomous scheduling in unicast scenarios.

### Performance evaluation of inter-UE coordination in multicast

As previously discussed, and as it can also be seen from Figure 3 and Figure 5, few users reasonably selected can provide the most important part of the significant assistance information.

One strategy of selecting users is to pick them in function of the distance from the transmitter UE, in a similar manner as for Rel.16 distance based HARQ. In the following dmin and dmax are understood as normalized with respect to the communication range. In our proposed simulation scenario, the communication range is ≈420 m. For example, with dmin=0.5 and dmax=1.1, UEs in the outer ring of the communication range at least 0.5*communication range away from the transmit UE are selected to provide assistance. We use dmax=1.1*communication range to account for the vehicles’ displacement in the 1 second between each groupcast formation, this displacement in 1s is ≈10%  of the communication range. In the chosen highway scenario, and since dmin is usually higher than the lane width, this is equivalent to choosing some front vehicles and some rear vehicles at least dmin and at most dmax away from the transmitter.

For example, in Figure 3, 5 vehicles are selected (the figure takes into account the fact that the communication range is significantly higher than the road width). With the numerical values simulated in Figure 6 and Figure 7, among 168UEs in the drop ≈35 UEs are part of the multicast group. On average, ≈35 are in the communication range at a given time. The strategy (dmin,dmax)=(0.25,1.1) selects on average 27.5UEs to send assistance info, and (0.5, 1.1) selects in average 18.5UEs . The strategy (0.75,1.1) selects on average 7.6UEs to send assistance info, and (0.9, 1.1) selects in average 3.5UEs. In order to have a better UE distribution, we also simulated the following cases:

- “2 UEs (1 front – 1 back”): (dmin, dmax) = (0.5,1.1) for a first selection step, and we further randomly select one front UE and one rear UE in a second selection step, as in Figure 4;

- “4 UEs (2 front – 2 back)”: 4 vehicles selection by picking one front and one rear vehicle in each range from (dmin, dmax) = (0.5, 0.75) and (0.75, 1.1), as in Figure 5.

We evaluated the different strategies of user selection to provide assistance. The extreme cases where TX UE has either no assistance or has full assistance information from all UEs in the multicast group are presented as reference.

Our multicast evaluations in Figure 6 show the potential PRR gains that can be achieved by using RX assisted scheduling instead of TX scheduling for groupcast/multicast transmissions with periodic traffic and when the TX has full knowledge of the RX sensing. In our simulation, the selected UE A candidates share an assistance report encompassing a prioritized list of non-preferred resources to the TX (UE B). This list of non-preferred resources is determined based on a given (predefined/pre-configured) RSRP threshold. Then, the ordering of prioritization of such non-preferred resources list is done based on the received RSRP. When the TX (UE B) receives these assistance reports from the candidate UE As, it carries out a second round of prioritization of the non-preferred resources based on the number of occurrences of such resource in the received assistance reports. In such a way, a resource which is declared as non-preferred resource in multiple assistance reports is to be excluded with a higher probability than a resource declared as not preferred in one report.

Figure 3 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges

Figure 4 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges – 2 UEs (1 front – 1 back)

Figure 5 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges – 4 UEs (2 front – 2 back)

Figure 6 – PRR in function of TX-RX distance with assistance information provided by a set of selected UEs from a multicast group with different selection strategies, scheme 1 with non-preferred resource set, full duplex

Figure 7 – PRR in function of TX-RX distance with assistance information provided by a set of selected UEs from a multicast group with different selection strategies over different delays, full duplex, scheme 1, non-preferred resource set

The list of receiver UEs from the multicast group is updated every 1 second to keep only UEs within the communication range as RXs UEs. The simulation assumptions are summarized in the appendix. Only a part of the users in the multicast group provide assistance information to UE-B. UE-A candidate selection is performed on distance-based criteria: only UEs lying in zones at a distance between dmin and dmax from the transmitter (UE-B) are selected as UE-A candidates.

Results show that RX assistance is also beneficial to the multicast case. Although the attainable gain is a bit inferior to the unicast case, assistance can still provide around ≈5-7% PRR gain. Having assistance information from a large number of users is overhead consuming. An overhead analysis was performed in our previous contribution [8]. Distance based selection of UE candidates to act as UE-A allows to get significant gain with reduced overhead. Selecting only 4 UEs in a well distributed manner procures almost the full amount of the achievable gain as shown in the results in Figure 6. This is intuitively explained in the setting in Figure 5, where it can be seen that 4 selected UEs can in some cases provide the full amount of significant assistance information for e.g. hidden node discovery.

Observation 3: Receiver-assisted TX scheduling can achieve 5-7% PRR gain with respect to TX autonomous scheduling in multicast scenarios.

Observation 4: Distance based selection of UE candidates to act as UE-A allows to get significant gain with reduced overhead. Selecting 4 UEs over different directions and distances out of all the users procures almost the full amount of the achievable gain.

In Figure 7, we present the results of the assistance-based multicast scenario when the latency in delivering the assistance is simulated in a realistic manner. We assume that a possible delay for assistance report can be N=1,2, or 4 slots, in a similar approach to Rel.16 based HARQ feedback, in order to account for possible delays that may arise due to assistance container limitations and RXs (UE As) processing. Also, the assistance report is decoded at the TX (UE B) side using a given MCS as shown in the simulation assumptions in the appendix. It is shown in Figure 7 that, in case only 4 UEs are selected as UE A to send assistance reports to the TX (UE B), although performance is declining with the latency increase (because the assistance information becomes outdated), a delay of up to 4 slots still shows gain over the case where no assistance is used.

Observation 5: The gain achieved by assistance in multicast scenarios depends on the delay of sending such assistance reports. A latency of up to 4 slots still provides gain over the case where no assistance is used.

# Conclusions

The contribution investigates inter-UE coordination, focusing on inter-UE coordination for mode 2 reliability enhancements. Based on our analysis and simulation results, we make the following proposals:

Proposal 1: For inter-UE coordination triggered by a condition other than request reception Scheme 1:
-	Broadcast of non-preferred resource set is supported only if the non-preferred resource set has been determined by condition 1-B-2 (protect reception of UE-A) 	 
- Groupcast of non-preferred resource set is supported only in resource pools configured with Alt 2 and transmitted together with data 	
 - Unicast of non-preferred resource set is supported only when UE-A is a destination UE of a TB transmitted by UE-B

Proposal 2: For inter-UE coordination triggered by a condition other than request reception Scheme 1,
 - Broadcast/groupcast of preferred resources is not supported	 
 - Unicast of preferred resource set is supported only when UE-A is a destination UE of a TB transmitted by UE-B

Proposal 3: For scheme 1 inter-UE coordination information transmission triggered by a condition other than explicit request, UE-A sends preferred/non-preferred resources via unicast to UE-B only if the distance between UE-A and UE-B is larger than and/or smaller than a threshold.

Proposal 4: Confirm the working assumption on UE-A being a destination UE of a UE transmitted by UE-B for scheme 1 with explicit request. The trigger is sent with the same L1 source/destination ID as a TB to be transmitted by UE-B.

Proposal 5: For scheme 1 inter-UE coordination information transmission triggered by explicit request, UE-A drops sending the coordination message when the distance between UE-A and UE-B is larger than and/or smaller than a threshold.

Proposal 6: Support that UE-B uses inter-UE coordination for the resource selection for transmission of a TB regardless of the TB’s cast type.

# References

### RP-202846, WID revision: NR sidelink enhancement, RAN#90, December 2020.

### RAN1#102-e Chairman’s notes, August 2020.

### RAN1#103-e Chairman’s notes, October/November 2020.

### RAN1#104-e Chairman’s notes, January/February 2021.

### RAN1#104b-e Chairman’s notes, April 2021.

### RAN1#105-e Chairman’s notes, May 2021.

### RAN1#91b Chairman’s notes, October 2018.

### R1-2005903, Inter-UE coordination for enhanced resource allocation, Mitsubishi Electric, RAN1#102-e, August 2020.

# Annex: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario [TR 37.885] - Vehicle speed= 140 km/hr |
| Channel model | NR V2X channel model (LoS and NLoSv components) [TR 37.885] |
| Vehicle density | 7 vehicles/km/lane |
| Spectrum allocation | - Carrier frequency: 6 GHz - Simulated bandwidth: 20 MHz |
| Vehicle type | Type 2 [TR 37.885] (omnidirectional antenna) |
| Subcarrier spacing | 15 KHz |
| Communication mode | Unicast, Groupcast |
| UE-to-UE association (Unicast) | - UEs are randomly paired, within the transmitter communication range initially - Each UE may participate only in a single pair - Only one UE in each pair transmit data |
| UE-to-UE association (Groupcast) | - A given Tx UE selects all UEs, dynamically within its communication range, to be receivers for its groupcast transmissions |
| Traffic model | Periodic variable packet size traffic [TR 37.885]: - Packet size: [800,1200] Byte with probabilities [0.8,0.2] respectively - Inter-packet arrival time: 20 ms - Latency requirement: 20 ms |
| TTI structure | NR slot: 10 symbols for data, 4 symbols total overhead (DMRS, SCI, etc.) |
| Sidelink control TX parameters | - 64 bits - QPSK modulation, polar coding |
| SCI/Data frequency resource allocation | - PSCCH: 5 PRB - PSSCH: 25 PRB (per sub-channel) |
| SCI/Data time resource allocation | - PSCCH: 2 symbols - PSSCH: 10 symbols |
| Data packet TX parameters | - 800 Byte packet: 16-QAM, LDPC (CR=0.5) - 2 sub-channels - 1200 Byte packet: 16-QAM, LDPC (CR=0.5) - 2 sub-channels |
| Assistance Parameters | - 16-QAM, LDPC (CR=0.5) |
| HARQ feedback | - Disabled |
| Resource selection | - Semi-persistent resource allocation (Similar to LTE mode 4 sensing and selection, with considering the NR additional procedures) |