3GPP TSG RAN WG1 #108-e                                                                  R1-2202364

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.4

Source:	NEC

Title:	Remaining Issues for NR NTN

Document for:	Discussion

# Introduction

In RAN1#107-e, the RAN1 discussion on Rel-17 non-terrestrial networks (NTN) has been mostly concluded. There are still some remaining issues. In this contribution, we provide our views on the remaining issues of ‘non-terrestrial networks (NTN)’.

# Beam switching

Several options have been proposed for beam management in NTN making use of predictability of beam switching based on the knowledge of satellite movement and UE position. Examples of such techniques include timer-based switching and geometry-based switching which do not rely on measurements. Instead, they rely on assistance information from gNB to perform either gNB or UE triggered beam switching. However, some companies have raised concerns that determination of beam management should also consider the channel condition, especially with consideration on the LoS/NLoS condition and it cannot be purely location based predictable event. Therefore, mechanisms such as location aware CSI-RS transmission and measurement have been proposed which can reduce signaling overhead by adjusting periodicity of CSI-RS transmission and corresponding reporting according to the location of the UEs. Such methods can control measurement effort at the UE side by adjusting measurement period according to its location. Another measurement-based method which can reduce measurement reporting overhead is switching triggered by UE with configuration on certain threshold, e.g., elevation angle, or RSRP threshold. For example, as shown in Figure 1, if UE0 traverses a path as shown in the figure due to satellite and/or UE movement, when it reaches edge of beam 2, if UE is only provided a measurement threshold without a prioritized list or restricted list of candidate beams, it may switch to beam 2 based upon CSI-RS measurement and then switch to beam 7 due to the movement of satellite and/or UE, which will cause unnecessary beam switching. However, such undesirable results can be avoided with the help of assistance information by letting the UE know its candidate beam e.g. by indicating the candidate beams for switching to the UE in which case it will not switch to beam 2 since it is not in its candidate list.

Figure 1 Path traversed by UE due to satellite and/or UE movement

Since CSI-RS resources can be beam specific, it is possible to assign mutually exclusive CSI-RS resources surrounding any particular beam of the satellite. For example, CSI-RS resources assigned to the seven beams in Figure 1 are not shared among the beams in this cluster. Note that these are not actual beam numbers in satellite footprint. For any UE, beam 1 is the serving beam and beam 2 to beam 7 are logical indices of surrounding beams in corresponding directions. Depending on beams’ footprints, each beam may be surrounded by more or less than 6 other beams. CSI-RS resource sets are defined so that based on the resource set a UE is signaled to measure, it can determine if it needs to perform measurements for beam switching and know target candidate beams. For example, referring to beam layout in Figure 1, resource sets can be defined as follows:

Set A = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 2, CSI-RS Resource Beam 3}

Set B = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 3, CSI-RS Resource Beam 4}

Set C = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 4, CSI-RS Resource Beam 5}

Set D = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 5, CSI-RS Resource Beam 6}

Set E = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 6, CSI-RS Resource Beam 7}

Set F = {CSI-RS Resource Beam 1, CSI-RS Resource Beam 7, CSI-RS Resource Beam 2}

Set G = {CSI-RS Resource Beam 1}

…

where “CSI-RS Resource Beam X” denotes CSI-RS resources configured to measure the quality/strength of beam with logical index X.

These resource sets may be either configured via RRC signaling or broadcast in system information. UE performs measurement on candidate beams for beam selection and switching only if quality of reference signal in the current beam is below a certain threshold and the UE has determined it is within/approaching coverage region of another beam based on the resource set it is signaled to measure. UE indicates the strongest beam, based on the measurements performed on reference signals, to the gNB for beam switching. Such a method can result in reduced overhead as not all measurement results are sent to the gNB. Moreover, UE’s channel condition is taken into account since beam switching decision is based on measurement of signals from candidate beams from the candidate CSI-RS set(s) configured for the UE. Beam indication is triggered upon meeting certain condition(s), e.g., when the RSRP of another beam is X dB higher than the serving beam (X can be configured by gNB). Selected beam indication takes place via L1 or MAC layer reporting or RRC signaling.

For a UE to perform measurements on candidate beams and switch to the selected beam, assistance information from the gNB about the target beams can be sent e.g., via a MAC-CE scheduled via a group common DCI to one beam. The group comprises of all UEs served by the current beam and the information is related to the candidate beams surrounding current beam.

Another option is to send relevant information of all beams of the satellite to all UEs served by the satellite in system information.

The assistance information does not necessarily need to include beam layout with information such as the beam width of BWPs/beam, center elevation and azimuth etc. Instead, with preconfigured resources and indication of candidate resources as above, the assistance information only needs to include beam specific parameters which would be necessary for a UE to know to perform measurements on candidate beams. Due to gNB’s control over candidate beam indication, it can indicate appropriate candidate beams to a UE for load balancing or interference coordination. For example, when UE1 in Figure 1 is at the edge of beam1 or for within overlapping region between beam6 and beam7, gNB can select target beam for UE1 that is more suitable for e.g. load balancing.

Proposal 1: To reduce measurement effort and retain some control of gNB over beam selection, it is proposed to support:

- Assign mutually exclusive CSI-RS resources to neighboring beams.

- Configure resource sets with possible combinations of candidate CSI-RS resources depending on the beam layout.

- Signaling from gNB indicates the candidate beams for measurement and beam switching.

- Assistance information from gNB includes any necessary information a UE would require to perform measurements on candidate beams.

- UE indicates selected beam for switching to the gNB based on measurement results from the candidate beams from the configured CSI-RS set(s).

# BWP switching

In LEO scenario with earth-moving beams, the BWP switching is caused by the satellite movement itself, and the contribution of UE’s movement to the determination of the relative position between UE and the satellite can often be negligible. Once the UE’s initial position is communicated to the network, the BWP switching pattern can be predetermined. The BWPs in which the UE will operate can therefore be predicted based on time and location since the satellite movement is known.

Beam switching does not always trigger BWP switching. Several satellite beams can be mapped to a BWP of a UE.  The satellite beams may be useful to extend the cell coverage without NR beamforming. The beam management operation such as beam indication, beam measurements and reporting may not be applicable. BWP switching can be done via the DCI, MAC-CE or inactivity timer (bwp-InactivityTimer).

It may be no longer feasible for the UE to fall back to the original BWP upon timer expiry in this scenario. Furthermore, the satellite cell has a large coverage footprint of tens or hundreds of km, the common initial BWP can become congested or blocked if the number of users is large. With consideration on the larger cell coverage and longer roundtrip time (RTT), mechanisms of configured BWP switching from a sequence of BWPs can be beneficial instead of switching back to the same initial BWP following a period of inactivity. If a UE keeps switching back to the initial BWP, prolonged delay or even beam failure may occur. Since the source and target BWP are from the same satellite, they can also share the same configuration towards the UE with regard to pdcch-Config, pdsch-Config, sps-Config, radioLinkMonitorConfig etc. Only the frequency location as identified by BWP ID may be changed and most of the configurations can remain the same.

The function of the bwp_InactivityTimer in the legacy system requires the UE to switch from the active BWP to default BWP. It was proposed that gNB may disable this function in NTN system by either providing a large value of timer duration or not providing the timer, in either case, the functionality of the inactivity timer is lost. The proposed solution not only keeps the functionality of the inactivity timer, but also improves the NTN system performance.

In addition, gNB may instruct the UE to perform L1 RSRP measurement to assist the BWP switching. These aperiodic measurements may be configured for UEs which are due to switch their BWPs based on the time and location.

If all L1 measurement reports are to be sent to the gNB, these may be out of date due to the longer propagation delay in NTN. Therefore, the measurement reports may be filtered at L1 and a measurement report will only be sent to the gNB if it meets certain configured threshold criteria for BWP switching.

The L1 measurement report will inform the gNB of the potential default BWP switching initiated by the UE upon timer expiry. The filtered L1 measurement report sent to the gNB may act as a notification for configured BWP switching.

Proposal 2: Support timer-based BWP switching to a new default BWP.

Proposal 3: Support the L1 filtered measurement report as a notification to gNB for BWP switching.

# Polarization Indication

In RAN1#106bis-e, it was agreed to support the following [1]:

Agreement:

- Support polarization signalling for target serving cell in handover command message.

- Support polarization signalling for non-serving cell in RRM measurement configuration.

Polarization is useful in mitigating the inter-beam and inter-cell interference. For both DL and UL, the polarization configuration should therefore be per beam. Each satellite beam may have a different polarization configuration. If multiple satellite beams are used within a cell, information on the polarization for each SSB should be included in the SIB. The polarization signaling {Linear, RHCP, LHCP} can be indicated per satellite beam for each cell, grouped for DL and UL respectively, with periodicity corresponding to the ephemeris information update due to satellite moving.

For polarization multiplexing, satellite needs to transmit different streams with different polarizations in the same satellite beam. This may not be supported by the existing satellite. For polarization used for inter-beam interference mitigation, the polarization for each beam in one cell will not need to change dynamically. Dynamic change of the polarization information may also have impact on the beam layout, so it is not necessary to support dynamic polarization indication in Rel-17.

For downlink, UEs with linear polarization antenna can receive circular polarization signal and UE with circular polarization antenna can receive linear polarization signal. For uplink, if UEs not supporting circular polarization transmission and UEs only support circular polarization transmission both exist in a scenario, the polarization capability of UEs should be known to gNB for UL scheduling. UE Capability Indication can be indicated to gNB, e.g. {Linear, RHCP, LHCP, both RHCP&LHCP}.

Proposal 4: Support polarization signaling per beam.

Proposal 5: Do not support dynamic polarization indication in Rel-17.

Proposal 6: Support UE polarization capability indication to gNB.

# Conclusion

From the discussion, we have the following observations and proposals.

Proposal 1: To reduce measurement effort and retain some control of gNB over beam selection, it is proposed to support:

- Assign mutually exclusive CSI-RS resources to neighboring beams.

- Configure resource sets with possible combinations of candidate CSI-RS resources depending on the beam layout.

- Signaling from gNB indicates the candidate beams for measurement and beam switching.

- Assistance information from gNB includes any necessary information a UE would require to perform measurements on candidate beams.

- UE indicates selected beam for switching to the gNB based on measurement results from the candidate beams from the configured CSI-RS set(s).

Proposal 2: Support timer-based BWP switching to a new default BWP.

Proposal 3: Support the L1 filtered measurement report as a notification to gNB for BWP switching.

Proposal 4: Support polarization signaling per beam.

Proposal 5: Do not support dynamic polarization indication in Rel-17.

Proposal 6: Support UE polarization capability indication to gNB.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #106bis-e, e-Meeting, October 11th – 19th, 2021.

[2] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, November 11th – 19th, 2021.