3GPP TSG RAN WG1 Meeting #108-e	R1-2202366

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.5.1

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Maintenance for Rx/Tx timing delays mitigation

Document for:	Discussion & Decision

# Background

The following definitions on timing error and timing error groups were agreed in RAN1#104e:

Agreement: The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

In TS 38.214 the definition of the UE TEGs was introduced as :

Timing Error Group(s) (TEG(s)) at UE side are defined: - ueRxTEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin. - ueRxTxTEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin

In this document we discuss further the definitions in view of the progress captured in RAN2 and RAN4 WGs.

# Tx/Rx Timing Error and Timing Error Margins

RAN4 agreed to support a single timing error margin associated with each Rx TEG. The timing error margin can be for example a value Tx so that the LMF can make the assumption that the error within the TEG is compensated to at least the value of  Tx.

It can be hence expected that RAN4 defines a timing error margin. That said, it is unclear if definition to be captured by RAN4 will include Rx/Tx timing error definitions to accommodate for possible realizations that have an impact on expected error at LMF or UE. More precisely, the LMF is expecting a timing error margin error margin shall compensation including the offset of the Rx antenna phase center to the physical antenna center (as per RAN1 agreement).

Proposal 1: 	From RAN1 perspective, the timing error margin(s) will be defined by RAN4 and  the definition takes the aspects Rx timing errors/Tx timing errors defined by RAN1 into account.

- Send an LS to RAN4

# TEG Definition

LPP proposed TEG definition in R2-2201723 (TS37.355 running CR):

TRP Tx Timing Error Group (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL-PRS Resources, which have the Tx timing error difference within a certain margin. UE Rx Timing Error Group (UE Rx TEG): A UE Rx TEG is associated with one or more DL timing measurements, which have the Rx timing error difference within a certain margin. UE Tx Timing Error Group (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS Resources for Positioning, which have the Tx timing error difference within a certain margin. UE RxTx Timing Error Group (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

It can be observed that ueRxTxTEG and ueRxTEG can be part of the captured definition in LPP. In our view LPP can capture the UE TEG definition which means also the  ueRxTxTEG and ueRxTEG definitions in TS38.214 can be removed

Proposal 2: 	To avoid duplication, ueRxTxTEG and ueRxTEG definitions are removed from TS38.214 if captured in LPP.

- Inform RAN2 on RAN1 conclusion.

# Conclusions

Based on the discussion in this document we have the follwoing proposals:

Proposal 1: 	From RAN1 perspective, the timing error margin(s) will be defined by RAN4 and  the definition takes the aspects Rx timing errors/Tx timing errors defined by RAN1 into account.

- Send an LS to RAN4

Proposal 2: 	To avoid duplication, ueRxTxTEG and ueRxTEG definitions are removed from TS38.214 if captured in LPP.

- Inform RAN2 on RAN1 conclusion.