3GPP TSG-RAN WG1 Meeting #108-e	R1-2202375

e-Meeting, 21st February – 3rd March, 2022

Title:	[DRAFT] Reply LS on interruption for PUCCH SCell activation in invalid TA case

Response to: 	R4-2120420, LS on interruption for PUCCH SCell activation in invalid TA case

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	Ericsson, [RAN1]

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

#### Name:	Sorour Falahati

E-mail Address:	sorour.falahati@ericsson.com

Attachments:	None

1. Overall Description:

RAN1 thanks RAN4 for the LS on NR-U channel information and procedures. RAN1 discussed the questions in the LS, and would like to provide the following answers to Q1 and Q2 raised in the LS:

Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells? - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306. - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities.

Answer from RAN1 to Q1:

The answer is NO.

The UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 cannot be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells.

For inter-band CA, the capability parallelTxPRACH-SRS-PUCCH-PUSCH and/or parallelTxMsgA-SRS-PUCCH-PUSCH  indicate if a UE can simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells, and there is no restriction on the SCS selection of PRACH.

For intra-band CA, the UE cannot support parallel transmission of PRACH and physical channels/signals on any other serving cell.

Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

Answer from RAN1 to Q2:

As mentioned in the answer for Q1, for inter-band CA, there is a capability irrespective of SCS to support parallel transmission of PRACH and physical channels/signals on any other serving cell. For inter-band CA, the capability parallelTxPRACH-SRS-PUCCH-PUSCH and/or parallelTxMsgA-SRS-PUCCH-PUSCH can be used regardless of the SCS used in different serving cells.

However, for intra-band CA the UE cannot support parallel transmission of PRACH and physical channels/signals on any other serving cell as specified in Clause 8.1 of TS 38.213:

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

Therefore, for intra-band CA, the UE shall transmit the PRACH in a slot different from the slot of physical channels/signals on other activated serving cells irrespective of SCS configurations.

2. Actions:

To RAN4:

ACTION:  RAN1 respectfully asks RAN4 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 	15 – 27 May 2022		Electronic Meeting

TSG-RAN WG1 Meeting #110 	22 – 26 August 2022		Toulouse, France