3GPP TSG RAN WG1 #108-e		R1-2202379

e-Meeting, February 21st – March 3rd ,2022

Title: 	About the LS on PEMAX for NR-V2X

Source: 	ZTE, Sanechips

Agenda item:	5

Document for:	Discussion and decision

Introduction

In this paper, we share our consideration on the RAN4 LS on PEMAX [1].

Discussions

Power Control

Rows 36-42 endorsed from RAN1 RRC list [2] on power control of PSCCH/PSSCH are copied below

Table 1 RRC Parameters for Power Control

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5G_V2X_NRSL-Core | SL power control | maximumtransmitPower-SL | new | maximumtransmitPower-SL | Indicates the maximum value of the UE’s sidelink transmission power on this resource pool in the unit of dBm. | -30..33 | Agreements made in RAN1#97: • For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss. • When the SL open-loop power control is configured to use both DL pathloss and SL pathloss, o The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken. * (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss. Agreements made in RAN1#98bis: • For PSFCH power control, o It is supported that the open-loop power control is based on the pathloss between PSFCH TX UE and gNB (if PSFCH TX UE is in-coverage): The nominal power and alpha for PSFCH power control are configured separately from the parameters used for PSCCH/PSSCH power control. o (working assumption) Sidelink pathloss based PSFCH power control is not supported. |
| 5G_V2X_NRSL-Core | SL power control | p0-DL-PSCCHPSSCH | new | p0-DL-PSCCHPSSCH | P0 value for DL pathloss based power control for PSCCH/PSSCH. If not configured, DL pathloss based power control is disabled for PSCCH/PSSCH. | -16..15 |  |
| 5G_V2X_NRSL-Core | SL power control | p0-DL-PSFCH | new | p0-DL-PSFCH | P0 value for DL pathloss based power control for PSFCH. If not configured, DL pathloss based power control is disabled for PSFCH. | -16..15 |  |
| 5G_V2X_NRSL-Core | SL power control | p0-SL-PSCCHPSSCH | new | p0-SL-PSCCHPSSCH | P0 value for SL pathloss based power control for PSCCH/PSSCH. If not configured, SL pathloss based power control is disabled for PSCCH/PSSCH. | -16..15 |  |
| 5G_V2X_NRSL-Core | SL power control | alpha-DL-PSCCHPSSCH | new | alpha-DL-PSCCHPSSCH | alpha value for DL pathloss based power control for PSCCH/PSSCH. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} |  |
| 5G_V2X_NRSL-Core | SL power control | alpha-DL-PSFCH | new | alpha-DL-PSFCH | alpha value for DL pathloss based power control for PSFCH. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} |  |
| 5G_V2X_NRSL-Core | SL power control | alpha-SL-PSCCHPSSCH | new | alpha-SL-PSCCHPSSCH | alpha value for SL pathloss based power control for PSCCH/PSSCH when sl-pathloss is enabled. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} |  |

Figure. 1 RRC Parameter in TS 38.331 for Power Control

Comparing the RRC parameter list, RAN1 agreement as well as the RRC parameter captured in TS 38.331, the paramter sl-MaxTransPower is designed to limit the transmit power of PSSCH/PSCCH. Thus it's reasonable to assume the power value used in RAN4 spec. is sl-MaxTransPower.

In the meantime, the paramter sl-MaxTxpower is used to limit the transmit power for CBR-based power control and it can take any value of sl-Txpower.

sl-MaxTransPower is designed to limited the transmit power of PSSCH/PSCCH.

Reply to RAN4 that the power defined in sl-MaxTransPower is used to limit the power of PSSCH/PSCCH.

The restriction on UE power limit should be on the sum of the SL power capability and that of Uu, while option 2 or 3 is to limit or correlate the SL power to Uu power capability, which seems to include additional constraint. Moreover, option 1 is inclusive of the other two options and relies on already existing configurations, option 2 or 3 would introduce additional spec. impact to reflect the constraints withoutsufficient technical motivation.

Reply to RAN4 that Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu is adopted.

Conclusion

Observation 1:	sl-MaxTransPower is designed to limited the transmit power of PSSCH/PSCCH.

Proposal 1:	Reply to RAN4 that the power defined in sl-MaxTransPower is used to limit the power of PSSCH/PSCCH.

Proposal 2:	Reply to RAN4 that Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu is adopted.

Reference

R1-2200866, LS on PEMAX for NR-V2X

R1-2003190, Updated consolidated parameter list for Rel-16 NR