3GPP TSG RAN WG1 #108	R1-2202385

e-Meeting, Feb 21th – Mar 3rd, 2022

Agenda item:		8.7.1.2

Source:	Nordic Semiconductor ASA

Title:					On TRS design for idle/inactive UEs

Document for:		Discussion and Decision

# Introduction

In RAN1#107bis, good progress has been achieved, the final FL summary captured in [1]. In this contribution we address the remaining essential open issue.

# Configuration of iTRS

Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

Based on RAN1#107 agreement, FFS is still open regarding further limitation on number of TRS resource sets. It is a bit unclear why a UE in lower bands, where e.g. max 4beams are allowed, should support up to 64 resource-sets of Idle TRS, it would be beneficial for UE implementation to specify max number of TRS resources as function of Lmax. For example, 2 or 3 times Lmax.

Furthermore, even if UE is not required to monitor all of those at given time, UE still must comprehend/store all the TRS configurations, because gNB can dynamically validate/invalidate the preferred one “on fly”.

Proposal-1: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

# Conclusions

In this contribution we discussed remaining aspect of iTRS design and we had the following proposal:

Proposal-1: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

# References

- R1-2200782, 	Moderator Summary #4 round discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	, Moderator (Samsung)