3GPP TSG RAN WG1 #108		R1-2202411

e-meeting, February 21st – March 3rd, 2022

Source:	Lenovo

Title: 	Physical layer aspects for small data transmissions

Agenda Item:   5.2

Document for:	Discussion

# 1	Timing alignment for small data transmission in INACTIVE

In Rel-17, there is an enhancement for so-called small data transmission (SDT), which enables a UE to transmit (generally small) packets of data while in RRC_INACTIVE state.

Using configured grant resources for SDT has strong benefits compared to a RACH procedure: Generally the packets can be somewhat larger, the transmission is more robust, and the transmission can occur on reserved resources where no contention for the medium is required. Therefore we think that CG-based SDT transmissions is an important operational scenario.

However, CG resources can only be used as long as the UE maintains its uplink timing alignment for the transmission. However so far a UE is required to maintain timing alignment only in the connected state (RRC_CONNECTED) but not in the inactive state (RRC_INACTIVE):

### TS38.133 v17.3.0 ### 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements. […]

Up to Rel-17 it makes sense that UE is not required to maintain its uplink timing alignment in RRC_INACTIVE, since UE is not performing any uplink transmission except PRACH. However, with the introduction of SDT, it actually makes also sense that UE maintains its uplink timing alignment in RRC_INACTIVE, in particular for the case of CG-SDT. Since UE performs UL transmission, e.g. CG PUSCH, in RRC_INACTIVE without a prior random access, it is beneficial for the timing accuracy if UE maintains its uplink timing alignment also in RRC_INACTIVE when being configured with CG-SDT resources and a TAT timer for the purpose of CG-SDT transmission in RRC_INACTIVE, e.g. SDT-TAT timer and CG-SDT are configured within the RRCRelease message. Maintaining uplink timing alignment means that UE in RRC_INACTIVE (gradually) adjusts its uplink timing when there is a DL timing difference observed by the UE, e.g. UE autonomously adjusts its uplink timing in order to follow the DL timing reference. It is assumed that a UE in RRC_INACTIVE (configured for CG-SDT) has a NTA value, e.g. last or most-recent NTA value used in RRC_CONNECTED before being released to RRC_INACTIVE. The UE would store and maintain NTA when being released to RRC_INACTIVE. If the UE for example moves further towards the cell edge, the received downlink timing arrives at a certain amount of time later than the previous reference downlink timing. The UE may change the NTA to a NTA,adjusted accordingly based on the new downlink timing. The downlink timing may be estimated based on e.g. SSB or PDCCHs for Paging.

As a consequence, we think that a UE capable and configured with SDT should be required to maintain its timing alignment even when in inactive state.

Proposal 1: A UE configured with SDT transmissions in inactive mode maintains its timing alignment. Adopt the provided TP for TS 38.133.

A corresponding TP is provided in the summary section of this document.

# 2	Summary

In this contribution, following proposals have been discussed:

Proposal 1: A UE configured with SDT transmissions in inactive mode maintains its timing alignment. Adopt the provided TP for TS 38.133.

### TP for TS38.133 v17.3.0 ### 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in connected state. Additionally a UE configured with small data transmission shall have capability to follow the frame timing change of the reference cell in inactive state. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements. […]